3GPP TSG-RAN WG4 #116bis	R4-2513552

Prague, Czech Republic, October 13-17, 2025

Agenda item:	6.12.2.2		Other RRM requirements

Source:	Qualcomm Incorporated

Title:	On additional performance requirements for MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #115 the work on RRM performance requirements for MIMO Phase 5 has been started. In this paper we provide our view on RRM performance requirements identified in [1] in RAN4 #116.

# Discussion

In this paper we share our view on the performance requirements for MIMO Phase 5 for coherent joint transmission reporting and asymmetric DL sTRP / UL mTRP

## CJTC Reporting

In RAN4 #116, the following way forward was agreed for coherent joint transmission reporting

#### Issue 2-2-3: Test scope

<Way forward >

| High level: | QC | Apple | Samsung | CMCC | Nokia | Ericsson | MTK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify CJTC-Dd, measurement period | Y | FFS | Y | Y | Y | Y | Y |
| Whether to define TCs to verify CJTC-f，measurement period | Y | FFS | Y | Y | Y | Y | Y |
| Whether to define TCs to verify CJTC-P， (To be verify?) | N | N | N |  |  |  |  |
| Whether to define TCs to verify scheduling restriction |  |  |  | Y (if core req.) |  |  |  |
| Whether to define TCs to verify CJTC-Dd, CJTC-f，accuracy |  |  |  |  | Y | Y |  |

:

In our opinion test cases for delay and frequency offset reporting should be defined. Test cases for phase offset reporting cannot be defined since no study has been done in RAN4 so far and we do not see how this could be done in the remaining time.

We also agree that test cases to verify the reporting accuracy should be defined.

Proposal 1:  RAN4 should define test cases for delay and frequency offset reporting.

Proposal 2: RAN4 should not define test cases for phase offset reporting

Proposal 3: RAN4 should define test cases to verify the reporting accuracy for delay and frequency offset reporting.

## Asymmetric DL sTRP/UL mTRP

#### Issue 2-3-1: Test scope

<Way forward >

| High level: | QC | Apple | Samsung | CMCC | Huawei | Nokia | MTK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify timing requirements? |  | Y | Y | Y | Y | Y |  |
| Whether to define TCs to verify TCI state switching requirements? | Y |  | N |  |  |  |  |
| FR1 and/or FR2? | both |  |  |  |  |  |  |
| Whether to cover Scenario#1 and Scenario#2? | Both for FR1. Scenario#2 for FR2 | Both |  | Both | Scenarios#1 | both | Scenarios#1 |

In RAN4 #116, the following way forward was agreed for asymmetric DL sTRP / UL mTRP.

In our opinion, RAN4 should define test cases for FR1 and FR2. Also, both scenarios #1 and #2 should be covered by test cases in FR1. For FR2, however, we are fine to define only test cases for scenario #2. All these cases are in the scope of the work item. If no performance requirements are defined, the task of the work item is not fulfilled in our opinion.

Proposal 4: RAN4 should define test cases for FR1 and FR2.

Proposal 5: Also, both scenarios #1 and #2 should be covered by test cases for FR1. For FR2, only test cases for scenario #2 should be defined.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #116. Our observations and proposals are summarized as follows:

Proposal 1:  RAN4 should define test cases for delay and frequency offset reporting.

Proposal 2: RAN4 should not define test cases for phase offset reporting

Proposal 3: RAN4 should define test cases to verify the reporting accuracy for delay and frequency offset reporting.

Proposal 4: RAN4 should define test cases for FR1 and FR2.

Proposal 5: Also, both scenarios #1 and #2 should be covered by test cases for FR1. For FR2, only test cases for scenario #2 should be defined.

# References

- R4-2512130, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4 #116, August 2025