3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513558

Prague, Czech, 13th – 17th Oct., 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for MIMO phase 6

Agenda item:	7.10.3

Document for:	Discussion

Introduction

In agreed revised WID for Rel-20 MIMO phase 6 [1], following objective was captured:

The detailed objectives are as follows: RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In this contribution, we provide our initial views on RRM core requirements for Rel-20 MIMO.

Discussion

SRS enhancement

Enhancement of frequency hopping

Following has been captured in RAN1#122

RAN1#122 Agreement: For SRS configured with RPFS (PF>1) and multiple repetitions (R > 1), support multiple frequency-domain starting positions across SRS repetition symbols within each SRS frequency hop based on the followings: - For SRS repetition symbols within each SRS frequency hop, the starting position patterns across the K different frequency locations are determined by network configuration - The R repetition symbols are equally divided into K subgroups - K is number of starting positions for SRS repetition symbols within each SRS frequency hop - Within each subgroup of R/K symbols, the SRS is transmitted at the same starting position in frequency domain. - Start position pattern for SRS repetition symbols within each SRS frequency hop is the same during the legacy SRS frequency hopping period (for a same value of ) FFS: whether/how to support enabling legacy RPFS start RB index hopping across multiple legacy SRS frequency hopping periods and intra-repetition hopping for SRS repetition symbols within each SRS frequency hop simultaneously. Agreement: For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, study the following configuration combinations: - PF=2 and K=2 - PF =4 and K=2 - PF =4 and K=4

Regarding SRS frequency hopping, multiple start position of partial frequency hopping is introduced to increase the performance of SRS by enhancing the channel interpolation. In TS38.133, the interruption requirement of SRS antenna switching and carrier switching are specified, but SRS frequency hopping is not specified. So far, it seems that no specficiation impact is observed at least from RAN4 perspective.

Proposal 1: Regarding SRS frequency hopping enhancement, RAN4 to postpone the discussion until any RRM impact is identified

Enhancement of cross-slot SRS

RAN1#122 Agreement: For a P/SP cross-slot SRS resource, the slot offset configured to the SRS resource refers to the first of the two slots spanned by the SRS resource. Agreement: When there is a cross-slot SRS resource in the end of a S slot and the beginning of a U slot in a serving cell, support transmitting PUSCH with a priority index 0 and corresponding DMRS after this cross-slot SRS in the U slot in the serving cell. Agreement: Support at least the following scenarios for cross-slot SRS transmission: - Scenario 1: a periodic, semi-persistent, or aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted across a first S slot and a second consecutive U slot. - FFS whether Scenario 2 below is supported. - Scenario 2: an aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted in a first S slot, and at least one another SRS resource with time-domain resource transmitted in a second consecutive U slot. Agreement: For a given AP-SRS resource set in case of SRS transmission across two adjacent S+U slots, study the following alternatives and down-select one to determine “available slot” in the next meeting: - Alt-0 (per SRS resource set): The available slot(s) can include one slot or two consecutive S and U slots satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Alt-1 (per SRS resource): The available slot(s) can include one slot or two consecutive S and U slots satisfying there are UL or flexible symbol(s) for the time-domain location(s) for at least one of the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Alt-2 (per slot): An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for a subset of SRS resource(s) with same slot offset in the SRS resource set, and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. Agreement: To determine the time-domain location of cross-slot SRS, - One set of time-domain resource allocation related parameters (i.e., startPosition, nrofSymbols, and repetitionFactor) is configured for the SRS resource without restriction on “within a slot”. - For the index of each SRS symbol l', it is the same as legacy spec., i.e., . - The offset counts symbols backwards from the end of the starting slot of the resource

Regarding cross-slot SRS, SRS repetition can be transmitted in cross-slot to ensure efficient utilization of UL resource, and avoid phase/power shifting. In the legacy NR, SRS resource is limited within a slot. In RAN1#122, according to agreement, RAN1 has agreed to introduce cross-slot SRS resource (Scenario 1). From the RAN4 perspective, at least regarding interruptions due to SRS antenna switching, no significant impact has been identified, as such interruptions are allowed and may occur either between SRS resources or between SRS resource sets. Furthermore, RAN1 has not achieved any agreement regarding supporting SRS usage. Therefore, we would like to suggest wait for RAN1 progress.

Proposal 2: Regarding cross-slot SRS, RAN4 to postpone the discussion until further progress in RAN1

DL enhancement

Early CSI acquisition triggering

RAN1#122 Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support at least aperiodic SRS-AS transmission triggered via MSG4 of 4-Step RACH. Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support aperiodic CSI reporting triggered via MSG4 of 4-Step RACH based on the followings: - The aperiodic CSI reporting is transmitted on PUSCH. - Support at least aperiodic CSI-RS for CSI associated with the aperiodic CSI reporting - Support PMI-based reporting with wideband PMI based on Rel-15 Type-I SP codebook and wideband CQI - FFS: Which report quantity(s) can be configured - Support PMI-free reporting with wideband CQI - FFS: Which report quantity can be configured Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3 - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting Note: The term “capability” above does not mean legacy RRC based UE capability. FFS: For a UE transition from INACTIVE to CONNECTED mode, whether above procedure can be reused at least for early aperiodic SRS-AS/CSI-RS/CSI triggering Note: Whether the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting can be configured/triggered simultaneously will be discussed separately Agreement: For early triggering of SRS-AS/CSI-RS/CSI when UE transition from IDLE to CONNECTED mode, study the following at least three options for Step-1 and Step-2: - Option-1: NW can provide the resource/report configuration in SIBx based on only one UE capability assumption, and UE can report through MSG3 whether the resource/report configuration received in SIBx is supported. - Option-2: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 which resource/report configuration(s) received in SIBx is/are supported. - Option-3: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 the supported capability(s) of early SRS/CSI/CSI-RS triggering (e.g., whether to support this feature, max number of CSI-RS ports, xTyR for SRS-AS, max bandwidth of the CSI-RS/SRS-AS etc.). Note: The term “capability” or “UE capability” above does not mean legacy RRC based UE capability. Agreement: For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) - FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell. - The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) - FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command? - FFS: Whether/what new information is provided in SCell activation command?

Regarding early CSI acquisition triggering, three transition time is discussed in RAN1. One is RRC_IDLE/INACTIVE mode to RRC_CONNECTED mode, another is deactivated SCell to SCell activation, and the other is switching out of SCell dormancy.

At least from RAN4 perspective, no spec impact is observed in CSI acquisition in RRC_IDLE/INACTIVE to RRC_CONNECTED. Regarding switching out of SCell dormancy, BWP switching delay requirement is specified, however, no related agreement was achieved in RAN1.

On the otherhand, CSI acquisition in SCell activation, RAN4 needs to specify the timeline of SRS-AS transmission and AP CSI-RS as stated in RAN1 agreement.

Proposal 3: Regarding CSI acquisition in RRC_IDLE to RRC_CONNECTED, RAN4 to postpone the discussion until any RRM impact is identified

Proposal 4: Regarding switching out of SCell dormancy, RAN4 to postpone the discussion until further progress in RAN1

Proposal 5: Regarding CSI acquisition in SCell activation, RAN4 to specify timeline of SRS-AS transmission and AP CSI-RS for CSI

For SCell activation with AP CSI-RS for CSI, timeline needs to be specified. Although the signaling of AP CSI-RS has not been specified yet, it seems that indication can be included in the SCell activation command according to RAN1 agreement. Thus, AP CSI-RS seems can be triggerd in the SCell activation command.

In the legacy SCell activation delay requirement, at least for FR1, following cases are specified :

- SCell activation delay requirement in known condition

- SCell activation delay requirement in unknown condition when TCI state indication is provided

- SCell activation delay requirement in unknown condition when TCI state indication is not provided

  - If semi-persistent CSI-RS or periodic CSI-RS is used for CSI reporting

RAN4 needs to define the applicable cases for early CSI acquisition in SCell activation. In our view, all the cases except cases of using semi-persistent CSI-RS or periodic CSI-RS can be considered. Since the cases of semi-persistent CSI-RS or periodic CSI-RS are already used for provide CSI reporting, it is not sure further enhancement is needed. However, we still think early CSI acquisition needs to be specified when TCI state indication is not provided. One solution is to specify the case of using AP-CSI indicated by SCell activation command when TCI stae indication is not provided. So we suggest RAN4 to consider the applicable cases as follows:

- SCell activation delay requirement in known condition

- SCell activation delay requirement in unknown condition when TCI state indication is provided

- SCell activation delay requirement in unknown condition when TCI state indication is not provided

  - FSS: If semi-persistent CSI-RS or periodic CSI-RS is used for CSI reporting

  - Aperiodic CSI-RS for CSI reporting

Proposal 6: RAN4 to consider following cases for early CSI acquisition in SCell activation with AP CSI-RS at least for FR1:

- SCell activation delay requirement in known condition

- SCell activation delay requirement in unknown condition when TCI state indication is provided

- SCell activation delay requirement in unknown condition when TCI state indication is not provided

  - FSS: If semi-persistent CSI-RS or periodic CSI-RS is used for CSI reporting

  - Aperiodic CSI-RS for CSI reporting

For SCell activation with SRS antenna switching, timeline needs to be specified. In order to transmit SRS, valid timing advance and PL-RS should be provided. At least from our understanding, related requirement is specified in SCell activation delay requirement for PUCCH SCell. Hence, we think SCell activation with SRS antenna switching can be specified based on PUCCH SCell activation delay requirement.

Proposal 7: RAN4 to specify early CSI acquisition in SCell activation with SRS antenna switching based on PUCCH SCell activation delay requirement

CSI-RS ports aggregation

RAN1#122 Agreement: CSI-RS frequency-domain density ρ = 1/4 can be configured to the K NZP CSI-RS resources at least for the following cases: - K=2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports - K=4 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports - K=2 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports - K=4 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 128 CSI-RS ports FFS: K=3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports Note: It’s not precluded that the frequency-domain density configured to the K NZP CSI-RS resources in the same CSI-RS resource set for 48/64/128 CSI-RS ports aggregation can be different Agreement: CSI-RS frequency-domain density ρ = 1/3 and 1/6 can be configured to the K NZP CSI-RS resources at least for the following cases: - K=3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports FFS: K=2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports Note: It’s not precluded that the frequency-domain density configured to the K NZP CSI-RS resources in the same CSI-RS resource set for 48 CSI-RS ports aggregation can be different

Regarding CSI-RS ports aggregation, it is introduced to reduce the CSI-RS overhead in frequency domain. So far, it seems that no specficiation impact is observed at least from RAN4 perspective.

Proposal 8: Regarding CSI-RS ports aggregation, RAN4 to postpone the discussion until any RRM impact is identified

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: Regarding SRS frequency hopping enhancement, RAN4 to postpone the discussion until any RRM impact is identified

Proposal 2: Regarding cross-slot SRS, RAN4 to postpone the discussion until further progress in RAN1

Proposal 3: Regarding CSI acquisition in RRC_IDLE to RRC_CONNECTED, RAN4 to postpone the discussion until any RRM impact is identified

Proposal 4: Regarding switching out of SCell dormancy, RAN4 to postpone the discussion until further progress in RAN1

Proposal 5: Regarding CSI acquisition in SCell activation, RAN4 to specify timeline of SRS-AS transmission and AP CSI-RS for CSI

Proposal 6: RAN4 to consider following cases for early CSI acquisition in SCell activation with AP CSI-RS at least for FR1:

- SCell activation delay requirement in known condition

- SCell activation delay requirement in unknown condition when TCI state indication is provided

- SCell activation delay requirement in unknown condition when TCI state indication is not provided

  - FSS: If semi-persistent CSI-RS or periodic CSI-RS is used for CSI reporting

  - Aperiodic CSI-RS for CSI reporting

Proposal 7: RAN4 to specify early CSI acquisition in SCell activation with SRS antenna switching based on PUCCH SCell activation delay requirement

Proposal 8: Regarding CSI-RS ports aggregation, RAN4 to postpone the discussion until any RRM impact is identified

Reference

[1] RP-252936, “Revised WID: NR MIMO Phase 6,” MediaTek

[2]