3GPP TSG-RAN4 Meeting #116bis	R4-2513576

Prague, Czech, Oct. 13-17, 2025

Agenda item:	 6.9.2.1

Source:	 vivo

Title:	Discussions on Rel-19 OTA requirements

Document for:	Approval

- Introduction

Last meeting, RAN4 made good progress on framework of Rel-19 OTA performance work [1].

This contribution shares our views on how to specify Rel-19 OTA requirements.

- Discussion

Status of Rel-19 OTA measurement campaign

Last RAN4 meeting, we share the latest measurement plan of each test lab based on offline coordination with all the volunteer test labs and interested operators, as summarized in the table below:

Due to limited time between RAN4#116 and RAN4#116bis meeting, the measurement progress in test labs is slow. Besides, last meeting agreed the template for UE model information [2], but the UE information (even the initial planned UEs) has not been sent to MCC yet. Therefore, an action in RAN4#116bis meeting is needed to check with test labs about the latest status of measurement campaign and UE information disclosure status.

Proposal 1: Check in RAN4#116 bis meeting, with volunteer test labs about the status of Rel-19 measurement campaign, and their plan of sharing the UE model information to 3rd party MCC.

Working procedure update

The working procedure for measurement campaign is updated and approved in WF [1]. There was some discussion points not concluded. First issue is the UE information threshold:

- Fulfilment of the following thresholds (for each band and power class) related to the devices pool for Rel-19

  - Minimum number of devices for each band, each device size, each power class: [45~50] or [60] (based on the assumption that all test labs can submit results on time according to current test plan)

  - Minimum number of device models: [10] less than above

  - Minimum number of devices' vendors: [6]

  - Percentage of devices from second-half 2022 to first-half 2025: 100%

  - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

  - Define the Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: target to be =TBD%

In Rel-18, the following table summarized the outcome of the measured UEs:

Table 1: Rel-18 UE model statistics analysis (part of key information in [3])

| Total Devices | 83 |  |  |
| --- | --- | --- | --- |
| Total Models | 81 |  |  |
| Total number of device vendors | 19 |  |  |
| Percentage of devices per year of production | 8 | 2021 | 9.64% |
|  | 30 | 2022 | 36.14% |
|  | 35 | 2023 | 42.17% |
|  | 10 | 2024 | 12.05% |
| Percentage of devices with at least one certification | 83 |  | 100.00% |
| Percentage of devices for each certification | 27 | GCF | 32.53% |
|  | 12 | PTCRB | 14.46% |
|  | 30 | NAL(CTA) | 36.14% |
|  | 30 | FCC | 36.14% |
|  | 35 | CE | 42.17% |
|  | 0 | Not Available |  |

It can be observed that the repeated model is just 2 from 83, which means repeated UE model measured by different test lab is really a corner case. Another key information not listed in the table above is that even the total is 83, but for each band the number of devices varies, the smallest number of devices is 47, for band n78 TRS.

Observation 1: In Rel-18, the smallest number of devices is 47 for n78 TRS.

Observation 2: The repeated model is just 2.4% in Rel-18 (2 from 83).

In our understanding, when we define the thresholds for Rel-19, the group needs to consider the outcome from Rel-18. Another aspect is that the measured n1/28/41/78 are popular NR bands in many regions, but in Rel-19 the focused bands n3/5/7/8 may be harder to collect sufficient supported devices.

Observation 3: Rel-19 may face difficulty on collecting the same amount of measurement results as Rel-18.

Therefore, based on above considerations we believe the reasonable for Rel-19 thresholds would be:

  - Minimum number of devices for each band, each device size, each power class: 40~45

  - Minimum number of device models: 5

  - Minimum number of devices' vendors: 6

  - Percentage of devices from second-half 2022 to first-half 2025: 100%

  - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

  - Define the Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: No need to define this value.

Proposal 2: RAN4 should consider the following thresholds as reasonable value for Rel-19 measurement campaign:

- Minimum number of devices for each band, each device size, each power class: 40~45

- Minimum number of device models: 5

- Minimum number of devices' vendors: 6

- Percentage of devices from second-half 2022 to first-half 2025: 100%

- Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

- Define the Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: No need to define this value.

FFS whether RAN4 will introduce a methodology to alter the device data pool according to the following options:

- Option 1: Use the device data pool as is

- Option 2: Assuming the total number of devices in a given band is > [50], remove 1 best and 1 worst-performing data point from the data pool for this band

- Option 3: Further discuss a mechanism to identify and re-measure faulty devices; possible mechanism options are as follows:

    - Option 3a: define a threshold under which the device is considered a faulty sample

    - Option 3b: Define a delta with respect to the maximum measured value under which the device is considered a faulty sample

    - Option 3c: Analyse the CDF curve to identify deviations of the lower tail and therefore a faulty sample of the device

    - Other options are not precluded

- Other options are not precluded

The second point for discussion is how to treat the data pool, given there are some views that few measured results are clearly “Bad” results, RAN4 should figure out a solution to remove those results based on a clear guidance/procedure.

In Rel-18, the similar comments were discussed many meetings, even until the last meeting of WI. The moderator did different analysis of data pool to show the difference to the group:

- Full set CCDF/CDF analysis

- CCDF/CDF analysis （dBm）after removing the Best and Worst value of each band

- CCDF/CDF analysis （dBm）after removing single-side-outlier values

- CCDF/CDF analysis （dBm）after removing two-side-outlier values

But the outcome show that the 80%-tile value did not change too much, the difference is typically within 0.5dB for all bands. Finally, RAN4 reached consensus that the group keep the data pool as it is and focus on the requirements discussion based on the CDF curve.

Observation 4: In Rel-18, different approach on data processing the measurement data pool was done. However, the outcome shows that only small changes at 80%-tile value, within 0.5dB for all bands.

In Rel-19, the same discussion repeated, but we think there would be not much difference given RAN4 is still trying to collect a large data pool based on globally input. The overall data pool is much representative of the UE performance covering different countries/regions market.

Therefore, our preference is still keeping the device data pool as it is (Option 1), but we think Option 2 (i.e., remove 1 best and 1 worst-performing data point from the data pool for this band.) could be considered if the number of devices is large enough (i.e., more than 60 for a band) to minimize companies concern on the “Bad” result impact.

Proposal 3: Same as Rel-18, keep the data pool as it is without removing data point, and focus on the requirement discussion. Remove 1 best and 1 worst-performing data point approach could only be considered if the total number for that band is more than 60.

In summary, our proposal on how to update the framework for performance measurement campaign and requirements is show below:

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: n3, n5, n7, and n8 [2Tx at bands n41]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand and PDA hand

  - Operation mode: SA with 1Tx as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: UE (width >56mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: [2022 2nd-half-2025 first-half]

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. PC3 for all 1Tx bands. PC2 for 2Tx band n41.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - Same UE model supporting different sets of bands can be measured. For this case, the UE model should be marked as different model e.g., model A-1, model A-2 (guidance on how to manage this case are provided in the UE information sheet in [TBD]

  - Logistical aspects for devices provisioning to the labs are TBD

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template [R4-2512613] can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Encourage Information of the devices that are going to be measured shall be shared with the RAN4 Secretary by Monday the 15th of September at 23:59 GMT using the template defined in R4-2512613

      - Encourage The RAN4 Secretary includes the received device information in the summary table as per the template R4-2512613 and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

      - The complete device information sheet is targeted to be available by RAN4#116-bis meeting (i.e., not later than the TDoc submission deadline) to evaluate the accomplishment of the defined thresholds and act accordingly in case these are not. If all the UE information for measurement campaign is fully collected.

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Fulfilment of the following thresholds (for each band and power class) related to the devices pool for Rel-19

      - Minimum number of devices for each band, each device size, each power class: 40~45

      - Minimum number of device models: 5

      - Minimum number of devices' vendors: 6

      - Percentage of devices from second-half 2022 to first-half 2025: 100%

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

      - Define the Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: N/A

        - If for any band with the number of measured data points < TBD this threshold is not met, RAN4 can request the test labs to measure up to TBD additional devices to shift the ratio closer to the target value

  - Laboratories use the same worksheet template in [R4-2512686] to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool. Separate worksheets will be used for submission of data from the 1Tx and 2Tx measurement campaigns.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is 15 (depends on how many test labs will join the activity) with a minimum of [3] devices.

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

  - All the measurement results shall be submitted by RAN4#117 meeting (i.e., not later than the TDoc submission deadline) to provide enough time to the companies to analyse the provided results and let RAN4 have an initial discussion on the requirements’ values during the meeting.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline.

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - Only PC3 requirements of selected bands for 1Tx.

  - No differentiation of Size 1 and Size 2 TRP/TRS requirements

  - FFS whether RAN4 will introduce a methodology to alter the device data pool according to the following options:

    - Option 1: Use the device data pool as is

    - Option 2: Assuming the total number of devices in a given band is > [50], remove 1 best and 1 worst-performing data point from the data pool for this band

    - Option 3: Further discuss a mechanism to identify and re-measure faulty devices; possible mechanism options are as follows:

      - Option 3a: define a threshold under which the device is considered a faulty sample

      - Option 3b: Define a delta with respect to the maximum measured value under which the device is considered a faulty sample

      - Option 3c: Analyse the CDF curve to identify deviations of the lower tail and therefore a faulty sample of the device

      - Other options are not precluded

    - Other options are not precluded

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - Only PC3 testing is needed.

  - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

- Test lab procedures for 2Tx

  - Only non-coherent device.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

  - The baseline TRP measurement should be performed with all the Tx antenna ON and consistently operating at maximum power level condition. Fixed TPMI=2 is the configuration to be used for non-coherent UE. For any additional test configuration refer to TR 38.870 clause 7.5.3.1.

  - FFS other details

Proposal 4: Approve above updated working procedure for Rel-19 performance measurement campaign.

- Conclusion

In this contribution, we share the following proposals for Rel-19 TRP TRS performance work:

Observation 1: In Rel-18, the smallest number of devices is 47 for n78 TRS.

Observation 2: The repeated model is just 2.4% in Rel-18 (2 from 83).

Observation 3: Rel-19 may face difficulty on collecting the same amount of measurement results as Rel-18.

Observation 4: In Rel-18, different approach on data processing the measurement data pool was done. However, the outcome shows that only small changes at 80%-tile value, within 0.5dB for all bands.

Proposal 1: Check in RAN4#116 bis meeting, with volunteer test labs about the status of Rel-19 measurement campaign, and their plan of sharing the UE model information to 3rd party MCC.

Proposal 2: RAN4 should consider the following thresholds as reasonable value for Rel-19 measurement campaign:

- Minimum number of devices for each band, each device size, each power class: 40~45

- Minimum number of device models: 5

- Minimum number of devices' vendors: 6

- Percentage of devices from second-half 2022 to first-half 2025: 100%

- Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

- Define the Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: No need to define this value.

Proposal 3: Same as Rel-18, keep the data pool as it is without removing data point, and focus on the requirement discussion. Remove 1 best and 1 worst-performing data point approach could only be considered if the total number for that band is more than 60.

Proposal 4: Approve above updated working procedure for Rel-19 performance measurement campaign.

# References

- R4-2512693, Way Forward for [116][337] TRP_TRS_MIMO_OTA_Ph3, vivo, RAN4#116.

- R4-2512613 Template for Rel-19 1Tx device information collection, Telecom Italia S.p.A., Vodafone, Deutsch Telekom, Orange, RAN4#116.

- R4-2410765, WF for [111][336] NR_FR1_TRP_TRS_enh, vivo, RAN4#111 meeting.