# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513601

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.13.2.2

Source:	Apple

Title:	On RRM Performance Requirements for other RRM requirements in MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#116 the RRM performance requirements for Rel-19 MIMO Phase 5 was discussed and way forward [1] was agreed. In this contribution we present our views on the performance requirements impact and test cases for CJT calibration reporting and asymmetrical DL sTRP/ UL mTRP.

# 2. Discussion

### CJT Calibration Reporting

In RAN4#116 the following agreements were made in [1] for performance requirements for CJT calibration reporting.

#### Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? Agreement: - RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P. #### Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P? Agreement: - RAN4 to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, further discuss the accuracy and SNR in the performance part.

In core requirements we have collected simulation results to determine if we need to define requirements with > 1 sample for measurement. Based on the results, the accuracy is diverse among companies.

- Based on the results collected for FO/ TO accuracy the results are very diverse even at higher SNR.

RAN4 should discuss feasibility of defining accuracy requirements for delay and frequency offset reporting for CJT calibration. Given the large span in results it is recommended that we align simulation assumptions and identify possible cause of results misalignment to aid defining accuracy requirements.

RAN4 to discuss feasibility of defining accuracy requirements for time offset frequency offset reporting based on 1 sample measurement.

Align simulation assumptions to identify cause of results misalignment to aid defining accuracy requirements.

The test scope can be discussed based on the table from [1] below:

| High level: | QC | Apple | Samsung | CMCC | Nokia | Ericsson | MTK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify CJTC-Dd, measurement period | Y | FFS | Y | Y | Y | Y | Y |
| Whether to define TCs to verify CJTC-f，measurement period | Y | FFS | Y | Y | Y | Y | Y |
| Whether to define TCs to verify CJTC-P， (To be verify?) | N | N | N |  |  |  |  |
| Whether to define TCs to verify scheduling restriction |  |  |  | Y (if core req.) |  |  |  |
| Whether to define TCs to verify CJTC-Dd, CJTC-f，accuracy |  |  |  |  | Y | Y |  |

Since the agreement is to define requirements based on single sample measurement, we don’t see the necessity to define requirement for measurement period for delay and frequency offset reporting. There is also no need to define requirements for scheduling restriction for CJT calibration reporting and this is the same as any CSI-RS-based L1 measurement.

For CJT-C do not define test cases for measurement period since requirements are based on single sample measurement.

Do not introduce any test cases for scheduling restriction.

### Asymmetric DL sTRP/ UL mTRP

RAN4 has agreed to define timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP. The following scenarios are considered:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

On test scope the following table from [1] is used as a starting point –

| High level: | QC | Apple | Samsung | CMCC | Huawei | Nokia | MTK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify timing requirements? |  | Y | Y | Y | Y | Y |  |
| Whether to define TCs to verify TCI state switching requirements? | Y |  | N |  |  |  |  |
| FR1 and/or FR2? | both |  |  |  |  |  |  |
| Whether to cover Scenario#1 and Scenario#2? | Both for FR1. Scenario#2 for FR2 | Both |  | Both | Scenarios#1 | both | Scenarios#1 |

RAN4 shall introduce timing requirements for both scenarios 1 and 2 for 2TA with asymmetrical DL sTRP/UL mTRP. Since the requirements are different, we see the necessity to introduce test cases for both scenario 1 and scenario 2. But it is to be noted that scenario 2 requirements are the same as mTRP requirements with 2 TA.

RAN4 to discuss defining test cases for timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP.

Test cases to cover both scenario 1 and scenario 2.

TCs for TCI state switching delay

In RAN4#115 it was agreed that existing requirements for unified TCI state switching delay for mTRP are applicable for scenario 2. We don’t see the necessity to introduce est case when no new requirement is introduced for asymmetrical DL sTRP/ UL mTRP use case.

Do not introduce test cases for TCI state switching for asymmetrical DL sTRP/ UL mTRP.

Applicable frequency range

The use case and requirements for asymmetrical DL sTRP/ UL mTRP are applicable to both FR1 and FR2. In practice FR2 would most likely use Scenario#2 rather than scenario 1. Instead of introducing test cases for both FR1 and FR2 for both scenarios, define requirements for scenario 1 in FR1 and scenario 2 in FR2.

Introduce test cases for scenario 1 in FR1 and scenario 2 in FR2.

# 3. Conclusion

In this paper, we present our views on the performance part impact and test cases for CJT calibration reporting and asymmetrical DL sTRP/ UL mTRP. Our observations and proposals are captured below:

CJT Calibration Reporting

- Based on the results collected for FO/ TO accuracy the results are very diverse even at higher SNR.

- RAN4 to discuss feasibility of defining accuracy requirements for time offset frequency offset reporting based on 1 sample measurement.

Align simulation assumptions to identify cause of results misalignment to aid defining accuracy requirements.

For CJT-C do not define test cases for measurement period since requirements are based on single sample measurement.

Do not introduce any test cases for scheduling restriction.

Asymmetric DL sTRP/ UL mTRP

RAN4 to discuss defining test cases for timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP.

Test cases to cover both scenario 1 and scenario 2.

Do not introduce test cases for TCI state switching for asymmetrical DL sTRP/ UL mTRP.

Introduce test cases for scenario 1 in FR1 and scenario 2 in FR2.

# Reference

- R4-2512130, “WF on RRM requirements for NR_MIMO_Ph5_Part1”, Samsung.