# 3GPP TSG-RAN WG4 Meeting #116bis                             R4-2513653

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.10.2.1

Source: 	CMCC

Title: 			Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN #103 meeting, the WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- Objective of core part - FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Objective of performance part - Specify RRM performance requirements and test cases for the Core part enhancements.

There was discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode, and a WF was approved [2]. This contribution provides discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor.

- Discussion

Issue 2-1-1: baseline setup for BSF test case - Proposal 1(Samsung): - For the AoA setup of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, RAN4 to discuss whether AoA Setup #5 can be considered. - To define the new test case of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, the test requirement should be defined by using serving cell’s quality based triggering condition - How to define the test parameters involving hysteresis and the TTT/evaluation period of the timer can wait for further solid conclusions. - [Way forward] - Continue discussion in RAN4#116bis. Issue 2-1-2: whether or not to test FBS quitting - Option 1(CMCC): it is proposed to discuss whether to test FBS quiting when FBS conditions are met, e.g. whether to test that UE fallback to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition. - [Way forward] - Continue discussion in RAN4#116bis.

In RAN4#114bis meeting, it was agreed not to test UE no using FBS for scenario 1~4 if the FBS condition is not met. In RAN4#115 meeting, there was discussion about quiting FBS even if FBS condition is met. The agreements are duplicated as following.

Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1.

Even if FBS condition is met, UE can fallback back to normal measurement when it has performed fast measurement for at least T1. Since the UE behaviour is different, it is proposed to discuss whether to introduce test case for quitting mechanism when FBS conditions are met. On one hand, it is better to introduce test case to verify the performance. On the other hand, the mechanism is that UE is allowed to fallback to normal measurement, which means that UE can choose not to fall back and continue to perform FBS after T1. we re open for discussion, and would like to hear companies’ views.

Proposal 1: it is proposed to discuss whether to test FBS quitting when FBS conditions are met, e.g. whether to test that UE fallback to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition.

- Conclusion

This contribution provides discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode. The proposals are:

Proposal 1: it is proposed to discuss whether to test FBS quitting when FBS conditions are met, e.g. whether to test that UE fallback to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition.

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2512124, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2508264, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements