# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513654

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.9.2

Source: 	CMCC

Title: 			Discussion on general aspects for enhancement on AI/ML for NR air interface

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements was approved [1]. The objectives for RAN4 are duplicated as following.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

This contribution provides discussion on general aspects.

- Discussion

In Rel-19 study on CSI compression, option 3 and option 4 are considered

- Option 3: Full decoder specification in standard (a test decoder is standardized for option 3)

- Option 4: TE vendor provides the decoder

- Option 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

- Option 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

- Option 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

For option 3, it further agreed to define the reference encoder including encoder structure and parameters, to verify the performance of CSI compression test decoder [2]. Dataset is specified for option 4a, furthermore, reference decoder and reference encoder need to be captured. For option 4a, for the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b).

According to the outcome of Rel-19 study, both option 3 track 2, and option 4a or 4b are considered for the WI. Option 3 and Option 4 are similar. Both need to capture encoder, decoder and training dataset. The difference is that for option 3, the TE vendor is mandated to use the standardized test decoder. In option 4 they could choose to use the reference test decoder or to create their own based on the standardized information.

One of the issues identified to be discussed in WI is to discuss and align on assumptions for the test decoder and reference encoder structures [3].

During Rel-19 RAN4 study, for the simulations for checking the feasibility of above testing options for CSI compression and prediction, AI/ML model backbone of CNN is in use. Different encoder (Transformer, CNN and MLP) are evaluated with the reference decoder based on CNN.

In RAN1 #119 meeting, it was agreed to adopt Transformer as the backbone structure for temporal domain Case 0 in case of take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output. Following Figure is the encoder structure considered in RAN1 as the example in the study for Case 0 and taking spatial-frequency domain precoding matrix as input.

Figure 1: Encoder structure example

Encoder description in RAN1 study is elaborated in Table 1. The decoder has a mirroring design as the encoder.

Table 1: Encoder description for CSI feedback using two-sided model

| Domain | Parameters |
| --- | --- |
| Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer) | Number of tokens Feature dimension of each token Output dimension dmodel per token |
| Positional encoding | Number of tokens positions Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nℎead Dimension of attention head dℎead Dimension of latent space inside feedforward module dFF Activation function choice |
| Output linear layer | Reshape the matrix to a vector Output dimension zdim |
| Quantization | Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit∗Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit∗Nseg - Number of segments = Zdim/Nseg - Total payload size for Vector quantization = Number of segments * bits per segment |

According to the WID, RAN1 scalability study outcome need to be taken into account to define/specify reference model. Since RAN1 take Transformer as baseline, it is proposed to consider Transformer for RAN4 discussion, and example structure and parameters in RAN1 can be used as baseline.

Proposal 1: it is proposed to consider transformer for RAN4 discussion, and the example structure and parameters in RAN1 can be used as baseline.

Another issue is how many test decoders and encoders are needed [3]. This issue is related with scalability discussion. In Rel-19 study, for the model structure feasibility study of Case 0 with precoding matrix in spatial-frequency domain as input, RAN1 consider following alternatives on scaling over token dimension (e.g., number of subbands, number of Tx ports, or combination thereof), feature dimension (e.g., number of Tx ports, number of subbands, or combination thereof), and payload configurations are studied. RAN1 concludes that the scalable model structure specification is feasible for sub-option 3a-1 of Direction A, and the scalable model structure specification and model parameters specification is feasible for Direction C. With good scalability, the number of test decoders and encoders can be reduced.

TR 38.843 - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side.

In Rel-19, 1-1 joint training, training based on the Samsung frozen decoder, “N-to-1 Multi-encoder structure aware training” are simulated in RAN4. The conclusion is that if a high complexity test decoder is used, it seems to be compatible with different encoder backbones (at least CNN and transformer). Also to be noted that Whether a low complexity decoder can be compatible with different encoder structures was not studied. The benefits of N-to-1 Multi-encoder structure aware training are not fully clear, however it may improve the performance of the lowest complexity MLP encoder. In Rel-20, RAN4 can further perform the simulation, which is helpful in deciding the number of decoders.

Proposal 2: the number of test decoders and encoders can be further discussed based on the scalability outcome and simulation results.

- Conclusion

This contribution provides discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models. The proposals are:

Proposal 1: it is proposed to consider transformer for RAN4 discussion, and the example structure and parameters in RAN1 can be used as baseline.

Proposal 2: the number of test decoders and encoders can be further discussed based on the scalability outcome and simulation results.

- Reference

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

- R4-2503953, WF on AI/ML air interface study

- R4-2511849, WF on [116][132] NR_AIML_air_part2, Ericsson

[4] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface