# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513659

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.12.3

Source: 	CMCC

Title: 			 Discussion on RRM requirements for RRM measurement prediction

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [1]. In RAN #109 meeting, the revised WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [2]. The objectives for RAN4 are duplicated as following.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any

This contribution provides discussion on RRM requirements for RRM measurement prediction.

- Discussion

According to the WID, for RRM measurement prediction, temporal domain prediction case A, temporal domain prediction case B, and frequency domain prediction for co-located cases are the main target. These three use cases are also prioritized during the study phase.

Both absolute and relative accuracy of predicted L3-RSRP are considered as the candidate metrics for RRM measurement prediction use cases for intra-frequency scenario and inter-frequency scenarios.

The absolute accuracy of predicted L3-RSRP is defined as:

- Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP.

The relative accuracy of predicted L3-RSRP for intra-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2),

  - cell 1 and cell 2 are on the same frequency

  - the reported L3-RSRP of cell 2 can be measured or predicted.

The relative accuracy of predicted L3-RSRP for inter-frequency prediction is defined as:

- Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported L3-RSRP of cell 2) – (ground truth of L3-RSRP of cell 1 – ground truth of L3-RSRP of cell 2),

  - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1

  - the reported L3-RSRP of cell 2 can be measured or predicted.

As captured in the TR [3], it is not precluded to update the definition based on further RAN2 progress in WI phase. Since RAN2 work has not started, we can keep the current definition, and further check based on RAN2 progress.

One issue is the ground truth. As a baseline, the ground truth of L3-RSRP in FR2 is defined as the reported L3-RSRP measurement result under sufficient high SNR. But the ground truth of L3-RSRP in FR1 is open.

The ground truth of L3-RSRP in FR1 is based on:

- Alt1: The transmitted or reception power

- Alt2: The reported measurement value under certain conditions.

Both alternatives have pros and cons. Table 1 provide the comparison between alternative 1 and alternative 2. Alternative 1 is the legacy way. However, some issues are identified. In legacy FR1 test case set up, no L3 filtering is assumed and AWGN channel is in use. For AI test, fading channel will be employed, which means there is channel fluctuation in the time domain. If we go with alternative 1 (Ground truth for FR1 will be based on the transmitted or reception power), since UE perform L1/L3 filtering will have impact on the ideal RSRP and TE has no such information, it is difficult for TE to derive the ground truth. While for alternative 2 (Ground truth for FR1 will be based on the reported measurement value under certain conditions), issues are also observed. Considering the channel is fluctuated, the channel condition to get ground truth and the channel condition to predict is different, which will result in mismatch. And there is cheating issue since both the ground truth and predicted value are reported by UE. It is necessary to further discuss the ground truth for FR1 during the WI phase.

- Table 1 Comparison between Alternative 1 and Alternative 2 for ground truth definition for FR1

|  | - Alternative 1 - (Ground truth for FR1 will be based on the transmitted or reception power) | - Alternative 2 - (Ground truth for FR1 will be based on the reported measurement value under certain conditions) |
| --- | --- | --- |
| - Impact of L3 filtering | - If L3 filtering is considered, how to perform L3 filtering is based on L3 filtering formula with filtering coefficient configured, and TE has no such information | - N/A |
| - Impact of L1 filtering | - How to perform L1 filtering, e,g, the number of samples, whether sliding or non-sliding filtering in use, is up to UE implementation, and TE has no such information | - N/A |
| - Timing window | - N/A | - For Case B (target goal is to reduce measurement overhead), the time instance in prediction window will not be measured, which means UE cannot measure and report the ground truth. |
| - Channel condition | - N/A | - For fading channel, the channel condition at T1 to get ground truth and the channel condition at T2 to predict is different, which result in mismatch |
| - UE cheating issue | - N/A | - Since both the ground truth and predicted value are reported by UE, there may be cheating issue |

Proposal 1: the ground truth of L3-RSRP in FR1 need to be further discussed and determined in the work item phase.

In study phase, RAN2 agreed the KPIs about measurement reduction rate, which is defined in the temporal domain (called MRRT) by assuming same length of measurement time instances and spatial domain respectively(called MRRS).

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

In legacy, there is no RAN4 requirements on measurement reduction, whether to introduce related requirements for AI/ML based mobility can be further discussed.

Another consideration is that the measurement delay requirements may also need to be newly defined for some scenarios. Taking FR1 inter-frequency inter-cell prediction as an example, prediction from low-frequency cell to high-frequency cell is expected, which means that not all configured MOs need to be measured. The legacy measurement delay is scaled with the number of MOs, which need to be updated to only consider the actually measured frequency layers. As for the intra-frequency temporal domain prediction, e.g. case B, the legacy measurement delay also need to be updated. In legacy, SMTC or MGRP is in use in the measurement delay requirements, while for temporal domain prediction, UE only measure subset of SMTC/MGRP, and predict the results of other measurement occasions.

Proposal 2: for RRM measurement prediction, the measurement delay requirements need to be defined/updated

- For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy

- For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements

- Conclusion

This contribution provides discussion on RRM requirements for measurement event prediction. The proposals are:

Proposal 1: the ground truth of L3-RSRP in FR1 need to be further discussed and determined in the work item phase.

Proposal 2: for RRM measurement prediction, the measurement delay requirements need to be defined/updated

- For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy

- For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measured, which means that only the actually measured frequency layers are considered in the measurement delay requirements

- Reference

- RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR