# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513667

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.14.4

Source: 	CMCC

Title: 			 Discussion on demodulation performance requirements for device 1

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised WID on Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. According to the WID, the objectives of RAN4 scope are duplicated as following.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In last meeting, there is discussion on demodulation performance requirements for A-IoT device 1, and a WF was approved [2]. This contribution provides discussion on this topic.

- Discussion

Issue 1-1: Deployment and topology - Agreement - The assumed deployment and topology D1T1-B for A-IoT demodulation requirements discussion in R19 - No need to capture it in RAN4 specification Issue 1-2: Operation mode - Agreement - Standalone and in-band operation for Device 1 - Same demodulation performance requirements for standalone and in-band operation. - How to capture it in the specification can be discussed during the CR drafting if necessary - Standalone operation for BS Issue 2-1: Whether to define PRDCH demodulation requirements for Ambient-IoT - Way forward - Option 1: Define PRDCH demodulation requirement. - Option 2: Not define PRDCH demodulation requirements for Ambient-IoT considering that it can be verified by the RF REFSENS requirement

For the open issue on whether to define PRDCH demodulation requirements for Ambient-IoT device, we prefer to  follow legacy way to define PRDCH demodulation requirement to guarantee the performance. And the test procedure in RF can be used as baseline to define demodulation tests.

Proposal 1: it is proposed to define PRDCH demodulation requirements for Ambient-IoT device to guarantee the performance.

Issue 2-2: Test methodology - Agreement - 10% BLER with BLER=1− Number of responsed Message 1Number of transmitted Message 0 or BLER=1− Number of responsed Message 3Number of transmitted Message 2 for testing - During countering the number of responded Message 1 or 3, the TE should check if the responded Message 1 or Message 3 is the correct response of Message 0 or Message 2 by checking the device ID - Further discuss the message type to be tested

- BLER=1− Number of responsed Message 1Number of transmitted Message 0  or BLER=1− Number of responsed Message 3Number of transmitted Message 2  are the similar, no need to have both. To simplify the test procedure, BLER=1− Number of responsed Message 1Number of transmitted Message 0  is preferred.

Proposal 2: no need to consider both BLER=1− Number of responsed Message 1Number of transmitted Message 0  and BLER=1− Number of responsed Message 3Number of transmitted Message 2  , it is preferred to adopt BLER=1− Number of responsed Message 1Number of transmitted Message 0  .

Issue 2-10: M-chips for OOK - Way forward - Option 1: 6 - Option 2: 24 - Other options are not precluded

According to TR38.769, minimum of PRB is associated with M value, as shown in following table 1. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. To reduce test time, M value of 24 is preferred.

Table 1:  M values and the associated minimum Btx,R2D value

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

Proposal 3: it is proposed to use M value of 24 to define PRDCH requirements.

Issue 2-12: Channel Bandwidth - Way forward - Option 1: 2 PRBs - Option 2: 3 PRBs - Other options are not precluded

As shown in above Table 1, if we adopt M value of 24, the corresponding number of PRBs is 3.

Proposal 4: it is proposed to use 3 PRBs to define PRDCH requirements.

Issue 2-13: Channel model - Way forward - Option 1: TDLA30-10 - Other options are not precluded - TE vendors are encouraged to check the feasibility to emulate TDL channel in the chamber for Device 1 testing

According to the WID, for Rel-19 Ambient IoT, the target scenario is indoor deployment scenario (D1). it is expected that device is stationary or in very low mobility. It is valid to adopt TDLA30-10 to define PRDCH demodulation requirements. If TE vendors confirm that it is feasible to emulate TDL channel in the chamber for Device 1 testing, it is proposed to use TDLA30-10 as channel model.

Proposal 5: it is proposed to adopt TDLA30-10 to define PRDCH requirements.

Issue 2-14: Specification structure (if specified) - Way forward - Option 1: RAN4 should specify the Ambient IoT device demodulation requirements in “Ambient IoT device radio transmission and reception” - Option 2: The demodulation requirements can be captured in clause 10 of TS 38.191

We do not think these two options are contradict. TS 38.191 is for Ambient-IoT device radio transmission and reception, and clause 10 is reserved for demodulation.

Proposal 6: the demodulation requirements for A-IoT device is captured in clause 10 of TS 38.191.

- Conclusion

This contribution provides discussion on demodulation performance requirements for A-IoT device. The proposals are:

Proposal 1: it is proposed to define PRDCH demodulation requirements for Ambient-IoT device to guarantee the performance.

Proposal 2: no need to consider both BLER=1− Number of responsed Message 1Number of transmitted Message 0  and BLER=1− Number of responsed Message 3Number of transmitted Message 2  , it is preferred to adopt BLER=1− Number of responsed Message 1Number of transmitted Message 0  .

Proposal 3: it is proposed to use M value of 24 to define PRDCH requirements.

Proposal 4: it is proposed to use 3 PRBs to define PRDCH requirements.

Proposal 5: it is proposed to adopt TDLA30-10 to define PRDCH requirements.

Proposal 6: the demodulation requirements for A-IoT device is captured in clause 10 of TS 38.191.

- Reference

- RP-250796, Revised WID: New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc

- R4-2512603, Way Forward for [116][335] A-IoT_demod, Huawei, CMCC