# 3GPP TSG-RAN WG4 Meeting #116bis		  R4-2513692

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	7.7.3

Source:		Qualcomm Incorporated

Title:			Views on UAV coexistence

Document for:	Approval

# 1  Introduction

New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) has been approved in RAN#109 [1] with the objective of analyzing the coexistence for UAV with higher transmit power.

- Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

In this paper, we provide our view on UAV coexistence.

# 2  Discussion

## 2.1 Coexistence scenarios

The coexistence scenarios can be listed as below with the assumption of the synchronized scenario. For FDD band, 2GHz is considered as carrier frequency. For TDD band, 4GHz is considered as carrier frequency.

|  | Aggressor |  | Victim |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | deployment scenario UL/DL | System BW duplex mode | deployment scenario UL/DL | System BW duplex mode | Simulation frequency | ACLR/ACS requirements |
| 1 | TN UL | 100MHz TDD | UAV UL | 100MHz TDD | 4 GHz | UAV BS ACS |
| 2 | UAV DL | 100MHz TDD | TN DL | 100MHz TDD | 4 GHz | UAV BS ACLR |
| 3 | TN DL | 100MHz TDD | UAV DL | 100MHz TDD | 4 GHz | UAV UE ACS |
| 4 | UAV UL | 100MHz TDD | TN UL | 100MHz TDD | 4 GHz | UAV UE ACLR |
| 5 | TN UL | 20MHz FDD | UAV UL | 20MHz FDD | 2 GHz | UAV BS ACS |
| 6 | UAV DL | 20MHz FDD | TN DL | 20MHz FDD | 2 GHz | UAV BS ACLR |
| 7 | TN DL | 20MHz FDD | UAV DL | 20MHz FDD | 2 GHz | UAV UE ACS |
| 8 | UAV UL | 20MHz FDD | TN UL | 20MHz FDD | 2 GHz | UAV UE ACLR |

## 2.2 Assumptions for coexistence

The preliminary coexistence analysis is provided in [2] and it is suggested that more study is needed in work item phase for the scenario of UAV UL is interfering to TN UL.

To analyze the coexistence issue for the case of UAV UL interfering to TN UL, the following system assumptions are considered as the starting point.

Table 1: Parameters for UAV coexistence analysis

| UAV BS altitude (co-located with TN BS) | 30m |
| --- | --- |
| Carrier frequency | 2GHz |
| Channel bandwidth | 5MHz~100MHz per UE |
| UAV UE altitude | 5-~100m |
| ISD of UAV BS | ~4000m |
| ISD of TN BS | 500m for Uma, 1732m for RMA |
| UAV UE max Tx power | 33dBm |
| Note 1: In below analysis, we assume 5MHz CBW for both NR UE and UAV UE. Note 2: In below analysis, we consider 100m as the UAV UE altitude. |  |

### 6.2.3.1 UAV BS antenna model

For UAV BS with AAS antenna, the following BS model could be used for 2GHz carrier frequency referring to ATG BS antenna model defined in section 6.2.3 of TR 38.876.

Table 2: AAS Antenna parameters for UAV BS (non sub-array model)

| UAV BS Parameter | Values for evaluation |
| --- | --- |
| Antenna pattern | TR 38.921 |
| Element gain (dBi) | 7.1 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 8 × 8 elements |
| Number of supported polarizations, P | 2 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 1) | 28 for 2GHz 35 for 4GHz |
| Base station maximum coverage angle in the horizontal plane (degrees) | 120 |
| Base station vertical coverage range (degrees) (Note 2) | 25 |
| Note 1: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 2: The vertical coverage range includes the mechanical downtilt. |  |

For UAV BS with non-AAS antenna which has similar antenna pattern as sub-away AAS model, the following antenna gain and antenna pattern could be used for 2GHz carrier frequency referring to NR BS antenna model defined in section 6.2.2.2 of TR 38.769.

Table 3: Non-AAS Antenna parameters for UAV BS NR

| UAV BS Parameter | Values for evaluation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:15) |

### 6.2.3.2 UAV UE antenna model

For 2GHz, we assume that UE is equipped with omni-directional antennal and assume 33dBm UE output.

For 4GHz, we assume that UE is equipped with directional antenna (4*1*2, 5dBi element gain) and assume EIRP of 47dBm (Antenna model defined in section 6.2.3.2 of TR 38.876 can be reused).

### 6.2.3.3 Propagation model

The free space path loss (FSPL) for a separation distance d  in meter and frequency fc in GHz is used for propagation between UAV UE and UAV BS:

The UMa propagation model is used to calculate the pathloss between TN BS and TN UE for Uma scenario which is referring to TR 38.901. It’s noted that we assume all TN UEs are outdoor and only NLOS scenario is considered for simplification.

The RMa propagation model is used to calculate the pathloss between TN BS and TN UE for RMa scenario which is referring to TR 38.901. It’s noted that we assume all TN UEs are outdoor and only NLOS scenario is considered for simplification.

## 2.2 Coexistence analysis

As shown in Figure 1, we use [Horizontal distance (m), Height (m)] to indicate the location of TN BS, TN UE, UAV BS and UAV UE.

UAV UE at [3750, 100] is accessing to the BS at [0, 0] which is TN and UAV co-located BS from operator A. For other BSs, they are the victim TN BS from operator B. The preliminary coexistence analysis is provided in Table 4 (Note that 750m ISD is considered for the analysis which is middle value between Uma and RMa ISD).

Figure 1: Coexistence scenario for UAV UL interfering TN UL

Table 4: Preliminary coexistence analysis for UAV

| Frequency (GHz) | UAV UE Transmit power + UE antenna gain (dBm) | UAV location [Horizontal distance (m), Height (m)] | Victim TN BS location [Horizontal distance (m), Height (m)] | Pathloss between UAV UE and TN BS (dB) | Azimuth angle relative to the TN BS (degree) | Estimated BS Antenna gain between UAV UE and TN BS (dBi) referring to Annex A/B | Interference level from UAV UE to TN BS (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 33 | [3750, 100] | [750, 30] | 168 | 1.3 | AAS: 14 | -121 |
|  |  |  |  |  |  | Non-AAS:13 | -122 |
| 2 | 33 | [3750, 100] | [1500, 30] | 166 | 1.8 | AAS: 12 | -121 |
|  |  |  |  |  |  | Non-AAS:12 | -121 |
| 2 | 33 | [3750, 100] | [2250, 30] | 162 | 2.7 | AAS: 10 | -119 |
|  |  |  |  |  |  | Non-AAS:11 | -118 |
| 2 | 33 | [3750, 100] | [3000, 30] | 156 | 5.3 | AAS: 2 | -121 |
|  |  |  |  |  |  | Non-AAS:7 | -116 |
| 2 | 47 | [3750, 100] | [750, 30] | 168 | 1.3 | AAS: 14 | -107 |
|  |  |  |  |  |  | Non-AAS:13 | -108 |
| 2 | 47 | [3750, 100] | [1500, 30] | 166 | 1.8 | AAS: 12 | -107 |
|  |  |  |  |  |  | Non-AAS:12 | -107 |
| 2 | 47 | [3750, 100] | [2250, 30] | 162 | 2.7 | AAS: 10 | -105 |
|  |  |  |  |  |  | Non-AAS:11 | -104 |
| 2 | 47 | [3750, 100] | [3000, 30] | 156 | 5.3 | AAS: 2 | -107 |
|  |  |  |  |  |  | Non-AAS:7 | -102 |
| Note 1: The downtilt of TN only BS is 3degree. The downtilt of UAV and TN co-located BS is FFS. |  |  |  |  |  |  |  |

For UMa the victim TN BS at 2GHz, the TN UE UL received power at TN BS could be calculated as following:

23dBm + 0dB (UE antenna gain) – 132dB (UMa-NLOS pathloss) +15dBi (Maximum BS antenna gain of non-AAS) - 8dB (shadow fading)= -102dBm

23dBm + 0dB (UE antenna gain) – 132dB (UMa-NLOS pathloss) +18dBi (Maximum BS antenna gain of AAS) - 8dB (shadow fading)= -99dBm

For RMa the victim TN BS at 2GHz, the TN UE UL received power at TN BS could be calculated as following:

23dBm + 0dB (UE antenna gain) – 142dB (UMa-NLOS pathloss) +15dBi (Maximum BS antenna gain of non-AAS) - 8dB (shadow fading)= -112dBm

23dBm + 0dB (UE antenna gain) – 142dB (UMa-NLOS pathloss) +18dBi (Maximum BS antenna gain of AAS) - 8dB (shadow fading)= -109dBm

The noise floor at BS receiver with 5MHz CBW is -102dBm assuming 5dB noise figure at BS.

From the preliminary analysis in Table 4, we have the following observations:

Observation 1: For both AAS and non-AAS UAV BS, the interference from UAV UE UL with directional antenna to TN BS UL is comparable to the useful signal strength of TN UL at some of TN BS locations. And the interference is also comparable to the noise floor at TN BS. The interference issue in RMa is more serious than UMa.

Observation 2: For both AAS and non-AAS UAV BS, the interference from UAV UE UL with omni- antenna to TN BS UL is much lower than to the useful signal strength of TN UL and noise floor at TN BS.

Observation 3. The UL coverage for UAV UE with both omni-antenna and directional antenna is very limited assuming that the downtilt of co-located UAV BS and TN BS is 3degree. With optimized UAV BS downtilt or uptilt, the further analysis for coexistence between UAV UL to TN UL is needed.

With above observations, we have the following proposals:

Proposal 1: RAN4 to conduct the coexistence simulation at least for the scenario of UAV UL interfering TN UL. The simulation assumptions used in section 6.2.3.1, 6.2.3.2 and 6.2.3.3 could be considered as the starting point for coexistence simulation.

- For UAV BS antenna pattern, AAS with non-subarray, AAS with subarray, and non AAS should be considered.

- Both RMa and UMa should be considered

Proposal 2: RAN4 to evaluate and refine key deployment assumptions such as downtilt or uptilt angles, ISD of UAV BS for both aggressor and victim operators, UAV height, to ensure adequate UAV coverage UAV in coexistence simulation.

# 3. Conclusion

In summary, we have following observations and proposals.

Observation 1: For both AAS and non-AAS UAV BS, the interference from UAV UE UL with directional antenna to TN BS UL is comparable to the useful signal strength of TN UL at some of TN BS locations. And the interference is also comparable to the noise floor at TN BS. The interference issue in RMa is more serious than UMa.

Observation 2: For both AAS and non-AAS UAV BS, the interference from UAV UE UL with omni- antenna to TN BS UL is much lower than to the useful signal strength of TN UL and noise floor at TN BS.

Observation 3. The UL coverage for UAV UE with both omni-antenna and directional antenna is very limited assuming that the downtilt of co-located UAV BS and TN BS is 3degree. With optimized UAV BS downtilt or uptilt, the further analysis for coexistence between UAV UL to TN UL is needed.

Proposal 1: RAN4 to conduct the coexistence simulation at least for the scenario of UAV UL interfering TN UL. The simulation assumptions used in section 6.2.3.1, 6.2.3.2 and 6.2.3.3 could be considered as the starting point for coexistence simulation.

- For UAV BS antenna pattern, AAS with non-subarray, AAS with subarray, and non AAS should be considered.

- Both RMa and UMa should be considered

Proposal 2: RAN4 to evaluate and refine key deployment assumptions such as downtilt or uptilt angles, ISD of UAV BS for both aggressor and victim operators, UAV height, to ensure adequate UAV coverage UAV in coexistence simulation.

# 4  References

- RP-252908, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), RAN #109, September 2025

- RP-252600, Support of high power UAV UE in Rel-20, RAN #109, September 2025

# Annex A: Atenna pattern for AAS UAV BS

# Annex B: Atenna pattern for non-AAS UAV BS