3GPP TSG-RAN WG4 Meeting #116bis	R4-2513712

# Prague, CZ, October 13rd-17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on CSSF optimization solutions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The draft Big CR was endorsed in R4-2512348 in RAN4#116 meeting. In 9.2.3.2, according to the draft Big CR, the selected SCC is determined by UE implementation, if there are multiple SCCs on which UE is configured to report SSB based measurements in this FR2 band and network does NOT indicate the specific SCC. However, in this case, UE shall follow the legacy requirements and select one of SCCs configured with measurement reporting. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is based on the latest draft spec and the new change is marked with “Nokia_116bis”. In 9.2.3.2, changing the rule under “otherwise” to legacy text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requirements for CSSF optimization solutions are not completely correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | NA |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

#### 9.2.3.2	Requirements for FR2

For UE supporting CSSF enhancement for one serving CC measurement per-band, if UE receives network indication via enable one serving carrier measurement in a band to enable one serving carrier measurement for an FR2 band,

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

- 	PCC, when UE is configured with SA NR or NE-DC operation mode with PCC in the band; or

-	PSCC, when UE is configured with EN-DC with PSCC in the band; or

-	PSCC, when UE is configured with NR-DC with PSCC in the band; or

-	an SCC, if such SCC is the only one SCC on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band; or

-	an SCC, indicated by network via enable one serving carrier measurement in a band, if such SCC is one of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band;

-	otherwise

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise one SCC determined by UE implementation.

UE is not required to perform SS-RSRP, SS-RSRQ, and SS-SINR measurements on both serving and neighbour cell for each of the other intra-frequency layer(s) in the same band.

For UE not supporting CSSF enhancement for one serving CC measurement per-band, or UE supporting CSSF enhancement for one serving CC measurement per-band but does not receive network indication via enable one serving carrier measurement in a band to enable one serving carrier measurement for each FR2 band,

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

<End of Change>