3GPP TSG-RAN WG4 Meeting #116bis	R4-2513715

Prague, Czech Republic, 13– 17 October 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SBFD operation

Agenda item:	4.19.3

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made [1]:

### Sub-topic 2-1: RRM impacts of SBFD operation #### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements (Monday online session) - For UE supporting simultaneous DL reception in 2 DL subbands, - For measurement period: - For UE indicating CSI processing time scaled by 2, - Add a lower bound of 8ms on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2. - For UE not indicating CSI processing time scaled by 2, - Lower bound is not needed and existing measurement period applies - Further discuss the measurement accuracy requirements in Perf part of the WI - For UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB for Case 3 apply. - Option 2: accuracy requirements based on 24 RB for Case 3 apply. - For UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB for Case 3 apply. - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase. #### Issue 2-1-1d: Lmax in NR-U approach - Agreements (Thursday online session) - Not define Lmax values, and not define the applicability condition based on upper bound for measurement period. #### Issue 2-1-2: Requirements for RACH - Agreements - Discuss whether and how to update RACH requirements based on RAN2 spec in maintenance phase. #### Issue 2-1-3: SCell activation - Agreements (Monday online session) - The existing SCell activation delay requirement apply when activating a SBFD SCell. Further discuss whether any clarification is needed in the CR. - Note: the agreement can be revised if not aligned with RAN1 existing and future agreements. #### Issue 2-1-4: Condition for SSB based measurement with basic UE operation - Agreements - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

RRM impact due to SBFD operation

Extension of the measurement period based on the NR-U approach

In the last RAN4 meeting, it has been agreed not to define Lmax values for the extension of the measurement period when the SRS or CSI-RS overlap with symbols of different type, or when the UE is dynamically scheduled in the UL in symbols where the CSI-RS is configured.

During the discussions, it has been raised that the measurement period might be excessively extended when it DRX is configured. This concern is valid because when defining the extension of the measurement period when DRX is configured, L1 has been defined as the number of DRX cycles in which at least one occasion of the reference signal is overlapping with symbols of different type or when the CSI-RS is overlapping with the dynamic UL transmission. One example of the definition is available below, copied from the Big CR endorsed in the last RAN4 meeting (R4-2512353, clause 9.x.2.5).

4> When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the L1-SRS-RSRP resource is overlapping with non-SBFD symbols during TL1-SRS-RSRP_measurement_period … 4> When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of theL1-SRS-RSRP resource is overlapping with SBFD symbols, during TL1-SRS-RSRP_measurement_period

The text has been inspired by the NR-U approach, in which the measurement period of SSB-based measurements is extended by the number of SMTC occasions not available during the measurement period. When DRX is configured, it is written in the requirements that: “When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle” (example from Table 9.2A.5.1-2). So it makes sense that the L1 extension considers the DRX cycles in which at least one SMTC occasion is not available.

In NR-U approach, it is assumed “When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle”.

As we mentioned in our contributions in the past few meetings, despite the extension of the measurement period being inspired by the NR-U approach, we see important differences between the cause for extension of the measurement periods in the Duplex Evolution feature, and in NR-U. UEs that support SBFD are fully aware of which samples are overlapping with symbols of different types, and which samples are overlapping with dynamically configured UL transmissions. Furthermore, both network and UE are aware of the DRX configuration.

Therefore, we propose that the definition of the extension of the measurement period, L1, is updated when DRX is configured, so that the measurement period is only extended when all the occasions of the measurement resources are overlapping with symbols of different type or with UL transmissions. The proposed change is captured in our draftCR R4-2513717 as a reference.

When DRX is configured, update the definition of the extension of the measurement period, L1, so that the measurement period is extended by the number of DRX cycles in which all the occasions of the measurement resources within the DRX-onDurationTimer are overlapping with symbols of different type or with UL transmissions.

Requirements for RACH

In RAN4 #116 it has been agreed that:

Issue 2-1-2: Requirements for RACH - Agreements - Discuss whether and how to update RACH requirements based on RAN2 spec in maintenance phase.

Here, we note that there are three main differences between the RACH procedure described for legacy UEs in RAN4 and the agreements in RAN2.

- Selection of the RO-type for PRACH transmission: a legacy UE shall have the capability of transmitting the Random Access Preamble on the next available PRACH occasion corresponding to the SSB measured with SS-RSRP above rsrp-ThresholdSSB. For UEs configured with SBFD RACH, the UE may select ROs in SBFD symbols or in UL symbols, depending on the measured SSB power and on the threshold sbfd-RSRP-ThresholdRO-Type configured by the gNB.

- Fall-back to a different RO-type: a SBFD UE shall switch to an RO with different type (SBFD or non-SBFD) when the preamble transmissions exceed preambleTransMaxRO-Type-r19.

- Calculation of the PRACH transmission power: if there is a fall-back to a RO of a different type, the UE needs to calculate the PRACH transmit power based on a new offset configured by the network.

The RACH procedure has changed for SBFD capable UEs regarding the selection of RACH opportunities, fall-back between ROs of different RO-type and calculation of the PRACH transmission power when there is a fall-back to a different RO type.

Considering the points highlighted above, the requirements on random access in clause 6.2.2 needs to be updated and clarified to support the SBFD operation. we have provided a CR with the proposed changes in R4-2513716.

The requirements for 4-step RA type in clause 6.2.2.2 are updated to take into account RACH procedure for UEs that support RACH in SBFD and UL symbols.

SCell activation

At RAN4#116 meeting, the following was agreed on SCell activation delay. It is not clear which existing SCell activation delay shall apply for SBFD SCell.

#### Issue 2-1-3: SCell activation - Agreements (Monday online session) - The existing SCell activation delay requirement apply when activating a SBFD SCell. Further discuss whether any clarification is needed in the CR. Note: the agreement can be revised if not aligned with RAN1 existing and future agreements.

In existing specification, we have defined the SCell activation delay for activating a downlink only SCell in TS 38.133 clause 8.3.2 and the SCell activation for activating a PUCCH SCell in TS 38.133 clause 8.3.7. The difference between the two is the UE needs to additionally activate the uplink for transmitting PUCCH on the PUCCH SCell hence additional procedures may be needed including random access procedure, PL-RS measurement etc.

When activating a SBFD SCell, the SBFD SCell is configured with both DL and UL subbands. Even PUCCH is not configured on the SBFD SCell, the network is expecting to schedule both DL and UL transmission on the SBFD SCell once it is activated. Otherwise, if SBFD SCell is activated as a downlink only SCell, the resource on the UL subband would be wasted. Therefore, we are expecting the SBFD SCell can be activated on both DL and UL subbands reusing the PUCCH SCell activation procedure. Some clarification needs to be added.

RAN4 to agree that SBFD SCell shall be activated on both DL and UL subbands reusing PUCCH SCell activation procedure.

Requirements for basic UE operation

In RAN4 discussion, the “basic” UE operation was used to indicate the UE not supporting simultaneous DL reception from 2 DL subbands i.e. it can receive only one DL subband. The RRM requirements on such UE basic UE operation was pending on the RAN1 conclusion of the UE capability.

#### Issue 2-1-4: Condition for SSB based measurement with basic UE operation - Agreements - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

However, in the latest RAN1 meeting, it was agreed that SBFD UEs support PDSCH and CSI-RS reception across two DL subbands as basic feature (FG 60-1). Hence it was understood such basic UE operation does not exist unless RAN1 makes further progress in maintenance phase. Therefore, RAN4 should assume UE always supports simultaneous DL reception from 2 DL subbands before RAN1 makes any further conclusion.

Agreement: - FG 60-1 to include support of PDSCH/CSI-RS reception in two DL subbands

RAN4 to assume UE always supports simultaneous DL reception from 2 DL subbands before RAN1 makes any further conclusion.

UE-to-UE CLI handling

On measurement restriction, the following was agreed when the measurement resources for L1-SRS-RSRP (Method#2) and L1-CLI-RSSI in DL subbands (Method#1) are partially or fully overlapping.

#### Issue 1-2-1: Collision between Method #1 and Method #2 - Agreement (Tuesday AH session) - UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol. Note: this can be revised if RAN1 have different agreement for this Collision between Method#1 and Method#2.

This issue is still under discussion in RAN1. From network point of view, our preference is to allow for simultaneous configuration of L1-CLI-RSSI for Method#1 and L1-SRS-RSRP for Method#2 on the same symbol. This will be beneficial for UEs capable of simultaneously receiving DL signals and measuring SRS-RSRP. Note that this is a UE capability already agreed in RAN1, as shown below.

Updated RAN1 UE features list for Rel-19 NR after RAN1 #122 60. NR_duplex_evo; 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling.

For the UEs not supporting such capability, we expect RAN1 to define collision handling rules to determine which one of the 2 measurements reports should be prioritized. We believe RAN4 shall define the measurement restrictions in line with the RAN1 discussion accordingly.

Depending on the measurement resource type and reporting type, the following collision/cases between CLI-RSSI and SRS-RSRP are expected as being discussed in RAN1:

|  |  | AP reporting of L1-CLI-RSSI |  |  | P reporting of L1-CLI-RSSI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | P CLI-RSSI resources | SP CLI-RSSI resources | AP CLI-RSSI resources | P CLI-RSSI resources | SP CLI-RSSI resources | AP CLI-RSSI resources |
| AP reporting of L1-SRS-RSRP | P L1-SRS-RSRP resources | Prioritize SRS-RSRP | Prioritize CLI-RSSI | Prioritize CLI-RSSI | Prioritize SRS-RSRP | N/A | N/A |
|  | SP L1-SRS-RSRP resources | Prioritize SRS-RSRP | Prioritize SRS-RSRP | Prioritize CLI-RSSI | Prioritize SRS-RSRP | N/A | N/A |
|  | AP L1-SRS-RSRP resources | Prioritize SRS-RSRP | Prioritize SRS-RSRP | Prioritize SRS-RSRP | Prioritize SRS-RSRP | N/A | N/A |

The table above includes the outcome after applying the proposed collision handling rules. The following criteria are used to determine whether CLI-RSSI or SRS-RSRP shall be prioritized:

- Aperiodic reporting shall be prioritized over periodic reporting

- Aperiodic resources shall be prioritized over periodic and semi-persistent resources

- Semi-persistent resources shall be prioritized over periodic resource

- SRS-RSRP resources shall be prioritized over CLI-RSSI resources when both have the same resource type

Therefore from RAN4 point of view, when measurement resources for L1-SRS-RSRP (Method#2) and L1-CLI-RSSI (Method#1) are partly or fully overlapping in time, the UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI and the above rules will apply when determining which one of the measurement resource is to be measured if this will be agreed in RAN1.

When measurement resources for L1-SRS-RSRP (Method#2) and L1-CLI-RSSI (Method#1) are partly or fully overlapping in time, the UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI and the following rules will apply when determining which one of the measurement resource is to be measured if this will be agreed in RAN1:

1)	Aperiodic reporting shall be prioritized over periodic reporting

2)	Aperiodic resources shall be prioritized over periodic and semi-persistent resources

3)	Semi-persistent resources shall be prioritized over periodic resource

4)	SRS-RSRP resources shall be prioritized over CLI-RSSI resources when both have the same resource type

Conclusion

The following Observations and Proposals were made:

- In NR-U approach, it is assumed “When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle”.

- When DRX is configured, update the definition of the extension of the measurement period, L1, so that the measurement period is extended by the number of DRX cycles in which all the occasions of the measurement resources within the DRX-onDurationTimer are overlapping with symbols of different type or with UL transmissions.

The RACH procedure has changed for SBFD capable UEs regarding the selection of RACH opportunities, fall-back between ROs of different RO-type and calculation of the PRACH transmission power when there is a fall-back to a different RO type.

The requirements for 4-step RA type in clause 6.2.2.2 are updated to take into account RACH procedure for UEs that support RACH in SBFD and UL symbols.

RAN4 to agree that SBFD SCell shall be activated on both DL and UL subbands reusing PUCCH SCell activation procedure.

RAN4 to assume UE always supports simultaneous DL reception from 2 DL subbands before RAN1 makes any further conclusion.

When measurement resources for L1-SRS-RSRP (Method#2) and L1-CLI-RSSI (Method#1) are partly or fully overlapping in time, the UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI and the following rules will apply when determining which one of the measurement resource is to be measured if this will be agreed in RAN1:

1)	Aperiodic reporting shall be prioritized over periodic reporting

2)	Aperiodic resources shall be prioritized over periodic and semi-persistent resources

3)	Semi-persistent resources shall be prioritized over periodic resource

4)	SRS-RSRP resources shall be prioritized over CLI-RSSI resources when both have the same resource type

- References

- R4-2512133, WF on RRM requirements for NR_duplex_evo, RAN4#116 Huawei