3GPP TSG-RAN WG4 Meeting #116bis	R4-2513719

Prague, Czech Republic, 13 October 2025 – 17 October 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Performance requirements for SBFD operation

Agenda item:	6.13.3.2

Document for:	Discussion

- 1	Introduction

In the past RAN4 meetings, RAN4 has discussed the impact of the introduction of SBFD symbols on the CLI measurements and RRM requirements in TS 38.133. In RAN4#114bis meeting, RAN4 agreed on the TS 38.133 clauses that would be modified to support SBFD symbols (R4-2504904). A summary of the affected clauses is given below:

Impacted Core requirements and clauses

9.x L1 CLI measurement (new clause)

8.1.3.2 Minimum requirement

8.5.3.2 Minimum requirement

8.5.6.2 Minimum requirement

9.5.4.2 CSI-RS based L1-RSRP Reporting

9.8.4.1 L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

9.8.4.2 L1-SINR reporting with SSB based CMR and dedicated IMR configured

9.8.4.3 L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

9.10.1 Introduction

9.1.2 Measurement gap

8.6 Active BWP switch delay

Others, if identified

In this contribution, we discuss the impact of introducing the SBFD feature on the RRM test cases.

Test cases

- 2.1 Test case coverage

SBFD has been specified on top of existing TDD operation. Therefore, it should be clear that the SBFD aware UE is also tested in TDD mode, considering the existing TDD test case list. The new test cases defined for SBFD aware UEs should be additional to any TDD test cases that are applicable to Rel-19 UEs. In case the SBFD tests are defined in the same clauses as the existing TDD test cases, as a new configuration, RAN4 should make it clear that a TDD and a SBFD configuration should be tested.

TDD test cases are also applicable to SBFD aware UEs. The new test cases introduced for SBFD aware UEs are additional to the test cases defined for TDD UEs.

- 2.2 Test case configuration

In Rel-16, the CLI test cases were defined for standalone (SA) and dual connectivity (DC). In an earlier RAN2 meeting, RAN2 sent an LS to RAN1[3] asking whether SBFD and DC can be configured simultaneously. RAN1 replied that SBFD and NR-DC can be configured simultaneously from RAN1 perspective, although it is up to RAN2 to take a decision whether to support it or not (considering also the potential specification impact to RAN4).

It is under discussion in RAN2 whether SBFD can be configured with DC.

RAN4 to define the test case list for SBFD in SA operation and wait for decision in RAN4 on the support of SBFD with DC.

Introduce the SBFD test cases in clauses A6 (SA in FR1) and A7 (FR2) in TS 38.133.

Additionally, there is need to provide a new TDD configuration that supports SBFD symbols. For simplicity, to ensure that the SSBs are overlapping with SBFD symbols, we propose a TDD configuration that does not contain any DL symbols to be used in the SBFD test cases: XXXXU, for example.

Introduce a new TDD UL/DL configuration for RRM tests in FR1 and FR2, for a 30 kHz SCS and 120 kHz respectively: DDDSU.  For the SBFD time domain configuration, all the Downlink and flexible symbols in the TDD configuration are assigned as SBFD symbols, so that the resulting SBFD configuration is XXXXU.

Another important aspect is the test case configuration, in terms of the BW of the channel and of each subband. We note that the feasibility study of SBFD done in Rel-18 has considered a channel BW of 100 MHz, with a DUD configuration, in which the BW of the DL subband – UL subband and DL subband were 40 MHz, 20 MHz and 40 MHz respectively.

The SBFD test case configuration considers at least a FR1 TDD carrier with 100 MHz channel bandwidth, for a 30 kHz SCS, in DUD configuration, in which 40 MHz – 20 MHz – 40 MHz are assigned to DL, UL and DL subbands respectively, following the configuration used for the feasibility study in Rel-18 (TS 38.858). In FR2, the configuration is: 100 MHz channel bandwidth, 120 kHz SCS and a 40 MHz – 20 MHz – 40 MHz DUD.

Due to the large bandwidth size required to accommodate the different SBFD subbands, a new configuration of SSB for the test cases will need to be provided. Furthermore, according to the RAN1 working assumptions, the SSBs are always confined in the DL subband of a SBFD symbol. Currently, there is no such restriction in the existing SSB configurations. Considering these two aspects, we propose the following configuration to be used in the SBFD test cases, in which the values marked in red were adapted from the existing SSB Pattern 2 in FR1 (A.3.10.1.2 from TS 38.133).

Introduce a new SSB configuration for FR1 SBFD test cases, as follows:

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSB within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: In SBFD symbols, PRBs containing SSB shall be configured in frequency locations completely contained in a DL subband within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. NOTE 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

For FR2, our view is that SSB.1 FR2 configuration can be reused for SBFD symbols, as long as note 1 is modified to ensure that the SSB is placed within a DL subband.

Modify the note for SSB.1 FR2 configuration, to restrict the placement of the SSB in FR2 to RBs confined within the DL subband in SBFD symbols, as follows: “NOTE 1:	In DL symbols, PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. In SBFD symbols, PRBs containing SSB shall be configured in frequency locations completely contained in a DL subband within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13].”

Table A.3.10.2.1-1: SSB.1 FR2: SSB Pattern 1 for SSB SCS = 120 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 120 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSBs Note 2 | 4-7 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |  |
| PRB numbers containing SSBs within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |  |
| NOTE 1: In DL symbols, PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. In SBFD symbols, PRBs containing SSB shall be configured in frequency locations completely contained in a DL subband within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

- 2.3 Test case list

Since new RRM core requirements have been introduced for the support of the SBFD feature, RAN4 should define new test cases, at least to cover the requirements in clauses agreed to be modified in the last RAN4 meeting. In some cases, even though the requirement is the same as legacy NR, RAN4 should define test cases for operation in SBFD symbol to ensure that the requirements are met. This is the case for SSB L1 and L3 measurements.

In the table below, we discuss the clauses with affected Core requirements, as agreed in RAN4 #114bis [1]. In the bottom of the table, RACH test cases are also proposed, to cover the procedure defined by RAN2.

RAN4 to consider the test case list provided in table 1:

Table 1 - List of test cases

| Core requirements clauses | New test case clauses |  | Comments |
| --- | --- | --- | --- |
| 8.1.3.2 | Minimum requirement | A.6.5.1.X (in-sync) FR1 A.6.5.1.Y (out-of-sync) FR1 A.7.5.1.X (in-sync) FR2 A.7.5.1.Y (out-of-sync) FR2 | CSI-RS based RLM A new test case to test the requirements in SBFD symbols should be defined. In this test case, the UE should be tested on the out-of-sync and in-sync evaluation periods when SBFD symbols (or DL symbols) are used for RLM. |
| 8.5.3.2 | Minimum requirement | A.6.5.5.X (FR1) A.7.5.5.X (FR2) | CSI-RS based BFD A new test case to test the requirements in SBFD symbols should be defined. In this test case, the UE should be tested on the evaluation periods when SBFD symbols (or DL symbols) are used for BFD. |
| 8.5.6.2 | Minimum requirement | A.6.5.5.X (same as above) A.7.5.5.X (same as above) | CSI-RS based CBD A new test case to test the requirements in SBFD symbols should be defined. |
| 8.6 | Active BWP switch delay |  | - |
| 9.1.2 | Measurement gap |  | - |
| 9.2 | Intra-frequency measurements | A.6.6.1.X Intra-frequency measurements (FR1) A.7.6.1.X Intra-frequency measurements (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing intra-frequency measurements within the legacy requirements. In this test case, it would be tested whether the UE can send measurement reports after triggered by a mobility event, when the SSB is transmitted in SBFD symbols. |
| 9.3 | Inter-frequency measurements | A.6.6.2.X Intra-frequency measurements (FR1) A.7.6.2.X Intra-frequency measurements (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing inter-frequency measurements within the legacy requirements. In this test case, it would be tested whether the UE can send measurement reports after triggered by a mobility event, when the SSB is transmitted in SBFD symbols. |
| 9.5.4.1 | SSB based L1-RSRP measurement | A.6.6.4.X1 (FR1) A.7.6.3.X1 (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing SSB based L1 measurements within the legacy requirements. |
| 9.5.4.2 | CSI-RS based L1-RSRP Reporting | A.6.6.4.X2 (FR1) A.7.6.3.X2 (FR2) | Test cases are needed to test L1-RSRP measurement in DU and DUD configurations. |
| 9.8.4.2 | SSB based L1-SINR | A.6.6.8.X1 (FR1) A.7.6.6.X1 (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing SSB based L1 measurements within the legacy requirements. |
| 9.8.4.1 | L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured | A.6.6.8.X2 (FR1) A.7.6.6.X2 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.8.4.2 | L1-SINR reporting with SSB based CMR and dedicated IMR configured | A.6.6.8.X3 (FR1) A.7.6.6.X3 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.8.4.3 | L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured | A.6.6.8.X4 (FR1) A.7.6.6.X4 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.10.1 | Introduction | A.6.6.10.X (intra-frequency measurements) A.6.6.11.X (inter-frequency measurements) (FR1) A.7.6.7.X (intra-frequency measurements) A.7.6.8.X (inter-frequency measurements) (FR2) | CSI-RS based L3 measurements Test cases are needed to test L3 measurements in DU configurations, and for DUD |
| Clauses affected – pending RAN4 agreement |  |  |  |
| 6.2.2 | Random Access | A.4.3.2.2.X (FR1) A.7.3.2.2.X (FR2) | Test cases to test the RO type selection and fall back defined by RAN2. What requirements should be tested in this test case: the UE correct behavior when configured with ROs in SBFD symbols. UE behavior when the UE falls-back to a different type of RO when the maximum number of preamble transmissions for a given RO-type is configured. |

Performance requirements

Requirements for CSI-RS based L1 measurements

Regarding the CSI-RS based L1 measurements, the following agreements were reached in RAN4#116:

### Sub-topic 2-1: RRM impacts of SBFD operation #### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements (Monday online session) - For UE supporting simultaneous DL reception in 2 DL subbands, - For measurement period: - For UE indicating CSI processing time scaled by 2, - Add a lower bound of 8ms on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2. - For UE not indicating CSI processing time scaled by 2, - Lower bound is not needed and existing measurement period applies - Further discuss the measurement accuracy requirements in Perf part of the WI - For UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB for Case 3 apply. - Option 2: accuracy requirements based on 24 RB for Case 3 apply. - For UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB for Case 3 apply. - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

There are 2 open issues to be discussed on this topic. First, the handling of UEs not supporting simultaneous CSI-RS reception across two DL subbands. It is relevant for this discussion to highlight the latest RAN1 agreements about the SBFD UE capabilities. During RAN1#122, it was agreed that SBFD UEs support PDSCH and CSI-RS reception across two DL subbands as basic feature (FG 60-1):

Agreement: - FG 60-1 to include support of PDSCH/CSI-RS reception in two DL subbands

Consequently, the latest UE capabilities were updated as follows:

Table 2 - Excerpt of the 60-1 Basic feature for SBFD-aware UE

| 60. NR_duplex_evo | 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one/two DL subband in SBFD symbols … FFS: other component (which may be separate FG) |  | Component 7 applies only to UEs supporting FG 23-1-1h Note: Component 9 is only applicable for SRS resource sets with usage codebook, nonCodebook and beamManagement Component 10 candidate values: {2, 4} Component 11 candidate values: {“with factor-of-two relaxation”, “without factor-of-two relaxation”} FFS: details of counting |
| --- | --- | --- | --- | --- | --- |

RAN1 agreed to support PDSCH and CSI-RS reception in two DL subbands as basic SBFD UE feature.

Given that, according to RAN1, all SBFD UEs are capable of measuring CSI-RS in two DL subbands, RAN4 should no longer consider the case in which a UE is only capable of measuring one DL subband in a DUD SBFD configuration.

Following RAN1 agreements, RAN4 assumes that SBFD UEs can simultaneously measure CSI-RS in two DL subbands in a DUD SBFD configuration. Other UE operations, i.e., UEs only able to measure in a single DL subband in a DUD configuration, should not be considered.

The second open issue is about measurement accuracy requirements. In the past RAN4 meetings, we agreed to define relaxed accuracy requirements based on 24 PRBs for Case 3. In our understanding, this accuracy relaxation was introduced because some UE vendors were not able to simultaneously measure the CSI-RS resource across 2 DL subbands. However, this case is no longer valid as RAN1 recently agreed that basic SBFD UE feature should support simultaneous reception across two DL subbands. Therefore, we suggest revisiting the previous agreement and assume that, for UE supporting simultaneous DL reception in 2 DL subbands, the existing accuracy requirements based on 48 RB for Case 3 apply.

In RAN4, the relaxed accuracy requirements based on 24 PRBs were introduced because some UE vendors were not able to simultaneously measure the CSI-RS resource across 2 DL subbands. This reasoning is not valid based on latest RAN1 agreements on basic SBFD UE operation.

Apply the existing CSI-RS L1 accuracy requirements based on 48 PRBs for Case 3.

Moreover, we do not see the need to differentiate the accuracy requirements depending on whether the UE scales or not the CSI processing timeline. Note that in both cases, the UE shall measure the CSI-RS resources across the two DL subbands. Therefore, we propose to use the existing 48 PRBs accuracy requirements for both scenarios.

Apply the existing CSI-RS L1 accuracy requirements based on 48 PRBs for Case 3 regardless of whether the UE scales the CSI processing timeline or not.

Conclusion

The following Observations and Proposals were made:

Proposal 1: TDD test cases are also applicable to SBFD aware UEs. The new test cases introduced for SBFD aware UEs are additional to the test cases defined for TDD UEs.

Observation 1: It is under discussion in RAN2 whether SBFD can be configured with DC.

Proposal 2: RAN4 to define the test case list for SBFD in SA operation and wait for decision in RAN4 on the support of SBFD with DC.

Proposal 3: Introduce the SBFD test cases in clauses A6 (SA in FR1) and A7 (FR2) in TS 38.133.

Proposal 4: Introduce a new TDD UL/DL configuration for RRM tests in FR1 and FR2, for a 30 kHz SCS and 120 kHz respectively: DDDSU.  For the SBFD time domain configuration, all the Downlink and flexible symbols in the TDD configuration are assigned as SBFD symbols, so that the resulting SBFD configuration is XXXXU.

Proposal 5: The SBFD test case configuration considers at least a FR1 TDD carrier with 100 MHz channel bandwidth, for a 30 kHz SCS, in DUD configuration, in which 40 MHz – 20 MHz – 40 MHz are assigned to DL, UL and DL subbands respectively, following the configuration used for the feasibility study in Rel-18 (TS 38.858). In FR2, the configuration is: 100 MHz channel bandwidth, 120 kHz SCS and a 40 MHz – 20 MHz – 40 MHz DUD.

Proposal 6: Introduce a new SSB configuration for FR1 SBFD test cases, as follows:

Proposal 7: Modify the note for SSB.1 FR2 configuration, to restrict the placement of the SSB in FR2 to RBs confined within the DL subband in SBFD symbols, as follows: “NOTE 1:	In DL symbols, PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. In SBFD symbols, PRBs containing SSB shall be configured in frequency locations completely contained in a DL subband within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13].”

Proposal 8: RAN4 to consider the test case list provided in table 1:

Table 1 - List of test cases

| Core requirements clauses | New test case clauses |  | Comments |
| --- | --- | --- | --- |
| 8.1.3.2 | Minimum requirement | A.6.5.1.X (in-sync) FR1 A.6.5.1.Y (out-of-sync) FR1 A.7.5.1.X (in-sync) FR2 A.7.5.1.Y (out-of-sync) FR2 | CSI-RS based RLM A new test case to test the requirements in SBFD symbols should be defined. In this test case, the UE should be tested on the out-of-sync and in-sync evaluation periods when SBFD symbols (or DL symbols) are used for RLM. |
| 8.5.3.2 | Minimum requirement | A.6.5.5.X (FR1) A.7.5.5.X (FR2) | CSI-RS based BFD A new test case to test the requirements in SBFD symbols should be defined. In this test case, the UE should be tested on the evaluation periods when SBFD symbols (or DL symbols) are used for BFD. |
| 8.5.6.2 | Minimum requirement | A.6.5.5.X (same as above) A.7.5.5.X (same as above) | CSI-RS based CBD A new test case to test the requirements in SBFD symbols should be defined. |
| 8.6 | Active BWP switch delay |  | - |
| 9.1.2 | Measurement gap |  | - |
| 9.2 | Intra-frequency measurements | A.6.6.1.X Intra-frequency measurements (FR1) A.7.6.1.X Intra-frequency measurements (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing intra-frequency measurements within the legacy requirements. In this test case, it would be tested whether the UE can send measurement reports after triggered by a mobility event, when the SSB is transmitted in SBFD symbols. |
| 9.3 | Inter-frequency measurements | A.6.6.2.X Intra-frequency measurements (FR1) A.7.6.2.X Intra-frequency measurements (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing inter-frequency measurements within the legacy requirements. In this test case, it would be tested whether the UE can send measurement reports after triggered by a mobility event, when the SSB is transmitted in SBFD symbols. |
| 9.5.4.1 | SSB based L1-RSRP measurement | A.6.6.4.X1 (FR1) A.7.6.3.X1 (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing SSB based L1 measurements within the legacy requirements. |
| 9.5.4.2 | CSI-RS based L1-RSRP Reporting | A.6.6.4.X2 (FR1) A.7.6.3.X2 (FR2) | Test cases are needed to test L1-RSRP measurement in DU and DUD configurations. |
| 9.8.4.2 | SSB based L1-SINR | A.6.6.8.X1 (FR1) A.7.6.6.X1 (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing SSB based L1 measurements within the legacy requirements. |
| 9.8.4.1 | L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured | A.6.6.8.X2 (FR1) A.7.6.6.X2 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.8.4.2 | L1-SINR reporting with SSB based CMR and dedicated IMR configured | A.6.6.8.X3 (FR1) A.7.6.6.X3 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.8.4.3 | L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured | A.6.6.8.X4 (FR1) A.7.6.6.X4 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.10.1 | Introduction | A.6.6.10.X (intra-frequency measurements) A.6.6.11.X (inter-frequency measurements) (FR1) A.7.6.7.X (intra-frequency measurements) A.7.6.8.X (inter-frequency measurements) (FR2) | CSI-RS based L3 measurements Test cases are needed to test L3 measurements in DU configurations, and for DUD |
| Clauses affected – pending RAN4 agreement |  |  |  |
| 6.2.2 | Random Access | A.4.3.2.2.X (FR1) A.7.3.2.2.X (FR2) | Test cases to test the RO type selection and fall back defined by RAN2. What requirements should be tested in this test case: the UE correct behavior when configured with ROs in SBFD symbols. UE behavior when the UE falls-back to a different type of RO when the maximum number of preamble transmissions for a given RO-type is configured. |

Observation 2: RAN1 agreed to support PDSCH and CSI-RS reception in two DL subbands as basic SBFD UE feature.

Proposal 9: Following RAN1 agreements, RAN4 assumes that SBFD UEs can simultaneously measure CSI-RS in two DL subbands in a DUD SBFD configuration. Other UE operations, i.e., UEs only able to measure in a single DL subband in a DUD configuration, should not be considered.

Observation 3: In RAN4, the relaxed accuracy requirements based on 24 PRBs were introduced because some UE vendors were not able to simultaneously measure the CSI-RS resource across 2 DL subbands. This reasoning is not valid based on latest RAN1 agreements on basic SBFD UE operation.

Proposal 10: Apply the existing CSI-RS L1 accuracy requirements based on 48 PRBs for Case 3.

Proposal 11: Apply the existing CSI-RS L1 accuracy requirements based on 48 PRBs for Case 3 regardless of whether the UE scales the CSI processing timeline or not.

- References

- R4-2504904, WF on SBFD RRM, Huawei

- R4-2503623, summary [114][222] NR_duplex_evo, Huawei

- R2-2503191, LS on simultaneous configuration of SBFD and DC, RAN2