3GPP TSG-RAN WG4 Meeting #116bis			                                      R4-2513747

Prague, CZ, October 13rd – 17th, 2025

Agenda item:	7.7.4

Source: 	CMCC

Title: 	Discussion on UE RF core requirements for UAV

Document for:	Approval

# 1. Introduction

RAN#109 meeting approved RP-252954 [1] New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) in Rel-20. RF core requirements for HPUE UAV shall be specified in this WID. This paper will focus on the UE RF requirements which are different from UAV.

# 2. Discussion

## 2.1. General UAV RF requirements

The description for UAV RF in the WID is shown as below:

- Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

Based on the requirements of the operators, five example bands have been clearly defined in the WID, including four TDD bands and one FDD band. It is worth noting that all the UAV example bands are the operating bands of ATG as defined in TS38.101-1.

Observation 1: All the UAV example bands are the operating bands of ATG as defined in TS38.101-1

Currently, two UAV antenna architectures have been proposed in the WID, omni-direction antenna and antenna array. Before a clear conclusion on coexistence is reached, both types of antennas should be supported by UAVs. However, due to the limitations of the transportation capacity and architecture of the UAV UE, a single UAV UE can only choose to support one of the antenna types. In TR38.876, the ATG UE also explicitly states that it can support both antenna architectures.

Proposal 1: The UAV UE can support both antenna architectures, but the same UE can only choose to support one of them for Tx.

## 2.2. UAV UE Tx requirements

Proposal 2: Consideration in Table 1 could be set as the starting point of UAV UE Tx requirements.

Table 1.  UAV UE Tx requirements

| Requirement | Applicability |
| --- | --- |
| Transmitter power for Aerial UE | In the current spec, the requirements for PC3 UAV had been defined, and this WID will focus on HPUE UAV. So transmitter power for Aerial UE should be specified. |
| Maximum input level for Aerial UE | Maximum input level for Aerial UE should be specified, especially minimum output power for UAV with different antenna types. |
| Transmit signal quality for Aerial UE | Transmit signal quality for Aerial UE should be specified, especially frequency error and transmit modulation quality. |
| Output RF spectrum emissions for Aerial UE | Output RF spectrum emissions for Aerial UE should be specified, especially out of band emission. |

## 2.3. UAV UE Rx requirements

Unlike the UAV Tx requirements, in this WI, the main objective is the impact of increasing Tx power on the UAV. Therefore, the current legacy Rx requirement for handheld UE could be used as the starting point. It is worth noting that the selection of the DL antenna type may have an effect on the UAV Rx requirement, and further discussion on this topic is needed.

Proposal 3: Legacy Rx requirement for handheld UE could be used as the starting point.

# 3. Conclusions

In this contribution, UAV UE RF requirements are discussed with following proposals:

Observation 1: All the UAV example bands are the operating bands of ATG as defined in TS38.101-1.

Proposal 1: The UAV UE can support both antenna architectures, but the same UE can only choose to support one of them for Tx.

Proposal 2: Consideration in Table 1 could be set as the starting point of UAV UE Tx requirements.

Proposal 3: Legacy Rx requirement for handheld UE could be used as the starting point.

# 4. Reference

[1] RP-252954, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), HUAWEI