3GPP TSG-RAN WG4 Meeting #116bis                                                    	R4-2513752

Prague, Czech Republic, October 13 – 17, 2025

Source:	ZTE Corporation, Sanechips

Title:	TP for PRD 01 to correct guidelines for delta TIB and RIB for more than two bands combinations

Agenda item:			5.2.6

Document for:	Approval

# 1. Introduction

Some guidelines on how to introduce the ΔTIB for UE transmit power and ΔRIB for reference sensitivity to relax a given band combination have been specified in PRD 01. The guidelines in sub-clause 5.4.3 for the values of ΔTIB and ΔRIB with more than 2-band CA/DC should be derived from the next lower order CA/DC instead of 2-band CA/DC. Furthermore, the sub-clause number referenced in sub-clause 5.4.4 should be corrected.

Proposal 1.  It is suggested to capture the following TP for corrections on delta TIB and RIB for more than 2-band CA/DC in PRD 01.

2. Reference

[1] R4-2511177, RAN4 PRD 01 v0.4.0, Rules, guidelines and ways of working for introduction of band combinations in NR and LTE, Ericsson.

# 3. Text Proposal

<Start of Text Proposal>

5.4	Guidelines on ΔTIB and ΔRIB

5.4.1		General

The ΔTIB and ΔRIB values are introduced to allow relaxation to the UEs transmit power (ΔTIB) and reference sensitivity (ΔRIB) to compensate for hardware complexity of supporting a given band combination.

5.4.2		2-band CA/DC ΔTIB,c and ΔRIB,c

To define ΔTIB and ΔRIB when introducing new 2-band CA/DC band combinations the following guidelines can be followed.

Grouping of bands:

- L-bands are below 1 GHz.

- H-bands are above 1 GHz and below 3GHz

- VH-bands are above 3 GHz

Delta values apply as follows for 2-band CA/DC:

- VH bands ΔTIB =0.8 and ΔRIB =0.5

- H bands ΔTIB =0.5 and ΔRIB =0

  - Except for n1 and n3 which have ΔTIB =0.3

- H-L: Both gets ΔTIB =0.3 and ΔRIB =0

- L-L,H-H: Both gets ΔTIB =0.5 and ΔRIB =0

  - Except for n1 and n3 which have ΔTIB =0.3

- L-VH: L ΔTIB =0.3 and ΔRIB =0.2

- H-VH: H ΔTIB =0.5 and ΔRIB =0.2

- Carriers with any uplink harmonic falling onto the DL:

  - L-H: L ΔTIB =0.6 dB H ΔTIB =0.3 dB

  - L-VH: L ΔTIB =0.6 dB VH ΔTIB =0.8 dB

  - H-VH: H ΔTIB =0.6 dB VH ΔTIB =0.8 dB

- For SDL combinations SDL band gets “N/A” and  L,H ΔTIB =0.3 and VH ΔTIB =0.8

5.4.3			More than 2-band CA/DC ΔTIB,c and ΔRIB,c

Delta values for more than 2-band CA/DC are found as the MAX of the included delta values defined for the next lower order CA/DC combinations. For example, the delta values for n-band CA/DC (n>2) are derived from the max delta values for (n-1)-band CA/DC.

5.4.4			Exceptions of ΔTIB and ΔRIB for specific band combinations

Due to multiple reasons the delta values captured together with the band combinations in the specifications may differ from the presented guidelines in Clause 5.4.2 and Clause 5.4.3. These values are found on a case-by-case basis when they are introduced.

<End of Text Proposal>