3GPP TSG-RAN WG4 # 116-bis	R4-2513765

Prague, Czech Republic, 13th October 2025 – 17th October 2025

Agenda item:	6.11.3

Source:	MediaTek Inc.

Title:	Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

# Introduction

In the previous RAN4#116 meeting CSI reporting requirement and testing framework for CSI prediction was discussed and WF with several open issues was introduced for further work [1]. In this document we discuss remaining open issues with several observations and proposals.

# Discussion

In the latest WF document, several open questions remain regarding performance monitoring.

Issue 1-1: Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FSS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

In the PMI requirements study, we observed a Doppler sweet spot that enables the largest performance gains from precoding prediction. When Doppler is too low, there is little opportunity for gain; when Doppler is too high, the channel is under sampled and accurate prediction cannot be achieved.

Observation #1: The performance gains of precoding prediction depend strongly on Doppler.

Because measuring the absolute accuracy of performance monitoring is challenging, we propose evaluating the monitoring under conditions where the highest gains are expected and under conditions where no gain (or even a loss) is expected and verifying that the performance monitoring metrics reflect these outcomes.

Proposal #1: Evaluate the performance‑monitoring metric under test conditions corresponding to the expected Doppler sweet spot (highest achievable gains).

Proposal #2: Evaluate the performance monitoring metric under test conditions with very low or very high Doppler, where no gains - or even a loss - are expected.

# Conclusion

In this contribution we provide our views on the CSI reporting requirement and testing framework for CSI prediction issues. The following observations and proposals are made:

Observation #1: The performance gains of precoding prediction depend strongly on Doppler.

Proposal #1: Evaluate the performance-monitoring metric under test conditions corresponding to the expected Doppler sweet spot (highest achievable gains).

Proposal #2: Evaluate the performance monitoring metric under test conditions with very low or very high Doppler, where no gains - or even a loss - are expected.

# Reference

- R4-2511890, “WF on AI/ML for air interface for [116][131]”, Qualcomm inc.