# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513772

# Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	General aspects on BS RF requirement evolution

Agenda Item:	7.2.2

Document for:	Approval

1 Introduction

During RAN#109, RAN4 Rel-20 5G-Adavanced package has been approved. New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2 (BS RF requirement evolution) was agreed [1]. The work item concentrates on 2 work areas for BS RF requirement evolution:

- Co-location requirements enhancement:

- SBFD BS to SBFD BS adjacent channel coexistence

In this contribution we provide some consideration on the general aspects.

2 Discussion

2.1 General aspects

One work area is co-location requirements enhancement, which was extensively discussed in Rel-19.

- Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz

  - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements.

    - Consider impact on the following OTA co-location requirements:

      - Transmitter OFF power requirement

      - Transmitter co-location spurious emission requirement

      - Transmitter intermodulation requirement

      - Co-location OOBB requirement

  - Enhance OTA co-location requirements

    - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered

      - Option 1: Investigate and improve CLRA concept

      - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS), if any

As mentioned in Rel-15 TR 37.843, these issues still exist when considering the enhancement of OTA co-location requirements.

There are a number of issues which make translating the existing conducted requirement into an OTA requirement difficult.

-	The required level of RX band spurious emissions is too low to measure in the far field.

-	The out-of-band antenna gain of the AAS BS is not known.

-	The interferer power level for the IMD requirement in the far field is very high.

In our contribution in agenda 7.2.3 [x], we provide the general consideration on co-location requirements.

- SBFD BS to SBFD BS adjacent channel coexistence

  - Focus on FR1 WA.

  - Consider both scenarios including SBFD capable BS being co-located and non-collocated.

  - Focus on bands: n104, n79, n41.

  - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include

    - TX unwanted emission

    - RX in-band blocking

  - Note 1: whether to comply with requirements is declared by BS.

  - Note 2: The existing TDD requirements apply for non-SBFD BSs

The other work area is SBFD BS to SBFD BS adjacent channel co-existence.

We provide an overview on this work area in agenda 7.2.4 [R4-2513773].

2.2 Work plan

The proposed work plan for SBFD-SBFD co-existence can be found below.

Proposal 1: it is proposed to take the above work plan into account for SBFD-SBFD co-existence.

2.3 TR skeleton

For SBFD to SBFD BS adjacent channel co-existence TR, the following clauses are proposed for

- 1 Scope

- 2	References

- 3	Definitions, symbols and abbreviations

- 4	Objectives of study

- 5	Deployment scenarios

- 6	Co-existence study

- 7	RF requirements

Proposal 2: For SBFD to SBFD BS adjacent channel co-existence TR, the above clauses are proposed.

Conclusion

In this contribution we provide consideration on Rel-20 NR BS RF requirement evolution.

Proposal 1: it is proposed to take the work plan in clause 2.2 into account for SBFD-SBFD co-existence.

Proposal 2: For SBFD to SBFD BS adjacent channel co-existence TR, the following clauses are proposed:

- 1 Scope

- 2	References

- 3	Definitions, symbols and abbreviations

- 4	Objectives of study

- 5	Deployment scenarios

- 6	Co-existence study

- 7	RF requirements

References

- RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.

- ECC Report 281, Analysis of the suitability of the regulatory technical conditions for 5G MFCN operation in the 3400-3800 MHz band