3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2513787

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	On general aspects for SBFD operation

Agenda item:			4.19.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In RAN4#116, CR was agreed to TS38.104 to introduce BS RF requirement for SBFD-capable BS, which has been approved in the CR package [RP‑252418] in RAN#109. In this contribution, we would like to provide viewpoints to solve the remaining issues for maintenance purpose.

# 2. Discussion

## 2.1 Simultaneous configuration of SBFD and DC

In RAN2#131, RAN2 has analyzed the problem of simultaneous configuration of SBFD and dual connectivity (DC) and replied RAN1 and RAN4 with the following agreement captured in the reply LS [5].

RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion.

Based on RAN4 BS RF discussion for SBFD-capable BS requirement, in both Rel-18 study and Rel-19 work items, RAN4 mainly focused on the multi-carrier configuration for SBFD-capable BS, i.e., SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier, as below captured in the technical report [2]. For simultaneous configuration of SBFD and DC, RAN4 has not yet discussed such use case.

During Rel-18 SI, for SBFD-capable BS, RF requirement was only studied based on the semi-static configuration of subband time and frequency location, which is supported by SBFD-capable BS. RAN4 mainly focused on the following multi-carrier configuration for SBFD-capable BS: - SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier. During Rel-18 SI, RAN4 didn’t study the following multi-carrier configuration for SBFD-capable BS: - SBFD operates in more than one BS carriers, and legacy TDD operates in the other intra-band BS carrier(s) (if any), which is contiguous or non-contiguous to the SBFD carriers.

From BS RF perspective, our understanding is all the specified BS RF requirement shall be applicable to SBFD carrier, no matter DC is configured or not. From RRM perspective, there could be minor changes expected for simultaneous configuration of SBFD and DC.

Proposal 1: RAN4 reach the following agreement, based on which the reply LS to RAN1/2 is drafted:

- RAN4 valuated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following:

- SBFD-capable BS RF requirement: The requirement is specified in the way that it shall be applicable to the carrier with SBFD operation, no matter dual connectivity is configured or not.

- UE RRM requirement to support SBFD operations: There may be limited specification impacts, which can be provided in the WI maintenance if RAN1 has confirmed the necessity of simultaneous configuration of SBFD and DC.

## 2.2 Specification update for HPUE duty cycle evaluation

RAN4 has discussed the necessity of the specification update for HPUE duty cycle evaluation, and the following WF is provided in RAN4#114bis:

Way forward: - RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

While no further agreement achieved in RAN4#115, the following agreement is provided in WF in RAN4#116 and there is still FFS how to introduce in RAN4 spec if the update is needed:

##### Issue 2-1-1: The necessity to update for HPUE duty cycle evaluation Agreement: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. ##### Issue 2-1-2: If the update is needed, how to introduce in RAN4 spec - No further agreement, and the WF from RAN4#114bis can be followed in the maintenance of Rel-19 SBFD WI.

Based on RAN4 agreement, for SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission.

For existing UE RF requirement, RMC used for TDD in TS 38.101-1 are defined with the active UL slots specified in Table A.2.1-1 and TDD slot patterns as defined for reference sensitivity tests. In other words, from UE RF requirement perspective, only active uplink slots are considered for UE TX requirement.

Table A.2.1-1: TDD active uplink slots

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 4, 9 |
| 30 kHz | 8, 9, 18, 19 |
| 60 kHz | 16, 17, 18, 19, 36, 37, 38, 39 |

Observation 1: The existing TDD UE TX RF requirement is specified only on active UL slots.

Because we expect the RMC will be not be changed for SBFD-aware UE, to further clarify “the symbols used for uplink duty cycle evaluation” will not change anything for UE conformance requirement.

Observation 2: Further clarification on “the symbols used for uplink duty cycle evaluation” will not change anything for UE conformance requirement.

Accordingly, we suggest the following proposal:

Proposal 2: For SBFD-aware UE, no RAN4 requirement impact is expected for UE specification series.

# 3. Conclusion

In this contribution, we have provided we would like to provide viewpoints to solve the remaining issues for maintenance purpose:

Simultaneous configuration of SBFD and DC

Proposal 1: RAN4 reach the following agreement, based on which the reply LS to RAN1/2 is drafted:

- RAN4 valuated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following:

- SBFD-capable BS RF requirement: The requirement is specified in the way that it shall be applicable to the carrier with SBFD operation, no matter dual connectivity is configured or not.

- UE RRM requirement to support SBFD operations: There may be limited specification impacts, which can be provided in the WI maintenance if RAN1 has confirmed the necessity of simultaneous configuration of SBFD and DC.

Specification update for HPUE duty cycle evaluation

Observation 1: The existing TDD UE TX RF requirement is specified only on active UL slots.

Observation 2: Further clarification on “the symbols used for uplink duty cycle evaluation” will not change anything for UE conformance requirement.

Proposal 2: For SBFD-aware UE, no RAN4 requirement impact is expected for UE specification series.

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2512665, “Way Forward for [116][306] NR_duplex_evo_General”, Samsung, CATT, Charter, CMCC, Ericsson, Huawei, HiSilicon, Nokia, Qualcomm, Tejas Network, ZTE, RAN#116.

[4] R4-2511255, “CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS”, Samsung, RAN#116.

[5] R2-2506492 (R4-2513014), “Reply LS on simultaneous configuration of SBFD and DC”, ZTE, RAN2#131.