3GPP TSG-RAN WG4 Meeting #116bis		     R4-2513807

Prague, Czech Republic, 13 – 17 Oct 2025

Agenda Item:	6.11.7

Source: 	OPPO

Title: 	Demodulation and CSI reporting requirements for CSI prediction

Document for:	Approval

Discussion

Regarding the reference model for CSI prediction, the following agreements were reached during the RAN4#114 meeting:

Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

Regarding the simulation assumptions for AIML-based CSI prediction, following aspects are updated in RAN4#116 meeting[1],

- Introduce SGCS-1 and SGCS-3 as KPIs for Step-3 (under practical channel estimation, corresponding to 90% of maximum TP), to better align and understand companies’ simulation results.

- In addition to γ1_AI/ML(already agreed), also provide γ2_AI/ML:

γ1_AI/ML：AI/ML based CSI prediction is compared with Rel-15 Type I random PMI

γ2_AI/ML：AI/ML based CSI prediction is compared with Rel-16 eType II CB(sample and hold)

- Include MCS 17 and MCS 19 for Step-3

- Keep both 20Hz and 50Hz Doppler

- Keep 90% of maximum TP

- It is up to UE implementation whether to use one model for an SNR range or a separate model per SNR during simulations.

- Add beam steer approach for step-3. Randomization of the dual-cluster beam directions will be used as specified in AnnexB.2.3.2.3A in 38.101-4. The value of relative power ratio (p) shall be fixed as 1 during the test

- Note: companies are encouraged to clarify whether the SNR dataset for model training is per SNR or with an SNR range when providing the results of step-3 simulation

Accordingly, we conducted and updated Step-3 evaluations for the CSI prediction using the simulation assumptions(highlighted in yellow in Table 1). The simulation results are shown in Table 2 to Table 6.

Table 1: Updated Simulation assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Subcarrier spacing | 15kHz for FDD, 30kHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol for step-1 and step-2 simulation |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| gNB TX antennas | Baseline: 16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements Optional: 32(N1,N2)=(4,4) as defined in Rel-15 type1 |
| UE RX antennas | 4 |
| Channel model | TDLA30 with XP medium correlation as baseline |
| Doppler spread | 20Hz, 50Hz |
| Channel estimation for CSI prediction | Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| CSI-RS configuration | For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12). |
| Rank per UE | Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline |
| CSI feedback assumption | Step-1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account |
|  | Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed |
| Test setup(timeline) | Only for step-3 For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration |
| Beam steer approach | For step-3, randomization of the dual-cluster beam directions should be used as specified in AnnexB.2.3.2.3A in 38.101-4. The value of relative power ratio (p) shall be fixed as 1 during the test |
| SNR assumption | Only for step-3 Claimed by companies for step-3 first round simulation (TP as KPI) FFS: other suggestions |
| CSI feedback overhead | PC7 as baseline |
| Baseline | FFS - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-3 or performance requirements) |
| KPI | Step-1: - check SGCS before CSI feedback - SGCS is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-2: - check SGCS2 and SGCS3 - SGCS2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2) Step-3: - check SGCS1 and SGCS3, under practical channel estimation, corresponding to 90% of maximum TP with follow PMI using AI/ML-based CSI prediction - SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2) - check TPs after CSI feedback for both AI based CSI prediction and Random PMI with Rel-15 Type I single panel codebook, as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction γ1_AI/ML: AI/ML based CSI prediction is compared with Rel-15 Type I random PMI - check TPs after CSI feedback for both AI based CSI prediction and Rel-16 eType II codebook(sample and hold), as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction γ2_AI/ML: AI/ML based CSI prediction is compared with Rel-16 eType II CB(sample and hold) - Companies are also encouraged to provide the TP curve with corresponding SNRs. - The test parameters for TPs simulation can be referred as 38.101-4 section of 6.3 - FDD with 16Tx4Rx: Table 6.3.3.1.8-1 - FDD with 32Tx4Rx: Table 6.3.3.1.4-1: - TDD with 16Tx4Rx: Table 6.3.3.2.8-1 - TDD with 32Tx4Rx: Table 6.3.3.2.4-1: |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz,); 20Hz Doppler spreads. 50Hz Doppler spreads. |
| Companies can provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models Companies are encouraged to clarify whether the SNR dataset for model training is per SNR or with an SNR range when providing the results of step-3 simulation |  |
| Clarifications on the SGCS calculation: SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel SGCS2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. |  |

Table 2. Simulation results(step-1) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  | 50Hz |  |  |  |  | 100Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank1) |
| SGCS1 | 16 | Non-mix | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.8149 | 0.9585 | 0.9401 | 0.9585 | 0.9217 | 0.3853 |  |  |  |  |  |
|  |  | Mix | 0.9999 | 0.9999 | 0.9999 | 0.9999 |  | 0.9566 | 0.9374 | 0.9566 | 0.9181 |  |  |  |  |  |  |
| SGCS1 | 32 | Non-mix | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.8318 | 0.9597 | 0.9401 | 0.9596 | 0.9205 | 0.4170 | 0.2824 | 0.2123 | 0.2824 | 0.1422 | 0.2774 |
|  |  | Mix | 0.9997 | 0.9998 | 0.9999 | 0.9997 |  | 0.9591 | 0.9394 | 0.9591 | 0.9196 |  | 0.2871 | 0.2149 | 0.2871 | 0.1426 |  |
| SGCS1 is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3. Simulation results(step-2) for AIML based CSI prediction

|  | TX | Train | 20Hz |  |  |  |  |  | 50Hz |  |  |  |  |  | 100Hz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) | Rank1 | Rank2-ave L1&2 | Rank2-L1 | Rank2-L2 | Non-AI (Rank2-L1) | Non-AI (Rank2-L2) |
| SGCS2 | 16 | Non-mix | - | 0.7605 | 0.812 | 0.709 | 0.6772 | 0.4986 |  | 0.7202 | 0.7814 | 0.659 | 0.3500 | 0.1887 |  |  |  |  |  |  |
|  |  | Mix | - | 0.7603 | 0.8122 | 0.7084 |  |  |  | 0.7194 | 0.7809 | 0.6578 |  |  |  |  |  |  |  |  |
| SGCS3 | 16 | Non-mix |  | 0.9819 | 0.9859 | 0.9779 | 0.6933 | 0.5321 |  | 0.8111 | 0.8547 | 0.7674 | 0.3548 | 0.1961 |  |  |  |  |  |  |
|  |  | Mix |  | 0.9775 | 0.9829 | 0.9721 |  |  |  | 0.8079 | 0.8528 | 0.7629 |  |  |  |  |  |  |  |  |
| SGCS2 | 32 | Non-mix | - | 0.7239 | 0.7884 | 0.6593 | 0.6743 | 0.4752 | - | 0.687 | 0.7613 | 0.6127 | 0.3834 | 0.1872 |  | 0.2165 | 0.2862 | 0.1468 | 0.2811 | 0.1449 |  |
|  |  | Mix | - | 0.7236 | 0.7881 | 0.6591 |  |  | - | 0.6864 | 0.7608 | 0.6119 |  |  |  | 0.2206 | 0.2923 | 0.1489 |  |  |  |
| SGCS3 | 32 | Non-mix |  | 0.9786 | 0.9842 | 0.9729 | 0.7113 | 0.5360 |  | 0.8174 | 0.8631 | 0.7716 | 0.3956 | 0.2076 |  | 0.2343 | 0.3013 | 0.1673 | 0.2962 | 0.1668 |  |
|  |  | Mix |  | 0.9716 | 0.9786 | 0.9645 |  |  |  | 0.8151 | 0.8619 | 0.7683 |  |  |  | 0.2403 | 0.3091 | 0.1714 |  |  |  |
| SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4. Simulation results(step-3) for AIML based CSI prediction, MCS=13

| TX | Train | 20Hz Doppler spread |  |  |  |  | 50Hz Doppler spread |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  |
| 16 | Non-mix | 21665 | 3.7 dB | 2470 | 8.8 |  | 21665 | 4.0 dB | 2790 | 7.8 |  |
|  | Mix | 21665 | 3.9 dB | 2640 | 8.2 |  | 21665 | 4.0 dB | 2790 | 7.8 |  |
| - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/MLusing the precoders configured according to the UE reports. - trnd is the throughput measured at SNRue_AI/ML with random precoding. trnd is obtained with Rel-15 type I single panel codebook - Beam steer is not added |  |  |  |  |  |  |  |  |  |  |  |

Table 5. Simulation results(step-3) for AIML based CSI prediction, MCS=17

| TX | Train | 20Hz Doppler spread |  |  |  |  | 50Hz Doppler spread |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  |
| 16 | Non-mix | 29038 | 5.8 dB | 1660 | 17.5 |  | 29038 | 7.6 dB | 4332 | 6.7 |  |
|  | Mix | 29038 | 5.7 dB | 1550 | 18.7 |  | 29038 | 7.8 dB | 4885 | 5.9 |  |
| - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/MLusing the precoders configured according to the UE reports. - trnd is the throughput measured at SNRue_AI/ML with random precoding. trnd is obtained with Rel-15 type I single panel codebook - Beam steer is not added |  |  |  |  |  |  |  |  |  |  |  |

Table 6. Simulation results(step-3) for AIML based CSI prediction, MCS=19

| TX | Train | 20Hz Doppler spread |  |  |  |  | 50Hz Doppler spread |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  | tue_AI/ML (bits/slot) | SNRue_AI/ML | trnd (bits/slot) | γAI/ML |  |
| 16 | Non-mix | 34106 | 6.8 dB | 763 | 44.7 |  | 34106 | 9.0 dB | 4080 | 8.3 |  |
|  | Mix | 34106 | 6.7 dB | 711 | 47.9 |  | 34106 | 9.1 dB | 4497 | 7.6 |  |
| - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/MLusing the precoders configured according to the UE reports. - trnd is the throughput measured at SNRue_AI/ML with random precoding. trnd is obtained with Rel-15 type I single panel codebook - Beam steer is not added |  |  |  |  |  |  |  |  |  |  |  |

After accounting for CSI feedback, for TDL channels, the SGCS2 (calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel) deteriorates significantly. This may be due to the codebook-based CSI feedback inherently introduces quantization errors compared to the SVD eigenvectors from the ground truth channel. However, when the ground truth eigenvectors also incorporate the quantization errors from CSI feedback, the SGCS3 (calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel) shows much less degradation.

Regarding the simulation results of step-3, under the condition of TX=16 and rank=2, the throughput performance obtained by randomly selecting a Type I codebook is relatively poor. However, considering that the current baseline benchmark for PMI evaluation in RAN4 is also based on randomly selected Type I codebooks, this ensures fairness when comparing different candidate schemes. The key difference between different schemes(e.g. CSI prediction models from different companies) will primarily be reflected in their γ values relative to the baseline benchmark.

Regarding the updated simulation for AI/ML-based CSI prediction, the updated simulation results are shown in Table 5 for MCS=17 and Table 6 for MCS=19.

For step-2, the SGCS calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel deteriorates significantly. The SGCS calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel shows much less degradation.

For step-3, under the condition of TX=16 and rank=2, the throughput performance obtained by randomly selecting a Type I codebook is relatively poor. The key difference between these schemes (e.g. models from different companies) will primarily be reflected in their γ values relative to the baseline benchmark.

Conclusion

In this contribution, we have discussed the demodulation and/or CSI reporting requirements for CSI prediction, and got the following proposals:

Observation 1: 	Regarding the updated simulation for AI/ML-based CSI prediction, the updated simulation results are shown in Table 5 for MCS=17 and Table 6 for MCS=19.

Observation 2: 	For step-2, the SGCS calculated by comparing the feedbacked SVD of the model output with the SVD of the ground truth raw channel deteriorates significantly. The SGCS calculated by comparing the feedbacked SVD of the model output with the feedbacked SVD of the ground truth raw channel shows much less degradation.

Observation 3: 	For step-3, under the condition of TX=16 and rank=2, the throughput performance obtained by randomly selecting a Type I codebook is relatively poor. The key difference between these schemes (e.g. model from different companies) will primarily be reflected in their γ values relative to the baseline benchmark.

Reference

R4-2511796 WF on updated simulation assumptions for AI based CSI prediction

RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2