3GPP RAN WG4 Meeting #116bis		R4-2513849

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	7.1.4

Source:	Apple

Title:	On the introduction of PC1.5 with 2Tx for FDD bands

WI/SI:	NR_UE_RF_Ph5-Core

Release:	Rel-20

Document for:	Discussion

# 1	Introduction

The WI [1] has the goal to introduce PC1.5 with 2Tx for FDD bands. The objectives are captured as follows:

# 2  Discussion

##### 2.1 Antenna isolation assumption for FDD bands

Antenna isolation plays an important role in determining MPR for 2Tx configurations. The isolation performance has direct impact on the Tx RF performance. Low isolation between transmit antennas can lead to significant coupling effects, where power from one transmitter interferes with the other, potentially causing distortion, increased spurious emissions, and degraded EVM. This makes it a fundamental parameter that must be agreed upon first in the standardization process to ensure realistic and achievable MPR for FDD bands.

Observation 1: Antenna isolation is a fundamental aspect for MPR and A-MPR and needs to be analyzed and agreed before discussion on MPR and A-MPR can take place.

Proposal 1: RAN4 should start the discussion on PC1.5 for FDD bands by analyzing antenna isolation for low and mid bands to make a conclusion at RAN4#117.

##### 2.2 Reference sensitivity degradation (RSD) from PC3 to PC1.5 with 2Tx

- RSD for n1:

Due to wide duplex gap for n1 band, the Tx noise at the Rx port is extremely low (well below the thermal noise), based on our modelling. As a result, our calculations show that the RSD for n1 from PC3 to PCX1.5 with Tx diversity is negligible.

Observation 2: For band n1, due to the wide duplex gap, the TX noise at the RX input is extremely low (well below the thermal noise). The RSD due from PC3 to PC1.5 with 2Tx is negligible.

Proposal 2: Adopt the RSD requirements for n1 from PC3 to PC1.5 with 2Tx as shown in Table 2.2.1-1.

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |

Table 2.2.1-1 Reference Sensitivity Degradation from PC3 to PC2 for
FDD bands for UE supporting Tx Diversity

- RSD for n25:

For n25 RSD analysis, we used the existing requirements for n25 RSD from PC3 to PC2 with 2Tx ( see Table 7.3.2-1d of Clause 7.3.2 of TS38.101-1) to derive the Tx noise at the antenna. Then we added 3dB to those levels to get the noise levels for PC1.5 2Tx. The resulting RSD values for PC1.5 with 2Tx are shown is Table 2.2.1-2.

Proposal 3: Adopt the RSD requirements for n25 from PC3 to PC1.5 with 2Tx as shown in the Table below.

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 |  | 3.6 | 3.6 | 3.7 | 3.7 | 3.9 | 8.7 | 8.9 | 9.4 | 9.8 |  |

Table 2.2.1-2 Reference Sensitivity Degradation from PC3 to PC2 for
FDD bands for UE supporting Tx Diversity

# 4	Conclusions

This contribution provides initial insight and discussion on 2Tx PC1.5 for FDD. The following proposals and observations are made:

Observation 1: Antenna isolation is a fundamental aspect for MPR and A-MPR and needs to be analyzed and agreed before discussion on MPR and A-MPR can take place.

Proposal 1: RAN4 should start the discussion on PC1.5 for FDD bands by analyzing antenna isolation for low and mid bands to make a conclusion at RAN4#117.

Observation 2: For band n1, due to the wide duplex gap, the TX noise at the RX input is extremely low (well below the thermal noise). The RSD from PC3 to PC1.5 with 2Tx is negligible.

Proposal 2: Adopt the RSD requirements for n1 from PC3 to PC1.5 with 2Tx as shown in Table 2.2.1-1.

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |

Table 2.2.1-1 Reference Sensitivity Degradation from PC3 to PC2 for
FDD bands for UE supporting Tx Diversity

Proposal 3: Adopt the RSD requirements for n25 from PC3 to PC1.5 with 2Tx as shown in the Table below.

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 |  | 3.6 | 3.6 | 3.7 | 3.7 | 3.9 | 8.7 | 8.9 | 9.4 | 9.8 |  |

Table 2.2.1-2 Reference Sensitivity Degradation from PC3 to PC2 for
FDD bands for UE supporting Tx Diversity

# 4	References

- RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, vivo, 3GPP TSG RAN Meeting #109