3GPP TSG-RAN WG4 Meeting #116bis	R4-2513873

Prague, CZ, Oct. 13 – 17, 2025

Agenda item:	6.16.4

Source:	Samsung

Title:	Discussion on UE demodulation requirements for NR LP-WUS and receiver

Document for:	Discussion

- Introduction

The Rel-19 WI on “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)” was approved in the RAN#102 meeting and revised in the RAN#105 meeting [2] along with the completion of the Rel-18 SI on low-power wake-up signal and receiver for NR [3], in which the performance part starts from RAN4#115 in May 2025 and targeting to complete by RAN#111 in March 2026. The objectives for performance part are as below:

- Specify the UE demodulation performance and test cases for LP-WUS/WUR (RAN4). - Study testability of above requirements - Specify RRM measurement performance requirement if identified, and corresponding test cases for the LP-WUS/WUR (RAN4).

In RAN4#115 and RAN4#116 meeting, the work plan and some issues about testability aspects and test assumptions were agreed [4][5]. In this contribution, we share our views and analysis on the testability aspects and assumptions for demodulation requirements of NR LP-WUS and receiver.

- Testability Aspects

About testability aspects, below agreements and WF achieved in last RAN4#116 meeting [5].

Issue 1-1-1: Whether to define a test mode for LP-WUS/WUR - Agreement: Interested companies can provide an analysis of the expected test durations for the different modes next meeting. Further discuss if a test mode is required based on the provided analysis. Issue 1-1-2: Whether to consider shortening the test duration - Proposals - Option 1: Yes, if FAR test is introduced, introduce new test mode to reduce the FAR test run time. - Option 2: Based on the test mode, and the necessity to make sure the test confidence, it seems no need to add additional assumptions in order to shortening the test duration. Issue 1-1-3: Whether to define separate test cases for idle/inactive and connected modes - Proposals - Option 1: Define separate test cases procedure for the modulator in idle and connected modes.

Whether to define a test mode for LP-WUS/WUR

According RAN1’s design, no ACK/NACK for LP-WUS should be replied to network by UE, whether UE is listening to LP-WUS is transparent to network. Hence the test for LP-WUS/WUR should depends on other signal(s) in IDLE state or ACK/NACK of other signal(s) in CONNECTED state. The discussion about testability for LP-WUS on RF session get below agreements [6] and leave the decision about the potential test mode to RAN5.

TR38.774 Section 8.1.1 General Framework

It has been agreed that same LP-WUS RF requirements will apply to all RRC states, but only one RRC state needs to be tested. It is agreed to verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods). The ultimate test method including the potential test mode will be decided by RAN5 based on RAN4 input. For demodulation requirements FAR will also be used.

The motivation behind the proposal to investigate a test mode stems from two key concerns: the potential for excessively long test execution time and the operational inefficiency caused by frequent transitions between MR (Main Radio) and LR (Low power wakeup receiver). To address the concern about test execution time and evaluate the feasibility, it was agreed upon in the previous meeting that a preliminary estimation of the total test duration would be conducted and presented during upcoming session.

Generally, the steps for one test sample cycle could be summarized as

      - gNB/TE sends LP-WUS

      - UE receives the LP-WUS

      - UE decodes the LP-WUS

      - UE wakeup the MR, MR starts PDCCH monitoring

      - For IDLE state, UE sends MSG1(PRACH) to gNB/TE, receives MSG2 (RAR), transmits MSG3 to gNB/TE

For CONNECTED state, UE receives PDSCH and sends ACK/NACK to gNB/TE

      - No signal(s) for a long time

      - MR goes to sleep

RAN1 introduced the definition of wake-up delay which is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring as below, which corresponding to the time duration from step 2) to step 4) in one test sample cycle. At the same time, UE will turn its MR to sleep if no signals to receive or transmit for a time period, but this time period depends on UE implementation. We assume it should be no less than the Inactivity Timer, which typically set as 10s. The final time duration for one test sample cycle should be the sum of wake-up delay and the silent time period in which no signals to receiver or transmit (after this silent time period, MR goes to sleep). If we select the wake-up delay as 1200ms (1.2s), then time needed for one test sample cycle should be 11.2s.

Agreement The 3 capabilities for UE’s wake-up delay in idle/inactive mode are defined as follows: SSB periodicity (ms); Wake-up delay (ms) UE capability 1; Wake-up delay (ms) UE capability 2; Wake-up delay (ms) UE capability 3. 5/10/20; 70; 500; 900. 40; 130; 600; 1000. 80; 250; 800; 1200. 160; 490; 1200; 1600.

Observation 1: The time duration for one test sample cycle should be the sum of wake-up delay and the silent time period in which no signals to receive or transmit for MR turns to sleep. We assume the time period should be no less than the Inactivity Timer, which typically set as 10s. If we select the wake-up delay as 1200ms (1.2s), then time needed for one test sample cycle should be 11.2s.

For the LP-WUS demodulation test, the agreement was achieved about the test metric for MDR as below in RAN4#115 meeting [4], and the agreement was reached about FAR as the test assumption for MDR as below in RAN4#116 meeting [5].

RAN4#115 meeting agreement

Test metric for MDR test - Agreement - 1% MDR

RAN4#116 meeting agreement

Assumption of FAR for the MDR test - Agreement: 1% FAR

In order to promise the test reliability, we have to balance the statistical confidence and relative precision and reach a reasonable number of observed error events Nerrors. The number of errors in N trails follows a Binomial distribution. For a large number of samples, this can be approximated by a Normal distribution. The accuracy of the estimate is determined by how many errors you count. We typically want the estimate to be within a certain percentage (the relative precision, ε) of the true value with a certain statistical confidence, e.g., 95%. To achieve a relative precision ε at a (1-α) confidence level, the required minimum number of error events is given by

Nerrors≥Zα2ε21-MDRMDR

Where,

Zα2 is the quantile of the standard normal distribution, for 95% confidence, Z=1.960.

ε is the desired relative precision (e.g., 0.1 for 10%).

MDR is the expected true miss detection rate.

Then the error events Nerrors should be 38032 corresponding to 1% MDR, 95% confidence level and 10% relative precision, which is really high standard and hard to reach in practical test.

In practical engineering, a well-established and acceptable rule of thumb is used to balance accuracy and test time. For a target MDR of 0.01,

Nerrors≥100.01=1000

The total number of samples should be

Nsamples≥ NerrorsMDR=10000.01=100000

Which is also aligned with RAN5’s definition about test time of PDCCH demodulation case as below.

TS38.521-4 G.1.5 Minimum Test time

Table G.1.5-2: Minimum Test time for PDCCH demodulation Reference Channel; Demodulation scenario (doppler speed); Minimum number of active subframes (MNAS); MNAS to MNS Scaling factor (Note 3); Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS=. R.PDCCH 1-1.1 FDD, R.PDCCH.1-1.2 FDD, R.PDCCH.1-1.3 FDD, R.PDCCH.1-2.1 FDD, R.PDCCH.1-2.2 FDD, R.PDCCH.1-2.3 FDD, R.PDCCH.1-2.4 FDD, R.PDCCH.1-2.5 FDD, R.PDCCH.1-2.6 FDD; 10, 100, 400 Hz; 100000 (Note 1); 1.0526; 106000. R.PDCCH.2-1.1 TDD, R.PDCCH.2-1.2 TDD, R.PDCCH.2-2.1 TDD, R.PDCCH.2-1.3 TDD, R.PDCCH.2-1.5 TDD, R.PDCCH.2-1.6 TDD; 10, 100, 400 Hz; 100000 (Note 1); 1.2903; 130000. R.PDCCH.5-1.1 TDD, R.PDCCH.5-1.2 TDD, R.PDCCH.5-1.3 TDD, R.PDCCH.5-2.1 TDD; 75, 300 Hz; 100000 (Note 2); 1.25; 130000. R.PDCCH.1-2.4 FDD+ R.PDCCH.1-2.7 FDD; 10Hz; 100000 (Note 1); 2 (Note 4); 200000. R.PDCCH.2-1.2 TDD+ R.PDCCH.2-1.4 TDD; 100 Hz; 100000 (Note 1); 2.666 (Note 4); 267000. R.PDCCH.5-1.2 TDD, R.PDCCH.5-1.4 TDD; 300 Hz; 100000 (Note 2); 2.58 (Note 4); 258000. Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. Note 4: For power saving, MNS/MNAS ratio decided by scheduling pattern and DRX configuration, and is ratio of all slots in both DRX active time and DRX inactive time to DL slots in DRX active time..

Observation 2: The test samples for 1% MDR should be no less than 100000, which is aligned with RAN5’s definition about test time of PDCCH demodulation case.

Therefore, if without test mode, the test execution time for the MDR test should 100000*11.2s, about 311 hours.

Observation 3: If without test mode, the test execution time for LP-WUS MDR test is about 311 hours, with 100000 test samples and test duration of each sample as 11.2s.

According this test time analysis, we think the testability for LP-WUS MDR test has problem since the test time is too long. Hence a test mode should be defined if demodulation requirements for LP-WUS MDR test will be introduced.

Proposal 1:  Test mode is recommended for LP-WUS MDR test considering the test time is too long.

Whether to consider shortening the test duration

As we analysed above, a test mode is recommended to shorten the test duration. The expected test mode is UE could provide feedback for LP-WUS to help the detection rate statistic and keep MR sleep during MDR test.

Proposal 2: Based on the test time analysis above, a test mode is recommended to shorten the test duration.

Proposal 3: The expected test mode is UE could provide feedback for LP-WUS to help the detection rate statistic and keep MR sleep during MDR test.

Whether to define separate test cases for idle/inactive and connected modes

As we analysed in [7], according to the UE feature list [8] attached in the RAN1 LS [9], UE has full flexibility in terms of its supported RRC states. UE can support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) separately, and UE can also support IDLE/INACTIVE mode operation (FG62-1(a)) and CONNECTED mode operation (FG62-2(a)) together, i.e., there could be 3 types of UE in terms of supported RRC states:

- UEs supporting IDLE/INACTIVE mode only

- UEs supporting CONNECTED mode only

- UEs supporting both IDLE/INACTIVE mode and CONNECTED mode

| Features | Index | Feature group |
| --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

For FR1, supporting LP-WUS in RRC_Idle states showed most obvious power saving benefits [3]. Though UE may support not all 3 RRC states, the large commercialization possibility is that the FR1 LP-WUS UE would either support RRC_Idle or support RRC_Idle & RRC_Connected states. Therefore, it can be expected that test method based on RRC_Idle could cover most available UEs. It is reasonable to prioritize RRC_Idle if Demod requirements for LP-WUS will be defined only for one RRC state.

Proposal 4: If RAN4 define Demod test cases for two RRC states, separate test cases should be defined for idle/inactive and connected modes. If RAN4 define Demod test cases for only one RRC state, RRC_Idle should be prioritized.

- Assumptions for Demodulation Requirements

FAR test

About the FAR (False Alarm Rate), RAN4 Demod WF [5] as below and further discussion are needed. It is clearly that if FAR is too high, the power saving gain will be impacted, however the impact of FAR is not as critical as the impact as MDR (miss MR wake-up) and we have agreed to set FAR as 1% as the assumption for MDR test. In RAN1’s study, the FAR only from noise, doesn’t include the impact of the falsely alarmed from wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group. And from RAN1’s simulation results in [3], the difference between 1% and 0.1% FAR are not obvious. RAN1 also down select FAR to 1% for RRC idle/inactive state in RAN1#117 meeting. Therefore, considering the test effort and the FAR impact, 1% seems enough.

Issue 2-1-2: Whether to introduce FAR test - Proposals - Option 1: Yes - Option 2: No Issue 2-1-3: Test metric for FAR test (if introduced) - Proposals - Option 1: 1% FAR - Option 2: 0.1% FAR

RAN1#117 Chairman notes

Agreement For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS: - Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24 - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N. - Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N. - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1 - How to satisfy FAR is reported by companies, e.g., FEC/CRC - Note: multiple TDMed LP-WUSs can be used to support more subgroups for each option. - Note: Y% effective paging rate per PO is reported by companies - The followings are considered when down-select among options: - The number of supported UE subgroups per PO: M - Average network overhead to indicate the number of UE subgroups M per PO - False wake up rate due to subgroup-based indication, which will impact the power saving gain - Paging latency - Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

Proposal 5: RAN4 do not introduce LP-WUS FAR test since the impact of FAR is not as critical as the impact as MDR (miss MR wake-up) and we have agreed to set FAR as 1% as the assumption for LP-WUS MDR test.

Proposal 6: RAN4 use 1% FAR as test metric for LP-WUS FAR test if FAR test introduced.

Type 1 and Type 2 receivers

About the definition and requirements for OOK and OFDM based LP WURs, following open issues need further discussion.

Issue 2-1-21: Whether to define single requirement for Type 1 and Type 2 receivers - Proposals - Option 1: RAN4 to study the performance for both receiver types (Type 1: OOK and Type 2: OFDM) and further discuss based on simulation results if one or two sets of requirements are to be defined. - Option 2: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection Issue 2-1-23: Whether to re-use the definition from RAN4 RF for the receiver types (if separate requirements are introduced) - Proposals - Option 1: If agreed to introduce two sets of requirements, then re-use the definition from RAN4 RF for the receiver types.

It’s no doubt RAN4 should study the performance for both OOK and OFDM based LP WUR firstly, and then could further discuss if two sets of requirements are necessary based on the simulation results. Furthermore, we need to point out that there are no clear mapping relationship agreed in RF session between Type 1, Type 2 receivers and OOK, OFDM receivers. In RF session, the Type 1 and Type 2 receivers are just defined with different sum values of noise figures and impairment margins, i.e., Type 1 with 18dB and Type 2 with 13.5dB. Hence, it is not suitable for Demod session to reuse the definition of Type 1 and Type 2 LP WUR in RF session.

Proposal 7: RAN4 Demod should study the performance for both OOK and OFDM based LP WUR firstly, and then could further discuss if two sets of requirements are necessary based on the simulation results.

Observation 4: In RF session, the Type 1 and Type 2 receivers are just defined with different sum values of noise figures and impairment margins, i.e., Type 1 with 18dB and Type 2 with 13.5dB.

Proposal 8: RAN4 Demod session do not reuse the RF session definition of Type 1 and Type 2 since no clear mapping relationship agreed in RF session between Type 1, Type 2 receivers and OOK, OFDM based receivers.

Inclusion of requirements in the specification

Issue 2-1-27: Inclusion of requirements in the specification - Proposals - Option 1: - RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1. - RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

For the inclusion of requirements in the specification, it is straightforward to add new subclauses for WUS demodulation requirements if introduced.

Proposal 9: For the inclusion of requirements in the specification

- RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1.

- RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

- Conclusion

In this contribution, we provide analysis and views on the testability aspects and assumptions for demodulation requirements of Rel-19 LP-WUS. The observations and proposals could be summarized as:

Observation 1: The time duration for one test sample cycle should be the sum of wake-up delay and the silent time period in which no signals to receive or transmit for MR turns to sleep. We assume the time period should be no less than the Inactivity Timer, which typically set as 10s. If we select the wake-up delay as 1200ms (1.2s), then time needed for one test sample cycle should be 11.2s.

Observation 2: The test samples for 1% MDR should be no less than 100000, which is aligned with RAN5’s definition about test time of PDCCH demodulation case.

Observation 3: If without test mode, the test execution time for LP-WUS MDR test is about 311 hours, with 100000 test samples and test duration of each sample as 11.2s.

Proposal 1:  Test mode is recommended for LP-WUS MDR test considering the test time is too long.

Proposal 2: Based on the test time analysis above, a test mode is recommended to shorten the test duration.

Proposal 3: The expected test mode is UE could provide feedback for LP-WUS to help the detection rate statistic and keep MR sleep during MDR test.

Proposal 4: If RAN4 define Demod test cases for two RRC states, separate test cases should be defined for idle/inactive and connected modes. If RAN4 define Demod test cases for only one RRC state, RRC_Idle should be prioritized.

Proposal 5: RAN4 do not introduce LP-WUS FAR test since the impact of FAR is not as critical as the impact as MDR (miss MR wake-up) and we have agreed to set FAR as 1% as the assumption for LP-WUS MDR test.

Proposal 6: RAN4 use 1% FAR as test metric for LP-WUS FAR test if FAR test introduced.

Proposal 7: RAN4 Demod should study the performance for both OOK and OFDM based LP WUR firstly, and then could further discuss if two sets of requirements are necessary based on the simulation results.

Observation 4: In RF session, the Type 1 and Type 2 receivers are just defined with different sum values of noise figures and impairment margins, i.e., Type 1 with 18dB and Type 2 with 13.5dB.

Proposal 8: RAN4 Demod session do not reuse the RF session definition of Type 1 and Type 2 since no clear mapping relationship agreed in RF session between Type 1, Type 2 receivers and OOK, OFDM based receivers.

Proposal 9: For the inclusion of requirements in the specification

- RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1.

- RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

- Reference

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC (moderator, RAN VC), 3GPP TSG RAN Meeting #102, Edinburgh, GB, December 11-15, 2023.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

- TR38.869, Study on low-power wake up signal and receiver for NR, Release 18.

- R4-2508626, WF for [115][332] NR_LPWUS_Demod, Qualcomm, 3GPP TSG RAN4 Meeting #115, Malta, May 19-23, 2025

- R4-2512526, WF for [116][330] NR_LPWUS_Demod, RAN4#116, Bengaluru, India, Aug25-29, 2025

- TR38.774, Radio Frequency (RF) requirements for Low-Power Wake-up Signal and Receiver (Release 19)

- R4-2506970, Discussion on testability for LP-WUR, Samsung, 3GPP TSG RAN4 Meeting #115, Malta, May 19-23, 2025

- R1-2501397, Initial RAN1 UE features list for Rel-19 NR after RAN1 #120, Moderators (AT&T, NTT DOCOMO, INC.)

- R4-2503104_R1-2501399, LS on Rel-19 RAN1 UE features list for NR after RAN1#120, RAN1