3GPP TSG-RAN WG4 Meeting#116bis                                  R4-2513897

Prague, CZ, 13rd – 17th, Oct. 2025

Agenda item:	6.17.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on performance for CSI-RS based L1 measurement

Document for:	Approval

Introduction

The RRM core requirement on Mobility work item has been summarized, which include

- Event triggered L1 measurement reporting

- CSI-RS measurements for LTM procedures

- (Subsequent) Conditional LTM

This contribution provides our view on performance requirements for CSI-RS based L1 measurement.

Discussion

Based on the current status of each objective, it is possible to evaluate the required test cases at a high level. In this contribution, we provide our view based on core part. In the last meeting, we have reached consensus to define performance requirements and test cases for CSI-RS based L1-RSRP accuracy on neighbor cell. At present, we need to discuss whether and how to design test cases for CSI-RS based L1-RSRP measurement. For LTM, there are a lot of core requirements defined and there are various scenarios that may have different requirements. It’s fine for us to define a test case to cover multiple scenarios and avoiding duplicated testing on similar functions. First, we should discuss which cases can be specified.

For SSB based L1-RSRP measurement, we defined several test cases, including intra-frequency measurement, inter-frequency measurement with gap and inter-frequency without gap for both FR1 and FR2. In addition, test cases for inter-frequency L1-RSRP accuracy requirements. For CSI-RS based L1-RSRP measurement, based on agreements in core part, we don’t need to consider the test cases for measurement with gap and inter-frequency measurement.

| A.6.6.26.1 | Intra-frequency SSB based L1-RSRP measurement in FR1 |
| --- | --- |
| A.6.6.27.1 | Inter-frequency SSB based L1-RSRP measurement with measurement gap |
| A.6.6.28.1 | Inter-frequency SSB based L1-RSRP measurement without measurement gap |
| A.6.7.17.1 | Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1 |
| A.7.6.20.1 | Intra-frequency SSB based L1-RSRP measurement in FR2 |
| A.7.6.21.1 | Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM |
| A.7.6.22.1 | Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2 |
| A.7.7.15.1 | Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1 |

In addition to the validation that may need to be considered in R19 CSI-RS measurement mentioned above, in the discussion of the core part, based on the 3-step measurement in legacy, we also introduced 2-step measurements based on UE capability in R19. And a measurement delay requirement was defined for this procedure that N times of 3-step, where N=8. Based on this, both CSI-RS based L1-RSRP measurement procedures mentioned above need to be taken into account when conducting procedure validation.

Proposal 1: Verify the procedure of intra-frequency CSI-RS based L1-RSRP measurement in FR1 and FR2. For FR2, both the case with step 2 and the case without step 2 should be tested.

Regarding the CSI acquisition, although it is in the WID of R19 mobility, we do not have any requirements related to it in the core part, and the discussion of core part has already been closed in the last meeting. Therefore, in the performance part, there is no need to validate any behavior. In other words, there is no need to introduce test cases to verify the requirements corresponding to CSI acquisition.

Issue 4-2-4: others Candidate options: - Proposal 1: Define test case(s) to verify early CSI acquisition if core requirements are specified. (vivo) - Proposal 1a: Define test case(s) to verify early CSI acquisition before the cell switch command, conditionally to core requirement definition. (Nokia) - Proposal 1b: Define test case(s) to verify early CSI acquisition after the cell switch command, conditionally to core requirement definition. (Nokia) - Proposal 2: Discuss whether there is need to consider configuration where CSI-acquisition resource(s) are outside of the active BWP. (Nokia) Recommended WF: - Discuss the proposals.

Observation 1: No core requirements are specified on CSI acquisition. And there is no need to introduce test cases.

Conclusions

In this contribution, we put forward the following proposal on performance part for mobility enhancements.

Proposal 1: Verify the procedure of intra-frequency CSI-RS based L1-RSRP measurement in FR1 and FR2. For FR2, both the case with step 2 and the case without step 2 should be tested.

Observation 1: No core requirements are specified on CSI acquisition. And there is no need to introduce test cases.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple