# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513925

# Prague, CZ, 13th-17th Oct., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on general aspects for 4Tx HPUE

# Agenda Item:   7.1.2

# Document for:  Approval

- Introduction

In last RAN plenary meeting, a new WID on UE RF requirement enhancements for the deployment in NR FR1 was approved [1]. The objective for HPUE of this WID is:

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

In this contribution, we provide some of our considerations on the power class of HPUE 4Tx with 4x26dBm PA configuration has been confirmed for CPE/FWA UE.

- Discussion

In the WID, 4Tx with 4x26dBm PA configuration has been confirmed for CPE/FWA UE TDD single carrier with the maximum output power (MOP) up to 32dBm, where 4Tx is not a new configuration but PC1/([PC1b]) 4x26dBm PA configuration is new for TN UE. In Rel-18, 4Tx for PC1.5 CPE/FWA UE with 4x23dBm, 2x23dBm+2x26dBm, 4x26dBm PA configurations were discussed and corresponding 4x4 UL MIMO and TxD requirements were specified in TS38.101-1. In Rel-19, 4Tx PC1 NTN non-handheld UE was discussed and corresponding MOP, MPR and A-MPR requirements were specified in TS38.101-5. Currently PC1 TN UE with 31dBm MOP is only defined as single carrier operation without supporting UL MIMO. The original purpose for PC1 band (e.g. n14) is for public safety scenario. Later more NR bands were configured to support PC1 UE but not for public safety scenario.

During the discussion in RAN#109 meeting, companies had very different views on the MOP of 4Tx CPE/FWA UE with 4x26dBm PA configuration and also the power class. Some companies proposed that it is possible to achieve 32dBm MOP with 4x26dBm PA configuration considering full PA power capabilities. Although higher maximum transmission power is beneficial, a new power class, maybe PC1bis, needs to be introduced to avoid conflict with current PC1 with 31dBm MOP. With introducing of a new power class, the co-existence simulation should be studied first for ACLR evaluation to guarantee a UE with higher power class cannot cause high interference for the existing system. Accordingly, a series of new RF requirements for this new power class should be specified, which is a quite new work. Frankly speaking, it is not a good start point to decide a new power class at the very beginning.

First, power class is a capability signalling and 32dBm MOP is within the range of PC1 31dBm MOP with +2/-3dB tolerance. UE PHR depends on PCMAX defined in RAN4 specification, where a range of values are defined. It means that UE is not allowed to always transmit with MOP because of power backoff caused by ACLR, SEM, IBE, EVM requirements and network configuration. The UL performance not only depends on MOP, but also many other aspects. Moreover, 4Tx with 4x26dBm PA configuration will cause more serious reverse IMD (RIMD) compared to legacy UE. With higher PA output power in each chain, more severe RIMD products can be foreseen, which may further limit UE’s output power. This phenomenon has be pointed out and tested in Skyworks’ contribution in RAN4#116 meeting [2]. In this case, unless obvious benefit to improve 1dB compared to legacy PC1 31dBm MOP and the necessity of an additional power class are verified, current PC1 capability should be applied.

Observation 1: 32dBm output power is within the range of PC1 31dBm maximum output power with +2/-3dB tolerance.

Observation 2: UE is not allowed to always transmit with maximum output power because of power backoff and network configuration. 4Tx with 4x26dBm PA configuration will cause more serious reverse IMD compared to legacy UE, which may further limit UE’s output power.

Second, an additional power class with different MOP will have impact on RAN2 specification. Currently in RAN2 specification P-Max is up to 33dBm. If we define a new power class with 32dBm MOP, with widely recognized +2dB upper value of tolerance, then RAN2 needs to redefine P-Max.

Observation 3: A new power class with 32dBm maximum output power will impact current RAN2 specification.

Third, there already exists 4Tx PC1 NTN HPUE with 4x26dBm PA configuration and 31dBm MOP in Rel-19. To avoid inconsistency between TN and NTN, current PC1 is a better choice.

Observation 4: 4Tx PC1 NTN HPUE has been introduced in Rel-19.

Overall, we prefer to define 4Tx CPE/FWA UE with 4x26dBm PA configuration as PC1 UE. For the new power class with 32dBm MOP, to achieve the same power as 31dBm+2dB, only +1dB upper tolerance can be defined to keep the 33dBm P-Max value. In addition, we are at the later stage of 5G-Adv, to introduce a new power class with higher power with high workload in RAN4 and RAN2 impacts as well may not be a good choice.

If companies think additional 1dB is key for UL performance, RAN4 can further study how to deal with this potential 1dB more output power and how to reflect it in other aspects rather than MOP and tolerance, such as configured transmitted power, PHR or something else.

Proposal 1: Define 4Tx CPE/FWA UE with 4x26dBm PA configuration as PC1 UE with 31dB maximum output power and +2/-3dB tolerance.

Proposal 2: If necessary, RAN4 FFS how to deal with the potential 1dB more output power.

- Conclusion

Observations:

Observation 1: 32dBm output power is in the range of PC1 31dBm maximum output power with +2/-3dB tolerance.

Observation 2: UE is not allowed to always transmit with maximum output power because of power backoff and network configuration. 4Tx with 4x26dBm PA configuration will cause more serious reverse IMD compared to legacy UE, which may further limit UE’s output power.

Observation 3: A new power class with 32dBm maximum output power will impact current RAN2 specification.

Observation 4: 4Tx PC1 NTN HPUE has been introduced in Rel-19.

Proposals:

Proposal 1: Define 4Tx CPE/FWA UE with 4x26dBm PA configuration as PC1 UE with 31dB maximum output power and +2/-3dB tolerance.

Proposal 2: If necessary, RAN4 FFS how to deal with the potential 1dB more output power.

- Reference

- RP-252962 New WID: UE RF enhancements for NR FR1, Phase 5, RAN#109, Beijing, CN, Sept. 2025.

- R4-2511320 MPR and antenna isolation evaluation for 4Tx PC1, RAn4#116, Skyworks, Bengaluru, India, Aug. 2025.