3GPP TSG-RAN WG4 Meeting #116bis	R4-2513956

Prague, Czech, 13th – 17th October 2025

Agenda Item:	6.1.2

Source:	Ericsson

Title:	On UE demodulation and CSI reporting requirements for 6Rx

Document for:	Discussion

# 1	Introduction

Following the agreements made in the RF session below, the following UE demodulation and CSI reporting requirements will be defined but applied to handheld UE and FWA devices separately.

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

After the discussion in RAN4 #116 meeting in Bengaluru, companies have reached several agreements regarding the test scenarios and plenty of parameter assumptions.

In this paper, we will continue presenting more views on the aspects of the parameter assumption for introducing UE demodulation and potential CSI reporting requirements.

# 2	Discussion

During the previous meeting, the following three general issues were raised by multiple companies, indicating that these topics are of broad concern and merit careful consideration and discussion going forward:

- Issue 1-1: Test applicability rule for 6Rx tests [1]

- Issue 1-2: Test applicability rule for different numbers of Rx antenna ports [1]

- Issue 1-3: Specification structure [1]

Although our position remains that introducing complex test applicability rules at this stage may be premature, we recognize the relevance of these topics. In particular, we believe that Issue 1-1 and Issue 1-2, both of which relate to the design of test applicability rules, should be put on hold until the relevant test cases are better defined and agreed upon. This approach will help avoid unnecessary revisions and ensure consistency between the applicability framework and the finalized test scenarios.

Regarding Issue 1-3, which concerns the structure of the specification, we consider this to be an editorial matter. As such, we believe that any necessary changes to the structure can be addressed directly during the Change Request (CR) process at a later stage, without requiring further discussion at this point.

In summary, we propose:

- To defer further technical discussion on Issue 1-1 and Issue 1-2 until test case definitions are sufficiently mature.

- To handle Issue 1-3 through the regular CR process, where the structural proposals can be reviewed and discussed in context.

We remain open to continued dialogue on these matters in future meetings.

Proposal 1: Postpone the discussion of test applicability rules until the relevant test cases are better defined

Proposal 2: Postpone the discussion on the specification structure

## 2.1	PDSCH demodulation requirements

4-layer

For PDSCH requirements for 4 MIMO layers, it’s been agreed to consider MCS17 (Table 1) for case of FWA/handheld UEs with TDLA30-10 plus ULA low.

Issue 2-2-2: MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low - Agreement: - MCS 17 (Table 1)

However, the MIMO correlation for 4-layer case has not been agreed yet. Followings are the candidate options received so far:

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE - Proposals - Option 1: One single test case that is applicable for both FWA and Handheld UE - Option 1a: α = 0 and β = 0.07179 - Option 1b: α = 0 and β = 0 - Option 2: α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

From the perspective of performance, as evidenced by our simulation results below, summarized in the table below, we evaluated the case with MIMO correlation: α = 0 and β = 0.07179.

Table 2.1-2 Simulation for comparing different correlations

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 4 | 10 / 15 | MCS 17 (Table 1) | TDLA30-10 | 4x6, α = 0 and β = 0.07179 | 70 | 22 |

From the simulation results above, it can be observed that although the performance under the correlation configured with option 1 is not extremely too high to define requirements, it still  considered a less favorable option since it is with a relatively high operating SNR point (over 26dB after adding additional margins). The fact that ULA low correlation was not simulated should not be interpreted as an intention to exclude it; rather, it reflects our confidence that, under this correlation setting, the UE would operate properly and the resulting SNR would remain at a healthy level.

On the other hand, with respect to option 1 (a new MIMO correlation out of specification), its consideration should be proved by potential evidence that may be provided to demonstrate whether the UE, when receiving a transmission of 4 MIMO layers using 6 receive antennas, would experience a low or high level of correlation. On the contrary, reusing what has already been defined in the spec. is preferable, as it has already been justified and tested. Since MIMO correlation is strongly implementation-dependent, RAN4 would need to receive justification or even measurement data to determine whether the assumption should be low or high, and this justification should be widely agreed among companies. At present, only proposals from some companies have been received, but without detailed justification or indication that the proposals are based on measurement data.

What’s more, defining two separate cases will have to refer to the applicability rule which requires the definition of two UE types, for which currently have not been defined clearly in either core or performance specifications.

Proposal 3: Consider option 1a as the MIMO correlation for 4-layer test cases

6-layer

After the discussion in RAN4 #116 meeting in Bengaluru, it’s rather clear what to be assumed for the cases of 6 MIMO layers for FWA. Many of the simulation assumptions have already reached agreement. However, two issues remain unresolved. The first concerns the test applicability rule for FWA. Although only a single proposal has been put forward thus far, our position remains consistent—namely, to deprioritize the discussion on test applicability until RAN4 has established a clear case list for either conducting simulations or more importantly, for defining requirements.

Proposal 4: Postpone the discussion of test applicability rule for FWA

The other issue concerns how to assume/configure the DMRS. To date, two options have been proposed: in addition to the commonly used Rel-15 Type 1 with DMRS 2+2, there is clearly also a preference in some quarters for selecting Rel-18 eType I DMRS 1+1. While we acknowledge that the latter may offer certain advantages over the former under specific conditions—such as lower overhead—we continue to hold the view that binding an optional feature to the testing of 6 MIMO layers would, quite evidently, narrow the test coverage.

From a technical standpoint, we are willing to consider the Rel-18 eType I DMRS configuration, provided it can be included as an additional case. In other words, if there is to be only one test case for 6 MIMO layers for FWA, then we propose to choose option 1. However, if two test cases can be broadly agreed upon, we would of course support—and be pleased to see—the Rel-18 eType I DMRS included as one of them.

Proposal 5: Consider Rel-15 Type 1 DMRS 2+2 if there will be only one case for 6 MIMO layer for FWA test

## 2.2	CSI reporting requirements

Similar to the case of Rel-18 8Rx, it is necessary to define CQI reporting requirements for both handheld UEs and FWA UEs equipped with 6Rx. For handheld UEs and FWAs that support up to 6Rx only, the existing 8Rx CQI reporting requirements do not provide adequate coverage. Accordingly, the following key assumptions are proposed:

- Rank: 4

- Propagation channel: AWGN

- Antenna: 4T6R with static channel specified in Annex B.1

Except for the above, it’s preferred in the meantime to reuse other parameter assumptions for 8Rx CSI reporting requirements.

In general, we propose to define one case and apply it to both handheld UEs and FWA UEs, which means there is no need to define additional applicability rules for 6Rx CQI reporting requirements.

Proposal 6: Define CQI reporting requirements for 6Rx reception

Proposal 7: Consider following parameter assumptions:

- Rank: 4

- Propagation channel: AWGN

- Antenna: 4x6 with static channel specified in Annex B.1

Proposal 8: RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rules.

## 2.3	SDR requirements

The purpose of the SDR requirements is to verify that Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. It is considered essential to define the SDR requirement for the case of 6-receiver reception. The items that currently require discussion and decision are as follows:

- To decide whether to define SDR requirements for 4 MIMO layers.

Regarding whether to consider 4 MIMO layers with 1024QAM, we choose to objectively inspect the simulation results of it so that to discuss the feasibility of introducing such requirement. Based on our initial simulation results, it’s been observed that the necessary SNR for achieving 85% throughput is 28.9dB, which is to be seen as feasible to establish requirements.

Figure 2.3-1 PDSCH performance with 1024QAM + 4 layers

Observation 1: It’s feasible from the simulation to define 1024QAM 4-layer SDR requirements

Thus, we propose to define SDR requirements for 1024QAM with 4 layers.

Proposal 9: Define SDR requirements for 1024QAM with 4 layers

Based on the agreements of the WF [1], companies are encouraged to provide simulation results to decide the MCS value as per the agreed static channel matrix and parameters.

For QPSK/16QAM/64QAM/256QAM with 6 MIMO layers, we have conducted the following simulation cases as in the table below:

| TxRx antenna configuration | Maximum number of PDSCH MIMO layers | Maximum modulation format | MCS index Table for PDSCH | MCS | Scaling factor | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 8Tx6Rx | 6 | 6 (64QAM) EVM 6% | Table 5.1.3.1-1: MCS index table 1 | 28 | 1 | 22.9 |
|  |  |  |  | 27 |  | 20.8 |  |
|  |  |  |  | 26 |  | 18.7 |  |
|  |  |  |  | 25 |  | 17.7 |  |
|  | 6 | 8 (256QAM) EVM 3% | Table 5.1.3.1-2: MCS index table 2 | 27 |  | N/A |  |
|  |  |  |  | 26 |  | N/A |  |
|  |  |  |  | 25 |  | 25.3 |  |
|  |  |  |  | 24 |  | 23.6 |  |
|  |  |  |  | 23 |  | 21.8 |  |

From the simulation results above, we propose to consider MCS 26 for 64QAM and MCS 23 for 256QAM.

Proposal 10: Consider MCS26 for 64QAM and MCS23 for 256QAM

For 1024QAM with 2 MIMO layers, we have also conducted the following simulations:

| TxRx antenna configuration | Maximum number of PDSCH MIMO layers | Maximum modulation format | MCS index Table for PDSCH | MCS | Scaling factor | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 2Tx6Rx | 2 | 10 (1024QAM) EVM 2.5% | Table 5.1.3.1-4: MCS index table 4 | 24 |  | 25.9 |  |
|  |  |  |  | 23 |  | 23.8 |  |

We can observe that the performance of MCS23 is within a reasonable range, and thus we propose to consider MCS23 as the MCS value for 1024QAM with scaling factor 1.

Proposal 11: Consider MCS23 for 1024QAM with 2 layers

# 3	Summary

In this contribution, we shared our views for the demodulation requirement discussion for 6Rx covering both handheld UEs and FWA. Proposals are summarized as follows:

Proposal 1: Postpone the discussion of test applicability rules until the relevant test cases are better defined

Proposal 2: Postpone the discussion on the specification structure

Proposal 3: Consider option 1a as the MIMO correlation for 4-layer test cases

Proposal 4: Postpone the discussion of test applicability rule for FWA

Proposal 5: Consider Rel-15 Type 1 DMRS 2+2 if there will be only one case for 6 MIMO layer for FWA test

Proposal 6: Define CQI reporting requirements for 6Rx reception

Proposal 7: Consider following parameter assumptions:

- Rank: 4

- Propagation channel: AWGN

- Antenna: 4x6 with static channel specified in Annex B.1

Proposal 8: RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rules.

Observation 1: It’s feasible from the simulation to define 1024QAM 4-layer SDR requirements

Proposal 9: Define SDR requirements for 1024QAM with 4 layers

Proposal 10: Consider MCS26 for 64QAM and MCS23 for 256QAM

Proposal 11: Consider MCS23 for 1024QAM with 2 layers

# References

[1]	R4-2512617: Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon, RAN4 #116, Bengaluru, India

[2]	R4-2513957: Simulation results for 6Rx UE demodulation, Ericsson, RAN4 #116bis, Prague, Czech

# Annex

Parameter configurations and example test cases for information:

Table 5.2.5.1.1-2: Test parameters Parameter; Unit; Value. Test case type; Only single carrier test. TDD pattern; 7DS2U, S: 6D+4G+4U. Channel bandwidth / SCS; 40 MHz / 30 kHz. Propagation channel; 4 Layers: Refer to Issue 2-2-1 and 2-2- - TDLA30-10 ULA Low - TDLA30-10 ULA Medium 6 Layers: TDLA30-10 ULA Low. Antenna configuration; 4 Layers: 4T6R 6 Layers: 8T6R. MCS; 4 Layers - ULA Low: Option 1: MCS20 (Table 2); Option 2: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1). PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PRB bundling type; Static. PRB bundling size; 2. TRS configuration; Symbol#{5, 9}. PT-RS; Not configured. DM-RS configuration; 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Refer to Issue 2-3-3. Overhead; 0. Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx; Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1. (N1, N2), (O1, O2); 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1). Special slot (S slot) scheduling; Not schedule PDSCH in special slot for 6 layers cases. Number of HARQ Processes; 8. HARQ ACK/NACK bundling; Not configured. Maximum HARQ transmissions; 4. Redundancy version coding sequence; {0,2,3,1}. PDSCH & PDSCH DMRS Precoding configuration; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Receiver type assumption; MMSE-IRC with joint 6Rx MIMO detector. Tx EVM assumption; - 6% for QPSK/16QAM/64QAM - 3% for 256QAM. Test metric; SNR@70% max TP. Table 5.2.5.1.1-3: Minimum performance for Rank 4 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA medium B; 70; xxx. 1-2; xxx; 10 / 15; MCS17; TDLC300-100; 4x6, ULA medium B; 70; xxx. 1-3; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA Low; 70; xxx. Table 5.2.5.1.1-4: Minimum performance for Rank 6 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 2-1; xxx; 10 / 15; MCS13; TDLA30-10; 8x6, ULA Low; 70; xxx.