3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513968

Prague, CZ, October 13th – 17th 2025

Agenda Item:	6.13.2.3

Source:	Ericsson

Title:	Impact on SBFD BS RF RX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on conformance testing of BS RF RX requirements for SBFD. Discussion on conformance testing of BS RF TX requirements for SBFD and new requirements for SBFD are presented in separate submissions [1-2].

# 2	Discussion

Conformance testing for both SBFD and non-SBFD slots

It is important to ensure that conformance testing is performed for both SBFD and non-SBFD slot. Even for requirements that remain unchanged, the transmitter and receiver operate under different conditions in SBFD, where simultaneous uplink and downlink transmission leads to different hardware behaviour. Therefore, separate conformance testing is necessary for both cases to capture any deviations in RF performance.

- The transmitter and receiver operate under different conditions in SBFD and non-SBFD slots, for which separate testing is necessary.

Conformance testing shall be performed separately for both SBFD and non-SBFD slots.

RF channels to be tested for SBFD operation

For conformance testing of SBFD slots, since the SBFD-capable BS rely on analogue components, e.g., subband-specific analog filters, or interference cancellation circuits, which are frequency-selective. As a result, it becomes necessary to test the performance of these implementations not only in a single frequency location, but potentially across multiple points in the band, which means that conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are required.

For conformance testing of SBFD slots, since the SBFD-capable BS rely on analog components, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are required.

Transmitter status during conformance testing of receiver

In the approved WF [3], it requires to further study the transmitter status during conformance testing of receiver. It should be agreeable that when test OTA receiver requirements of SBFD-capable BS, the transmitter must be turned on. In our view, when conducting conformance testing of the receiver, the transmitter should always be turned on to consider internal leakage, etc. This ensures that the testing conditions accurately mirror real-world scenarios, where the transmitter is active and continuously sending signals while the receiver is monitoring and reacting to them. The presence of the transmitter during testing is essential to examine the receiver’s ability. By keeping the transmitter on, the test can evaluate how the receiver performs under typical operational conditions, where both components are actively interacting. This approach guarantees that the receiver’s performance is assessed in a realistic environment, providing a comprehensive evaluation of its capabilities.

When testing both conducted and OTA receiver requirements for an SBFD-capable BS, the transmitter should always be turned on, to ensure the receiver is evaluated under realistic scenarios.

# Conclusion

In this contribution, overview of impact on conformance testing of SBFD BS RF RX requirements is presented with the following observations and proposals.

Observation 1	The transmitter and receiver operate under different conditions in SBFD and non-SBFD slots, for which separate testing is necessary.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Conformance testing shall be performed separately for both SBFD and non-SBFD slots.

Proposal 2	For conformance testing of SBFD slots, since the SBFD-capable BS rely on analog components, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are required.

Proposal 3	When testing both conducted and OTA receiver requirements for an SBFD-capable BS, the transmitter should always be turned on, to ensure the receiver is evaluated under realistic scenarios.

# References

R4-2513967, Impact on SBFD BS RF TX requirements, Ericsson

R4-2513966, New SBFD BS RF requirements, Ericsson

R4-2512680, Way Forward for [116][307] NR_duplex_evo_BSRF, Huawei