3GPP TSG-RAN WG4 Meeting #116bis	R4-2513971

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.6.3.3

Source:	Ericsson

Title:	UE demodulation requirements with time varying Doppler shift and propagation delay model

Document for:	Discussion

# 1	Introduction

RAN4#116 agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1]. This contribution discusses the remaining open issue on the UE demodulation requirements with time-varying Doppler shift and propagation delay model.

# 2	UE demodulation requirements

## 2.1	NB-IoT UE demodulation requirements

The remaining open issue is the impact on UE demodulation requirements with the time-varying Doppler shift and propagation delay model.

Issue 2-1-2 NR NTN demodulation requirement with applying time-varying channel model agreement - Introduce an additional margin of 0.5 dB to the Rel-17 NR NTN UE demodulation performance requirements Issue 2-1-3 IoT NTN demodulation requirement with applying time-varying channel model agreement - RAN4 can take the residual frequency offset as 200Hz and time offset error as to check the demodulation requirement impact with applying time-varying doppler shift and propagation delay channel model for IoT NTN - Further discuss whether to add the additional margin based on simulation results in the next meeting

RAN4#116 agreed to define new NR NTN UE demodulation requirements with the time-varying Doppler shift and propagation delay by adding 0.5dB to the Rel-17 UE demodulation requirements. However, RAN4 did not conclude the IoT-NTN UE demodulation requirements with the time-varying Doppler shift and propagation delay.

Figure 1 shows our NPDSCH simulation result of TS 36.102 Table 8.3.1.1.1.1-2 Test 2 with and without frequency and time offset. In the simulation, we set the frequency offset and time offset to 200 Hz (0.1ppm@2GHz) and 3.16 µs (=97*Tc), according to the WF, respectively.

Figure 1:	NPDSCH throughput with/without frequency and time offset.

Table 1: Summary of NPDSCH simulation results with/without residual frequency and time offset.

|  | SNR to achieve 70% of the maximum throughput |  |
| --- | --- | --- |
|  | With FO/TO | Without FO/TO |
| TS 36.102 Table 8.3.1.1.1.1-2 Test 2 | -6.7 | -7.1 |

Table 1 shows the SNR to achieve 70% of the maximum throughput with/without FO/TO. The simulation result shows that the performance degradation due to the frequency and time offset is 0.4dB. As we discussed in RAN4#116, the assumed frequency and time offset is extremely large compared with realistic residual frequency and time offset after the pre-compensation. We therefore propose to define IoT-NTN UE demodulation requirements with the time-varying Doppler shift and propagation delay by adding 0.5dB to the Rel-18 IoT-NTN UE demodulation requirements, including NB-IoT and eMTC UEs.

Observation 1: The degradation of NPDSCH with residual frequency offset and time delay is about 0.4dB for TS 36.102 Table 8.3.1.1.1.1-2 Test 1.

Proposal 1: Define new IoT-NTN UE demodulation requirements with the time-varying Doppler shift and propagation delay by adding 0.5dB to TS 36.102 Table 8.2.1.1.1.1-2 Tests 1/2 and Table 8.3.1.1.1.1-2 Test 1.

## 2.2	Test applicability

RAN4 has agreed to define NR NTN and IoT-NTN UE demodulation requirements with the time-varying Doppler shift and propagation delay model based on the existing PDSCH demodulation requirements specified in TS 38.101-5 and TS 36.102. However, RAN4 has not concluded the test applicability between new Rel-19 requirements and the existing Rel-17/18 NR NTN and IoT-NTN UE demodulation requirements.

Issue 2-1-5 Test applicability of requirement with applying time-varying channel model WF - Option 1: - The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration. - Option 2 - For Rel-19 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18. - For Rel-17/Rel-18 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18. - Other options are not precluded

In our understanding, Rel-17/Rel-18 UE capable of satellite access should support the pre-compensation based on GNSS and ephemeris information. This means, Rel-17/Rel-18 UE should pass the new Rel-19 NR NTN and IoT-NTN UE demodulation requirements with time-varying Doppler shift and propagation delay.

We understand the UE vendors don’t want to perform the same test with and without time-varying Doppler shift and propagation delay. One solution could be to introduce a test applicability between the test cases with and without the time-varying Doppler shift and propagation delay model, that is, if a UE pass a test with the time-varying Doppler shift and propagation delay model, the UE can skip the test without the time-varying Doppler shift and propagation delay. On top of that, we define the requirements with the time-varying Doppler shift and propagation delay is release independent from Rel-17 for NR NTN and from Rel-18 for IoT-NTN, but the requirements are optional for UE Rel-17/18 UEs.

In appendix, we show an example of TP to TS 38.307.

Proposal 2: Introduce test applicability between Rel-19 tests with the time-varying Doppler shift and propagation delay model and Rel-17/Rel-18 tests without the time-varying Doppler shift and propagation delay model.

Proposal 3: Rel-19 tests with the time-varying Doppler shift and propagation delay model are release independence from Rel-17 for NR NTN and from Rel-18 for IoT-NTN.

Proposal 4: Rel-17/Rel-18 NTN UEs need to pass either tests with or without the time-varying Doppler shift and propagation delay model.

# 4	Summary

Observations:

Observation 1: The degradation of NPDSCH with residual frequency offset and time delay is about 0.4dB for TS 36.102 Table 8.3.1.1.1.1-2 Test 1.

Proposals:

Proposal 1: Define new IoT-NTN UE demodulation requirements with the time-varying Doppler shift and propagation delay by adding 0.5dB to TS 36.102 Table 8.2.1.1.1.1-2 Tests 1/2 and Table 8.3.1.1.1.1-2 Test 1.

Proposal 2: Introduce test applicability between Rel-19 tests with the time-varying Doppler shift and propagation delay model and Rel-17/Rel-18 tests without the time-varying Doppler shift and propagation delay model.

Proposal 3: Rel-19 tests with the time-varying Doppler shift and propagation delay model are release independence from Rel-17 for NR NTN and from Rel-18 for IoT-NTN.

Proposal 4: Rel-17/Rel-18 NTN UEs need to pass either tests with or without the time-varying Doppler shift and propagation delay model.

# References

- R4-2512525, “Way Forward for [116][323] NTN_testing_NGSO_channel_model_and_demod”, Samsung.

# Appendix

## A.1	TP to TS 38.307 9.2

This clause covers requirements for a Rel-P UE coming from all other release independent features that are not covered under clause 9.1.

Table 9.2-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| DMRS bundling for GSO scenario | Rel-17 | Table F.1-4 | Rel-18 WI NR_NTN_enh introduced phase continuity requirements for DMRS bundling for the GSO scenario: see Table C.1-1 |
| PDSCH demodulation with time-varying Doppler shift and propagation delay model | Rel-17 | Table F.x-y | Rel-19 WI NR_IoT_NTN_req_test_enh: see Table F.x-y. These requirements are optional for Rel-17 and Rel-18 UEs and can be executed based on UE declaration. |

## A.2	TP to TS 38.307 F.1.x Common UE demodulation requirements with time-varying Doppler shift and propagation delay for NR NTN in FR1-NTN

The requirements and test cases listed in Table F.1.x-1 are specified in Rel-P version of TS 38.101-5 [8].

Table F.1.x-1: UE demodulation requirements with time-varying Doppler shift and propagation delay for NR NTN in FR1-NTN

| Clause | Description |
| --- | --- |
| 8.2.1.2.2.1.x | PDSCH requirements with time-varying Doppler shift and propagation delay. |