3GPP TSG-RAN WG4 Meeting #116bis	R4-2514048

Prague, Czech Republic, 13-17 October, 2025

Source:	Nokia

Title:	On remaining LBCA requirements

Agenda item:	4.8.2

Document for:	Discussion

Introduction

In this contribution we discuss the remaining issues related to core part requirements for LBCA via switching.

Discussion

Activation and deactivation of switching pattern

RAN4 made the following agreements related to SCell activation delay and switching pattern activation in the last meeting:

Issue 1-3-5: Activation of Switching Pattern [Agreement]: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

The value of TLBCA parameter was left open in the agreed CR. In our view the value can be zero and the parameter can be removed.

TLBCA value is zero and the component is to be removed.

However, if the value for TLBCA is non-zero, this delay needs to be taken into account in the SCell activation delay. For example, when the SCell to be activated is known, the first step of SCell activation after receiving and processing the SCell activation command and sending HARQ/RRC response is DL synchronization through the first SSB. The first SSB on the SCell may be sent immediately after THARQ+3ms or it may be sent later, depending on when the SCell activation command is sent in relation to the SSB. If the SSB is sent on the SCell during TLBCA i.e. before the pattern is activated, not including TLBCA in the SCell activation delay or the definition of TfirstSSB would mean that the UE would need to receive the SSB on the SCell before the switching pattern is activated. This would lead to an interruption on PCell and for the need for RAN4 to define a related requirement.

Therefore, we think it is clearer to assume that if TLBCA is non-zero, the UE can receive the first SSB or perform any measurements related to unknown SCell activation when the pattern is already activated. Therefore, TLBCA parameter should be added in the SCell activation delay and any measurements related to activation (Tactivation_time) would happen only after the switching pattern is activated.

If Proposal 1 is not agreeable and TLBCA is non-zero, add TLBCA component in SCell activation delay before Tactivation_time.

The other FFS point is related to the start of the LBCA-sw pattern. In general, Option 1 for both RRC and MAC CE SCell activation mean that the UE would switch right away after the switching pattern activation delay. Figure 1 shows one example of how Option 1 and Option 2 can be interpreted. In Option 1, Figure 1 a), if the switching pattern activation delay ends in the middle of the SDL active period, the UE would have to switch immediately to the SDL. On the other hand, Figure 1 b) shows the expected UE behavior with Option 2. In that case, the UE will only switch to the SDL carrier in the next period where the SDL is active. Considering the switching gap would have to be allocated in Option 1, we think that the simplest solution is Option 2.

For MAC CE based SCell activation, according to the switching pattern,

  - UE switching to SCell not earlier than at the next switching period after n+(THARQ+3ms [+ TLBCA])/slot-length.

For RRC based direct SCell activation, according to the switching pattern,

  - UE switching to SCell not earlier than at the next switching period after n+(TRRC_Process [+ TLBCA])/slot-length.

- Option 1: UE switching to SCell immediately after n+(THARQ + 3ms [+ TLBCA])/slot-length.

- Option 2: UE switching to SCell at the next switching period after n+(THARQ + 3ms [+ TLBCA])/slot-length

Figure 1 – Interpretation of the options related to switching pattern activation

Proposals 3 and 4 have direct impact on the definition of TfirstSSB. If these proposals are agreed, TfirstSSB is the first SSB that is overlapping with a complete SCell active period after n+(THARQ+3ms [+ TLBCA])/slot-length or n+(TRRC_Process [+ TLBCA])/slot-length.

If proposals 3 and 4 are agreeable, capture the agreement in the definition of TfirstSSB, where TfirstSSB is the time to the first SSB that is overlapping with a complete SCell active period after activating the pattern.

Issue 1-3-5a: Deactivation of Switching Pattern [Agreement]: - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

Similar to SCell activation, we propose that TLBCA = 0 ms for SCell deactivation. For SCell deactivation this simply means that the component is removed.

For SCell deactivation, TLBCA = 0 ms and the component is to be removed.

Regarding the FFS point, we think this UE behavior is obvious and it is not specific for LBCA with switching. We do not see a need to clarify this in the RAN4 specification.

No need to clarify when the UE returns to PCell after SCell deactivation.

Impact of C-DRX on LBCA RRM Requirements

RAN4 introduced LBCA specific measurement delay requirements where the impact of LBCA switching pattern is taken into account in PCell and SCell measurement requirements.

Issue 1-3-1: Requirement design of activated SDL SCell L3 measurement Agreement in online session: - if the activated SCC measurement is MG-based in LB-CA, the legacy intra-frequency identification/measurement requirement with MG can be applied. - reuse the existing measurement requirements with gap for inter-frequency measurement and inter-RAT measurement in LB-CA. - Not consider the scenario and not define the requirement for measurement without gap for inter-frequency measurement and inter-RAT measurement in LB-CA. Agreement in online session: agree on the following approach, and further update on the wording is not precluded. Further discuss on the CR drafting. - if the activated SCC measurement is MG-less in LB-CA, Kp factor can be updated for activated SCC identification/measurement requirement (intra-frequency measurement without MG) for LB-CA. - Kp factor for MG-less activated SCC identification/measurement requirement in LB-CA can be updated as following: - Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, xRP_max, switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and switch pattern periodicity is the periodicity of the NW configured switch pattern for LB CA via switching, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W. - No requirement when Navailable = 0.

Regarding DRX, RAN4 never discussed how the switching pattern is applied. The agreement that was made about the Kp factor is general and applies to both non-DRX and DRX measurement delay, meaning that even when UE is in DRX and not scheduled, the UE is expected to follow the switching pattern for measurements. It is logical to assume that the pattern is active when the UE is being scheduled, but we think the pattern does not need to be followed for measurements during DRX.

It is defined in the measurement requirements that while the UE is being scheduled, non-DRX requirements apply. Hence, this would directly lead to UE following non-DRX requirements where the pattern is taken into account. However, when UE is in DRX and not scheduled, we think there is no need to take the pattern into account in the measurement delay.

Switching pattern is not applied when UE is in DRX.

  - Remove the impact of switching pattern for Kp factor for measurements during DRX.

If Proposal 8 is agreeable, inform RAN1 about the agreement in an LS

BFD/CBD and RLM enhancements

Related to BFD/CBD and RLM, the following agreements were made in the last meeting:

Issue 1-3-6: BFD/CBD requirements for SDL SCell [Agreement]: - RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching - follow the principle in issue 1-1-1 to define the BFD/CBD requirement for SDL SCell. - P factor in the existing BFD/CBD requirement can be updated for LB-CA via switching, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of BFD-RS or CBD-RS in W window. - Navailable is number of available BFD-RS or CBD-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage for P3. - Proposal 3 (LGE): RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell, and allow scheduling restriction/interruption for CBD evaluation after beam failure is triggered.

Issue 1-4-1: RLM requirement for PCell [Agreement]: - For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA. - P factor in the existing RLM measurement requirements can be updated, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of RLM-RS in W window. - Navailable is number of available RLM-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure.

Regarding the FFS issues for both topics, we understand the motivation behind such optimization. However, we do not think these proposals are mandatory to make the feature work. Therefore, we propose not to consider such optimization in the maintenance stage.

No optimization for switching pattern applicability is needed related to beam or link recovery (BFD, CBD, RLM) in Rel-19.

Conclusion

The following Observations and Proposals were made:

Proposal 1: TLBCA value is zero and the component is to be removed.

Proposal 2: If Proposal 1 is not agreeable and TLBCA is non-zero, add TLBCA component in SCell activation delay before Tactivation_time.

Proposal 3: For MAC CE based SCell activation, according to the switching pattern,

a.	UE switching to SCell not earlier than at the next switching period after n+(THARQ+3ms [+ TLBCA])/slot-length.

Proposal 4: For RRC based direct SCell activation, according to the switching pattern,

a.	UE switching to SCell not earlier than at the next switching period after n+(TRRC_Process [+ TLBCA])/slot-length.

Proposal 5: If proposals 3 and 4 are agreeable, capture the agreement in the definition of TfirstSSB, where TfirstSSB is the time to the first SSB that is overlapping with a complete SCell active period after activating the pattern.

Proposal 6: For SCell deactivation, TLBCA = 0 ms and the component is to be removed.

Proposal 7: No need to clarify when the UE returns to PCell after SCell deactivation.

Proposal 8: Switching pattern is not applied when UE is in DRX.

a.	Remove the impact of switching pattern for Kp factor for measurements during DRX.

Proposal 9: If Proposal 8 is agreeable, inform RAN1 about the agreement in an LS

Proposal 10: No optimization for switching pattern applicability is needed related to beam or link recovery (BFD, CBD, RLM) in Rel-19.