3GPP TSG-RAN WG4 Meeting #116bis	R4-2514050

Prague, Czech Republic, 13th-17th October, 2025

Source:	Nokia

Title:	Discussion on remaining UEIBM requirements

Agenda item:	4.18.2

Document for:	Discussion

Introduction

In RAN4#116 meeting the agreements were reached and all open items pertaining to core part requirements were closed. One such agreement was the reporting delay requirements when the eventInstanceCount-r19 > 1. In this document, we present our views regarding this agreement that in our view RAN4 needs to discuss in the RAN4#116bis meeting.

Discussion on reporting delay requirements with timer

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window 1-1-1 is supported?

Agreement:

For the window-based solution:

if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel

- eventInstanceCount-r19 is configured by NW

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19).

- TL1-meas_basic is the L1 measurement period which agreed in without window-based soltuion, in details, it is the maximum of L1-RSRP measurement periods on the beams corresponding to the event

The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic , and no requirement when eventDetectionTimeWindowLength-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

Send the LS capturing the RAN4 agreements to RAN1/2, and the action requested from RAN4 to RAN1/2 is to take RAN4 agreements into account.

In the last RAN4 meeting, it was agreed to define requirements for the case where the eventInstanceCount-r19 > 1

The agreement at present captures a scenario where the report is triggered within a timer that starts from the time the event condition is met for the first time. However, it is not necessary that the report will be triggered by a first timer itself. Instead, it is possible that the report is triggered by a timer that is a second timer or even later.  In such case, it is unclear how the UE shall decide about restarting the timer, so the starting point of the reporting delay when this later timer triggers the report is not obvious.

An event reporting condition may not necessarily be met when the UE starts the timer for the first time after the event is met over the air.

An event reporting condition may be met when the UE starts the timer a 2nd time or even later.

Figure 1

For example, as we have illustrated in Figure 1, the event reporting condition is fulfilled only when the UE starts a 2nd timer at t2. In such a case, we think that there are two possible alternatives that RAN4 should consider when defining the reporting delay:

- Reporting delay should be defined to start from the beginning of the timer during which the report is actually triggered.

- Reporting delay should be defined to start from the time the event condition is met the first time till the UE sends the first PUCCH, where the total delay also includes windows where the reporting condition is not met.

If the reporting condition becomes met after restarting the timer for the beam to be reported, the following starting points for the reporting delay may be considered:

  - Option a: the reporting delay is the time from the start of a subsequent timer during which reporting condition is met until the first PUCCH transmission, or

  - Option b: the reporting delay is the time from when the event condition is met for the first time till the PUCCH transmission, which also includes windows during which the reporting condition was not fulfilled.

Option a may be more correct, but considering that it is not clear how the UE makes the decision about restarting the timer after a timer that does not trigger a report, the moment of starting the timer would not be known to TE or network. Hence, Option b would be more suitable for the specification as here the starting point of the delay is not UE specific. For this option, the windows during which the report is not triggered should be included in the reporting delay formula. We have given a text proposal how to capture this in our companion CR.

If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from when the event condition is met for the first time till the PUCCH transmission, which also includes windows during which the reporting condition was not fulfilled.

Discussion on timeRestrictionForChannelMeasurement

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Agreement:

- RAN4 define different separate for measurement period for the UE is configured or not for “timeRestrictionForChannelMeasurement”

- To check with RAN1 whether the parameter can be configured if RAN1 identify any issue

On the topic of measurement period in the measurement reporting delay the following was agreed in RAN4#115 meeting

As stated in the LS [2] to RAN1, the RAN4 understanding is that if timeRestrictionForChannelMeasurement is not configured, then M shall be 3 for current and new beam, while if timeRestrictionForChannelMeasurement is configured, then M shall be considered as 1 for both.

This understanding is for sure valid when both the current beam and new beam are a part of the same resource set wherein they would follow the same value of M (i.e., M=1 or M=3). For a UE configured with a UEIBM CSI report configuration, i.e., a CSI-ReportConfig that includes high layer parameter eventType, that happens when both current beam and new beam are port of newBeamResourceSet in CSI-ReportConfig.

On the other hand, such restriction has not been agreed in RAN1, i.e. it is not necessary that the current beam and the new beam are a part of the same resource set. In such a scenario, it is even possible that the current beam is part of a first resource set where timeRestrictionForChannelMeasurement is configured, while the new beam is part of a second resource set where timeRestrictionForChannelMeasurement is not configured. However, the current beam measurements for UEIBM event determination/evaluation do not need to follow the configuration of such first resource set because the configuration of such first resource set is followed only for the current beam measurements that are reported according to that configuration.

Hence, we propose that RAN4 clearly specifies the agreement that both the current and the new beam shall follow the same value of M, as indicated in our companion CR [3].

New beam and current beam may not be part of the same resource set.

RAN4 to clarify in the specification that the current beam and the new beam shall follow the same value of M.

Conclusion

In this paper, the following observations and proposals were made:

Observation 1: An event reporting condition may not necessarily be met when the UE starts the timer for the first time after the event is met over the air.

Observation 2: An event reporting condition may be met when the UE starts the timer a 2nd time or even later.

Observation 3: If the reporting condition becomes met after restarting the timer for the beam to be reported, the following starting points for the reporting delay may be considered:

a.	Option a: the reporting delay is the time from the start of a subsequent timer during which reporting condition is met until the first PUCCH transmission, or

b.	Option b: the reporting delay is the time from when the event condition is met for the first time till the PUCCH transmission, which also includes windows during which the reporting condition was not fulfilled.

Proposal 1: If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from when the event condition is met for the first time till the PUCCH transmission, which also includes windows during which the reporting condition was not fulfilled.

Observation 4: New beam and current beam may not be part of the same resource set.

Proposal 2: RAN4 to clarify in the specification that the current beam and the new beam shall follow the same value of M.

- References

- R4-2512130    WF on RRM requirements for NR_MIMO_Ph5_Part1

- R4-2511656    LS on event triggered L1-RSRP reporting

- R4-2514051    DraftCR for UEIBM reporting delay, Nokia