3GPP TSG RAN WG4 Meeting #116-bis	R4-2514080

Prague, Czech, 13 – 17 October, 2025

Agenda item:			6.17.2.2

Source:	vivo

Title:	On performance requirements for CSI-RS based L1 measurement

Document for:	Discussion

# Introduction

In the last RAN4 meeting, RAN4 completed RRM core requirements for Rel-19 LTM. The core requirements were specified for CSI-RS based L1 measurement, event triggered L1 measurement reporting and conditional LTM.

There were proposals on performance requirements for mobility enhancement on CSI-RS based L1 measurement, which were captured in the topic summary [1].

### CSI-RS based L1 measurement #### Issue 4-2-1: CSI-RS based L1 measurement accuracy Candidate options: - Option 1: CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell. (Nokia, Apple, OPPO, MTK, CMCC, vivo, HW, E///) - Option 1a: CATT - CSI-RS based L1-RSRP accuracy requirements defined in clauses 10.1.19.2 and 10.1.20.2 can be a baseline for CSI-RS based L1-RSRP requirements for neighbor cell. - RAN4 to define new clauses for CSI-RS based L1-RSRP requirements for neighbor cell in clauses 10.1.19D, 10.1.19E, 10.1.20A and 10.1.20B as R18 SSB based L1-RSRP requirements for neighbor cell. - Option 1b: Apple Accuracy requirements; Detail. Intra-frequency L1-RSRP accuracy requirements; 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy. 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Absolute Accuracy. - Option 1c: - Reuse legacy accuracy requirements of CSI-RS based L1 measurements for serving cell under the following conditions: - The bandwidth of CSI-RS is 48 PRBs and the density is 3. - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP. - Specify absolute and relative intra-frequency L1-RSRP accuracy requirements for FR1 and FR2 - Reuse the legacy L1 report mapping specified in clause 10.1.6 Recommended WF: - CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell when the following conditions are met: - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS. - The bandwidth of CSI-RS is 48 PRBs and the density is 3. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP. - Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell. - New requirements are defined in the following sections Accuracy requirements; Detail. Intra-frequency L1-RSRP accuracy requirements; 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy. 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Absolute Accuracy. #### Issue 4-2-2: test case list for CSI-RS based L1 measurement Candidate options - Option 1: Nokia - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1 - Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1 - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping - Option 2: CATT - RAN4 to define the test case for LTM CSI-RS based Intra-frequency L1-RSRP measurement in FR1 and FR2. - Option 3: Apple, OPPO, [vivo?] - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 - Option 4: MTK - CSI-RS based L1 RSRP measurement for neighbour cell in FR1 - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 - Option 5: CMCC - For CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested. - Option 6: HW - As the CSI-RS based L1-RSRP measurement procedure has been verified by event triggered reporting test case for CSI-RS L1-RSRP, it is no need to specify specific test for CSI-RS based L1-RSRP measurement. - In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. - FR1 and FR2 LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases for neighbour cell are to be specified. - Option 7: E/// Intra-f L1-RSRP measurement for LTM; - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells. - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells,. Inter-f L1-RSRP measurement without gap for LTM; - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells. - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated. Recommended WF: - Discuss the options. #### Issue 4-2-3: test configuration for CSI-RS based L1 measurement - BW Candidate options: - Option 1: Test configuration is set to have 48 PRBs within the active BWP. (Nokia) Recommended WF: - Agree on option 1. #### Issue 4-2-4: others Candidate options: - Proposal 1: Define test case(s) to verify early CSI acquisition if core requirements are specified. (vivo) - Proposal 1a: Define test case(s) to verify early CSI acquisition before the cell switch command, conditionally to core requirement definition. (Nokia) - Proposal 1b: Define test case(s) to verify early CSI acquisition after the cell switch command, conditionally to core requirement definition. (Nokia) - Proposal 2: Discuss whether there is need to consider configuration where CSI-acquisition resource(s) are outside of the active BWP. (Nokia) Recommended WF: - Discuss the proposals.

In this contribution, we further provide our views on performance requirements for CSI-RS based L1 measurements.

# Discussion

Accuracy requirements

In Rel-18, accuracy requirements for SSB-based L1 measurement for LTM were specified. since both intra-frequency and inter-frequency SSB-based L1 measurements are supported, accuracy requirements were specified for both cases.

For CSI-RS based L1 measurement, only intra-frequency measurement without gap for CSI-RS based L1 measurement is supported. In the previous RAN4 meeting, following agreement was reached.

Agreement: - RRM requirements (delay and accuracy) apply under the following condition: - At least 48 RBs of the CSI-RS configured for measurement is within the active BWP.

In legacy CSI-RS based measurement for serving cell, the accuracy was specified based on CSI-RS with 48 RBs and density 3. With same side conditions, the legacy accuracy requirements for CSI-RS based L1 measurements for serving cell can be reused.

Proposal 1: For CSI-RS based L1 measurement for LTM, reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3.

Test cases

Compared to SSB-based L1 measurements, the difference is that the measurement is based on CSI-RS and accuracy could be different. The PCell cell switch and PSCell cell switch procedure should be the same regardless of it is based on SSB-based L1 measurements or CSI-RS based L1 measurements.

In Rel-19, only intra-frequency CSI-RS based L1 measurement was supported in both FR1 and FR2. Therefore, test cases for intra-frequency measurement in both FR1 and FR2 should be defined.

For accuracy requirements which is under discussion, since it would the same as serving cell in terms of side condition and accuracy requirements, there would be not much value to define corresponding test cases. Compared to CSI-RS based L1 measurement for serving cell, the test configuration need to be set by adding neighbor cells. In addition, it is better to test delay requirements and accuracy requirements in one test case.

In FR2, it is up to UE capability whether SSB-based L1-RSRP measurement for candidate cell can be skipped for CSI-RS-based L1-RSRP measurement.

| 52-3 | Skip SSB based L1-RSRP measurement for candidate cell CSI-RS-based L1-RSRP measurement | - Indicates support for skipping SSB-based L1-RSRP during neighboring cell CSI-RS-based L1-RSRP measurement. - Indicates support for skipping SSB-based L1-RSRP during both neighboring cell and serving cell CSI-RS-based L1-RSRP measurement. |
| --- | --- | --- |

Depending on whether indicating this UE capability, there could be three different UE implementations. Not skipping SSB-based L1-RSRP measurement for both serving cell and neighbour cell should be considered as baseline. Test cases should be defined to verify corresponding requirements. If UE supports the UE capability, it only needs to verify the neighbour cell measurement skipping SSB-based L1-RSRP measurement.

Proposal 2: For CSI-RS based L1 measurement for LTM, test cases are introduced to verify intra-frequency measurement delay and accuracy requirements for both FR1 and FR2.

Proposal 3: RAN4 is to define one test to verify both delay and accuracy requirements.

Proposal 4: In FR2, tests are defined for not skipping SSB-based L1 measurement and skipping SSB-based L1 measurement for neighbour cell only, respectively.

## 2.3 Early CSI acquisition

In the last meeting, following conclusion on early CSI acquisition was made.

Agreement: - No consensus on defining new RAN4 requirements for early CSI acquisition. - Clarification on the applicability of the existing requirement is not precluded if consensus on the exact clarification can be reached. - It is RAN4 common understanding that for baseline early CSI acquisition, - If fine T/F tracking is included in cell switch delay, UE shall start CSI acquisition at least after UE finishes fine T/F tracking of target cell. - Otherwise, UE shall start CSI acquisition at least after Tprocessing. - No need to capture above agreement in RAN4 spec.

Since there is no clear requirements for early CSI acquisition, it would be difficult to define test case.

Proposal 5: No test case(s) to verify early CSI acquisition functionality.

# Summary

In this contribution, we further provided our views on performance requirements for Rel-19 LTM. Following proposals and observations are present.

Proposal 1: For CSI-RS based L1 measurement for LTM, reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3.

Proposal 2: For CSI-RS based L1 measurement for LTM, test cases are introduced to verify intra-frequency measurement delay and accuracy requirements for both FR1 and FR2.

Proposal 3: RAN4 is to define one test to verify both delay and accuracy requirements.

Proposal 4: In FR2, tests are defined for not skipping SSB-based L1 measurement and skipping SSB-based L1 measurement for neighbour cell only, respectively.

Proposal 5: No test case(s) to verify early CSI acquisition functionality.

- References

- R4-2509066	Topic summary for [116][223] NR_Mob_Ph4_Part1, Moderator (Apple)