3GPP TSG-RAN WG4 Meeting #116bis	R4-2514086

Prague, Czech, 13th – 17th October, 2025

Agenda item:		6.13.2.1

Source:	Nokia

Title:	On potentially new BS RF requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#116 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the potentially new BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the potentially new BS RF requirements for SBFD operation.

# Discussion

## In-channel adjacent subband leakage

The following was agreed in RAN4#116:

Agreement: - Absolute limit for in-channel adjacent subband leakage for DU/UD case - Reuse legacy ACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1) for bands: Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit BS category / BS class; ACLR absolute basic limit. Category A Wide Area BS; -13 dBm/MHz. Category B Wide Area BS; -15 dBm/MHz. Medium Range BS; -25 dBm/MHz. Local Area BS; -32 dBm/MHz. - The necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104 shall be studied in maintenance of Rel-19 SBFD WI

RAN4 agreed on the above in RAN#116. In RAN4#115 there was the following agreement regarding FR2:

Agreement - For absolute limit for in-channel adjacent subband leakage: - For FR2 BS, legacy ACLR absolute basic limit (Table 9.7.3.3-2 for FR2 in TS 38.104) is followed.

Table 9.7.3.3-2: BS type 2-O ACLR absolute limit

| BS class | ACLR absolute limit |
| --- | --- |
| Wide area BS | -13 dBm/MHz |
| Medium range BS | -20 dBm/MHz |
| Local area BS | -20 dBm/MHz |

There is still ongoing discussion regarding the necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104. That will be discussed in the maintenance part and we have addressed our view on not introducing additional relaxation. Therefore, as we now have agreements for the in-channel adjacent subband leakage agreed for the core part for both FR1 and FR2, these should now be implemented in TS 38.141-1 and TS 38.141-2.

Agreed requirements for in-channel adjacent subband leakage to be implemented in TS 38.141-1 and TS 38.141-2.

RAN4 not to introduce additional relaxation for the in-channel adjacent subband leakage for band n104.

## In-channel adjacent subband blocking

The following was agreed in RAN4#116:

Agreement: - Adopt the following table as the requirement for both single carrier and carrier aggregation operations. Table: In-channel adjacent subband blocking requirement for SBFD-capable BS SBFD UL subband bandwidth (MHz); Wanted signal mean power (dBm) (NOTE 1); Interfering signal mean power (dBm) (NOTE 5); SBFD UD/UD or DUD case; Interfering signal centre frequency/frequencies; Type of interfering signal(s) （NOTE 2, 3, 4）. 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90; PREFSENS + 6 dB; Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34; DU or UD; FC,SBFD,DL; CP-OFDM NR signal 30 kHz SCS, BW = X (in the unit of RB). DUD; FC,SBFD,DL,1 FC,SBFD,DL,2; CP-OFDM NR signal 30 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB). NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver..

As RAN4 agreed on the core requirement for in-channel adjacent subband blocking requirement for SBFD-capable FR1 BS in RAN4#116, it should be also implemented to TS 38.141-1/2.

Agreed requirements for SBFD-capable FR1 BS in-channel adjacent subband blocking to be implemented in TS 38.141-1/2.

Regarding the in-channel adjacent subband blocking for FR2-1, the following was agreed in RAN4#116:

##### Issue 4-3-2: Interfering signal level for FR2-1 Agreement: - On the level of in-channel adjacent subband blocking requirement, - For FR2-1 WA BS: - the way to define the requirement is similar to FR1, i.e., by considering coupling loss between co-site sectors: - Absolute value is -52dBm ##### Issue 4-3-3: Assumption for coupling loss Agreement: - Adopt the following assumption in the specification, and the detailed wording can be further refined: - For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. - For FR2-1 WA BS, in-channel adjacent subband blocking requirement assumes 87 dBc coupling loss between interfering BS transmitter and SBFD BS receiver.

Similarly, as for FR1, these agreements need to be captured in TS 38.141-2. The absolute value for FR2-1 WA BS was agreed to be -52dBm and accompanied with a note that 87 dBc coupling loss was assumed.

Agreed requirements for SBFD-capable FR2-1 BS in-channel adjacent subband blocking to be implemented in TS 38.141-2.

# Conclusion

In this contribution, our views on the potentially new BS RF requirements for SBFD operation are presented. The following observations were listed:

- Agreed requirements for in-channel adjacent subband leakage to be implemented in TS 38.141-1 and TS 38.141-2.

- Agreed requirements for SBFD-capable FR1 BS in-channel adjacent subband blocking to be implemented in TS 38.141-1/2.

- Agreed requirements for SBFD-capable FR2-1 BS in-channel adjacent subband blocking to be implemented in TS 38.141-2.

- RAN4 not to introduce additional relaxation for the in-channel adjacent subband leakage for band n104.

# References

- R4-2512665 “Way Forward for [116][306] NR_duplex_evo_General”, RAN4#116, Bengaluru, India, 25th-29th August, 2025