3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2514115

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	1Tx PC1.5 for NB-IoT-NTN based on Release 19 PC1 work

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.6.3	IoT-NTN PC1.5 single Tx	[TBD]

Document for:	Approval

# 1	Background

In RAN#109, a new Release 20 WI [1] on NR NTN and IoT NTN further enhancement was agreed on that reconducts the development of the PC1.5 1Tx NB-IoT-NTN requirement that could not be finalized in Release 19. In this contribution, we discuss how this can be done based on the PC1 Release 19 work.

# Discussion

## 2.1 HPUE objectives in R20 new enhanced NR/IoT NTN WI

The new Release 20 WI on enhanced requirements for NR NTN and IoT NTN Phase 2 [1] re-conducts the work on PC1.5 1Tx HPUE for NB-IoT-NTN. The related objectives are copied below in italics.

HPUE for NTN Core Part

- Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types

  - Example bands are Band 256 and 255

  - Necessary Tx requirements, at least including

    - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

  - Necessary Rx requirements, at least including

    - Specify the RSD requirements on the example bands, if necessary

  - Specify the necessary signaling to support the above objectives

- The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

In this contribution, we discuss how the 1Tx NB-IoT-NTN 1Tx PC1.5 UL requirements can be directly derived from the Release 19 work on PC1 that also had the same Band n255/n256 example bands.

## 2.2 Release 19 PC1 NB-IoT-NTN assumptions and UL requirements

In release 19 we provided inputs to the PC1 NB-IoT-NTN MPR in [1]. This input was based on evaluating both a PC2 NR PA (using PC2 NR calibration) and a mid-band GSM PA. This input was used in the draft CR [3] to specify PC1 MPR and n255 and n256 A-MPR as follows:

Table 6.2B.2-4: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 2 dB | 0 dB | ≤ 2 dB | ≤ 3.5 dB | 0 dB | ≤ 3.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table 6.2B.2-5: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 2 dB | ≤ 1 dB |  | ≤ 2 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 2.5dB |  | ≤ 2.5dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 4 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Table 6.2B.3-2: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1 and PC2

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC1 | PC2 |
|  |  |  | ≤ 10 | ≤ 5.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |

Additionally, the NB-IoT-NTN PCmax equations accommodate boosting for PC2.

## 2.3 PC1.5 NB-IoT-NTN UL requirements

Since it is an obvious option for hardware reuse. especially for handheld devices, the PC1.5 1Tx NB-IoT-NTN UL requirement can also be derived from a PC2 NR PA implementation similar to PC1 in Release 19.

Proposal for UL: MPR and A-MPR for PC1.5 1Tx NB-IoT-NTN are based on Release 19 PC1 NPR and A-MPR since:

- The same PC2 NR PA with the PC2 NR calibration can implement both power classes

- The MPR limitation is only the NB-IoT mask and the NS_24 and NS_02N that are in absolute power

- In this case, the MPR and A-MPR can be reduced by 2dB as this is the power difference between PC1 and PC1.5

  - The 2dB reduction to MPR/A-MPR can be further discussed to account for different post-PA losses and potential thermal/DC power limitations in handsets.

  - With 2dB MPR reduction, 2dB boosting is feasible for the center single-tone allocations and 1dB boosting is feasible for the center 3-tone allocation.

    - The R19 PC2 boosting framework can be reused for the power boosting requirement for PC1.5.

    - Optional power boosting is specified for PC1.5 reusing the UE capability [powerBoosting-NBIotNTN-r19] and the IE [powerBoostNBIotNTN-r19]

    - Which single-tone and 3-tone allocations can be boosted may be further verified.

- As a starting point, the following MPR and A-MPR tables can be used for PC1.5 for further verification:

Table 1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 0 dB | 02 dB | ≤ 0 dB | ≤ 1.5 dB | 02 dB | ≤ 1.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. Note 2: For single-tone positions 2 to 9 for 15kHz SCS and 6 to 41 for 3.75 kHz, 0dB MPR also applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |  |  |

Table 2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0 dB | ≤ 02 dB |  | ≤ 0 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 0.5dB |  | ≤ 0.5dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. Note 2: For 3-tone positions 3-5 and 6-8, 0dB MPR also applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |

Table 3: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |
| NS_24 | 6.5B.4.4.3 | 256 | ≤ 8 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |

- To enable power boosting for PC1.5, the following modification in yellow are needed to the clause 6.2B.4 in TS 36.102.

For category NB1 and NB2 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 [7] shall apply except for:

The definition of PCMAX_L,c and PCMAX_H,c which would be substituted by:

	PCMAX_L,c = MIN { PEMAX,c ,  PPowerClass – MAX(MPRc + A-MPRc, P-MPRc) + ΔPNBBoost}

 PCMAX_H,c = MIN { PEMAX,c,  PPowerClass + ΔPNBBoost }

, wherein

-	PPowerClass is the maximum UE power specified in Table 6.2B.1-1 without taking into account the tolerance specified in the Table 6.2B.1-1

-	ΔPNBBoost is defined as

2dB for single tone positions:

    - 3 to 8 for 15kHz SCS and 8 to 39 for 3.75kHz for PC2

    - 2 to 9 for 15kHz SCS and 6 to 41 for 3.75kHz for PC1.5

1dB for:

    - Other single tone positions and all multi-tone cases for PC2

    - 3 tone allocations 3-5 and 6-8 for PC1.5,

when all of the following conditions are met:

if the UE indicates support for UE capability [powerBoosting-NBIotNTN-r19]

if IE [powerBoostNBIotNTN-r19] is set to 1

if UE indicates support for power class 2 or power class 1.5.

## 2.4 RSD for PC1.5 NB-IoT-NTN

In the new WI [1], there are an objective on the RX requirements:

  - Necessary Rx requirements, at least including

    - Specify the RSD requirements on the example bands, if necessary

It seems confusing since NB-IoT is HD-FDD and thus there is no impact to the DL by the UL, meaning that the PC3 REFSENS is valid for any power class.

Proposal on DL: the need for PC1.5 RSD requirenent for NB-IoT band NTN need to be clarified.

# Conclusion

In this contribution, we provide our preliminary views on the new Release 20 PC1.5 1Tx NB-IoT-NTN power class based on the PC1 Release 19 work.

Proposal for UL: MPR and A-MPR for PC1.5 1Tx NB-IoT-NTN are based on Release 19 PC1 NPR and A-MPR since:

- The same PC2 NR PA with the PC2 NR calibration can implement both power classes

- The MPR limitation is only the NB-IoT mask and the NS_24 and NS_02N that are in absolute power

- In this case, the MPR and A-MPR can be reduced by 2dB as this is the power difference between PC1 and PC1.5

  - The 2dB reduction to MPR/A-MPR can be further discussed to account for different post-PA losses and potential thermal/DC power limitations in handsets.

  - With 2dB MPR reduction, 2dB boosting is feasible for the center single-tone allocations and 1dB boosting is feasible for the center 3-tone allocation.

    - The R19 PC2 boosting framework can be reused for the power boosting requirement for PC1.5.

    - Optional power boosting is specified for PC1.5 reusing the UE capability [powerBoosting-NBIotNTN-r19] and the IE [powerBoostNBIotNTN-r19]

    - Which single-tone and 3-tone allocations can be boosted may be further verified.

- As a starting point, the following MPR and A-MPR tables can be used for PC1.5 for further verification:

Table 1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 0 dB | 02 dB | ≤ 0 dB | ≤ 1.5 dB | 02 dB | ≤ 1.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. Note 2: For single-tone positions 2 to 9 for 15kHz SCS and 6 to 41 for 3.75 kHz, 0dB MPR also applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |  |  |

Table 2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0 dB | ≤ 02 dB |  | ≤ 0 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 0.5dB |  | ≤ 0.5dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. Note 2: For 3-tone positions 3-5 and 6-8, 0dB MPR also applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |

Table 3: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |
| NS_24 | 6.5B.4.4.3 | 256 | ≤ 8 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |

- To enable power boosting for PC1.5, the following modification in yellow are needed to the clause 6.2B.4 in TS 36.102.

For category NB1 and NB2 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 [7] shall apply except for:

The definition of PCMAX_L,c and PCMAX_H,c which would be substituted by:

	PCMAX_L,c = MIN { PEMAX,c ,  PPowerClass – MAX(MPRc + A-MPRc, P-MPRc) + ΔPNBBoost}

 PCMAX_H,c = MIN { PEMAX,c,  PPowerClass + ΔPNBBoost }

, wherein

-	PPowerClass is the maximum UE power specified in Table 6.2B.1-1 without taking into account the tolerance specified in the Table 6.2B.1-1

-	ΔPNBBoost is defined as

2dB for single tone positions:

    - 3 to 8 for 15kHz SCS and 8 to 39 for 3.75kHz for PC2

    - 2 to 9 for 15kHz SCS and 6 to 41 for 3.75kHz for PC1.5

1dB for:

    - Other single tone positions and all multi-tone cases for PC2

    - 3 tone allocations 3-5 and 6-8 for PC1.5,

when all of the following conditions are met:

if the UE indicates support for UE capability [powerBoosting-NBIotNTN-r19]

if IE [powerBoostNBIotNTN-r19] is set to 1

if UE indicates support for power class 2 or power class 1.5.

Proposal on DL: the need for PC1.5 RSD requirenent for NB-IoT band NTN need to be clarified.

# References

[1] RP-252962 New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated), RAN#109

[2] R4-2509243 MPR input for PC1 NB-IoT-NTN, Skyworks Solutions Inc., RAN4#116

[3] R4-2511903 draft CR for Iot-NTN MOP related requirements, vivo, RAN4#116