3GPP TSG-RAN WG4 Meeting #116bis                                        R4-2514133 Prague, Czech Republic, October 13 - 17, 2025

Source: 	Huawei, HiSilicon

Title: 	On BS conformance for legacy requirements for SBFD

Agenda Item:	6.13.2.2

Document for:	Approval

Introduction

During RAN4#116, more analysis on SBFD conformance testing are presented. Discussion progress can be found in WF [1].

- Issue: BS conformance testing - manufacturer declaration - Way forward - FFS in next meeting - All the declarations in the WF is needed - In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Tx requirements shall be updated to include the relevant clauses for SBFD capable BS. - It is necessary to add the definitions of output power requirements for SBFD capable BS to TS 38.141-1 and TS 38.141-2. - It is necessary to add the definition of total power dynamic range requirement for SBFD capable BS to TS 38.141-1 and TS 38.141-2. - The definitions of EVM and averaged EVM for SBFD capable BS need to be added to TS 38.141-1 and TS 38.141-2. - The OTA measurement chamber setup, calibration, and measurement procedures defined for NR BS can serve as the baseline for testing Tx requirements of SBFD capable BS. - In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS. - The OTA measurement chamber setup and calibration (and measurement) procedures of Rx requirements testing for SBFD capable BS are different from those of NR systems, and these differences must be added to TS 38.141-2. - Issue: BS conformance testing - RF channels - Way forward - FFS in next meeting - New test models need to be set for SBFD BS. - Guarantee SBFD carrier is configured at the left-most, right-most and middle of the operating band - Issue: BS conformance testing - Test model - Way forward - FFS in next meeting - New parameters such as guard-band size should be described clearly in the spec. - The test model for SBFD DL subbband needs to be scalable. - New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband. - Signal statistics may change for SBFD test models and need to be investigated. The impact of DL subband on signal statistics needs to be analyzed. - In TS 38.141-1 and TS 38.141-2, the test models specified for NR BS can be reused for the existing Tx requirements testing of SBFD capable BS, but with the different allocated channel in tests. - A new test model which transmitter and the receiver operate simultaneously must be set up for Rx requirement testing of SBFD capable BS. - Test requirements for each Rx requirement of NR BS in TS 38.141-1 and TS 38.141-2 can serve as the baseline for SBFD capable BS. - In test requirements, NR channel bandwidth needs to be updated to the UL subband bandwidth of the SBFD capable BS. - Any test requirements for SBFD capable BS that differ from NR BS test requirements must be specified for SBFD capable BS in TS 38.141-1 and TS 38.141-2. - Issue: BS conformance testing - Test signal - Way forward - FFS in next meeting - define the test signal for SBFD conformance testing in the following way: - Table 4.7.2-1: Signal to be used to build NR SBFD TCs Operating Band characteristics; Band n39; Other FR1 bands. TC signal; BWchannel; 20 MHz (Note 1); 40 MHz (Note 1). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing. NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used.. - Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O Operating band characteristics; FDL_high – FDL_low ≤ 3250 MHz. TC signal; BWchannel; 100 MHz (Note 1, Note 2). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing declared per operating band (D.7). NOTE 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used.. - Issue: BS conformance testing - Beam configuration - Way forward - FFS in next meeting - When test Tx requirements of SBFD Capable BS, receivers can be turned off. - In the test of Tx requirements of SBFD Capable BS, the Tx antennas beam is test output beam. - When test OTA Rx requirements of SBFD capable BS, the transmitters must be turned on. - For OTA Rx requirements testing of SBFD capable BS, the beam configurations of Tx and Rx antennas need to consider the isolation requirement between them. - For OTA Rx requirements testing of SBFD capable BS, it is necessary to consider the directional correspondence between the test input antenna beam and the Rx antenna beam.

In this contribution, we would like to share our views regarding this topic.

Discussion

Test signal and test configuration

According to the CR [2], there are specific limitation on minimum BS channel bandwidth introduced for SBFD.

Note that the existing test signal used to build test configuration is as follow, where smaller channel bandwidth is adopted.

Per our understanding, a simple way to address this is to inherit the flexibility that has already been guaranteed for the channel bandwidth and subband size under SBFD operation i.e., replying on manufacture declaration.

Proposal 1: The bandwidth of the test signal for SBFD is up to manufacture declaration.

Test model

The existing test model and configuration are based on the maximum RB numbers of the whole carrier.

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

But for SBFD, it is clear that the transmission will only happen within DL subband. Therefore, we think it should be updated as NRB, SBFD_DL for DU/UD case and NRB, SBFD_DL_1, NRB, SBFD_DL_2 for DUD case, respectively.

Proposal 2: For SBFD, the NRB in the test model should be updated as NRB, SBFD_DL for DU/UD case and NRB, SBFD_DL_1, NRB, SBFD_DL_2 for DUD case.

Test procedure

Unlike transient period between SBFD and non-SBFD and those two new In-channel requirements, other RF requirements for SBFD symbols are basically the same with non-SBFD symbols especially from test procedure perspective. Thus we think the legacy test procedure can be reused unless any issue can be identified.

Proposal 3: For SBFD requirements other than transient period between SBFD and non-SBFD, In-channel adjacent subband leakage and blocking, the existing test procedure can be reused for the SBFD symbol.

MU

Although most of the test setups will be reused between SBFD and non-SBFD symbols, whether there could be any impact on MU caused by simultaneous operating between Tx and Rx can be further considered. Otherwise, the existing MU can be reused for SBFD testing requirements.

Proposal 4: Pending on analysis for the impact on MU potentially caused by simultaneous operating between Tx and Rx to see whether existing MU can be reused for SBFD testing requirements.

Conclusion

In this contribution we discussed on the conformance testing for legacy requirements for SBFD. We have the following proposals:

Proposal 1: The bandwidth of the test signal for SBFD is up to manufacture declaration.

Proposal 2: For SBFD, the NRB in the test model should be updated as NRB, SBFD_DL for DU/UD case and NRB, SBFD_DL_1, NRB, SBFD_DL_2 for DUD case.

Proposal 3: For SBFD requirements other than transient period between SBFD and non-SBFD, In-channel adjacent subband leakage and blocking, the existing test procedure can be reused for the SBFD symbol.

Proposal 4: Pending on analysis for the impact on MU potentially caused by simultaneous operating between Tx and Rx to see whether existing MU can be reused for SBFD testing requirements.

- References

- R4-2512680, “Way Forward for [116][307] NR_duplex_evo_BSRF”, Huawei, HiSilicon, Samsung, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.

- R4-2511255, “CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS”, Samsung, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.