# 3GPP TSG RAN WG4 Meeting #116	bis 														R4-2514187

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.13.3.2

Source: 	Ericsson

Title:	Discussion on RRM performance requirements for SBFD operation

Document for:	Discussion

# Introduction

In this paper we provide our view on RRM performance requirement within related WID [1] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

#### Issue 3-4-1: General test setup

The SBFD core requirements have been specified both consider FR1 and FR2.

We think for FR1, either the 15KHz or the 30KHz SCS can be used, the bandwidth needs to be test for the widest supported for FR1.

For FR2 , due the accuracy requirement have quite large deviation between case1 and case 2 for L1-SRS-RSRP measurement when SCS go above 60KHz, we think it is good to verify both 60KHz and 120KHz.

- Proposal 1: For FR1, either 15KHz or 30KHz SCS can be used for 100MHz bandwidth with TDD mode. For FR2 both 60KHz and 120KHz SCS with 100MHz bandwidth with TDD mode.

We think for the 1st release of SBFD, the test case shall verify both TDD DL/UL configuration with TDD pattern DDDFU, while the SBFD configuration needs to consider both the DU and DUD configuration.

For time domain configuration of the SBFD slot, we think it is good to have DL symbol start and have the UL symbol at the end of the slot to verify the UE TX/RX switching correctly.

From frequency domain, the 100MHz bandwidth needs to be verified for both DU and DUD configuration.

- Proposal 2: SBFD configuration shall test the TDD pattern with DDDFU, both DU and DUD configuration are needed to be configurated with 100MHz bandwidth.

- Proposal 3: The configuration of the SBFD slot needs to follow the DXXXU pattern to verify the UE TX/RX switching behaviour.

#### Issue 3-3-2: Test case list

For all Rel-19 SBFD specified core requirements, we think it is very important to verify the CSI-RS based L1 measurement delay and accuracy since it is the measurement that will be impacted by the SBFD operation.

The test case shall include CSI-RS based L1-RSRP, L1-SINR and RLM/BFD/CBD .

For the L1-SINR, we think the CSI-RS can be both the CMR and IMR.

- Proposal 4: RAN4 to specify test case for CSI-RS based L1-RSRP both for measurement delay and accuracy.

- Proposal 5: RAN4 to specify test case for CSI-RS based L1-SINR with CSI-RS being configured as both CMR and IMR.

- Proposal 6: RAN4 to specify test case for CSI-RS based RLM/BFD/CBD.

Test methodology and configuration

The SBFD operation impact on the RRM measurement requirement mostly reflected on the CSI-RS based L1-RSRP/SINR measurement, the RLM/BFD/CBD measurement and the L3 RRM measurement due to the fundamental difference between a SBFD symbol and non-SBFD symbol.

Currently the impact on CSI-RS based L1-RSRP/SINR measurement requirement is being discussed for the core requirement, however the measurement requirement impact is most likely on the measurement accuracy and measurement period due to the minimum number of CSI-RS resource will be impacted due to punctuation of CSI-RS resources. Another impact is that the UE will perform the CSI-RS based measurement during SBFD operation based on network indication of valid symbol type.

- Proposal 7: RAN4 further discuss how to specify CSI-RS based L1-RSRP/SINR measurement test case due to the CSI-RS resource punctuation and the valid symbol type indication.

Regarding the RLM/BFD/CBD measurement, the impact due to CSI-RS punctuation is quite similar as the L1-RSRP/SINR, the difference is that the valid symbol type indication is not applicable to the RLM/BFD/CBD measurement.

The legacy RLM/BFD test case have been specified for both SA and DC scenario, for SBFD the working assumption for Rel-19 is considering SBFD operation for single cell.

The legacy test procedure for Out of sync and In sync for RLM/BFD can be reused, the details for the test requirement for how long the time duration should be and how UE should behave due to SBFD impact on CSI-RS resources needs to be further discussed when the core requirement is stable.

- Proposal 8: RAN4 further discuss how to specify CSI-RS based RLM/BFD/CBD test case and test requirement due to the CSI-RS resource punctuation for SBFD symbols.

Due to the SBFD configured for Scell will follow the same legacy requirements, we do not see the motivation to verify multi carrier or multi cell scenario.

- Proposal 9: For 1st release of SBFD, the test case shall focus on single cell operation.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: For FR1, either 15KHz or 30KHz SCS can be used for 100MHz bandwidth with TDD mode. For FR2 both 60KHz and 120KHz SCS with 100MHz bandwidth with TDD mode.

	Proposal 2: SBFD configuration shall test the TDD pattern with DDDFU, both DU and DUD configuration are needed to be configurated with 100MHz bandwidth.

	Proposal 3: The configuration of the SBFD slot needs to follow the DXXXU pattern to verify the UE TX/RX switching behaviour.

	Proposal 4: RAN4 to specify test case for CSI-RS based L1-RSRP both for measurement delay and accuracy.

	Proposal 5: RAN4 to specify test case for CSI-RS based L1-SINR with CSI-RS being configured as both CMR and IMR.

	Proposal 6: RAN4 to specify test case for CSI-RS based RLM/BFD/CBD.

	Proposal 7: RAN4 further discuss how to specify CSI-RS based L1-RSRP/SINR measurement test case due to the CSI-RS resource punctuation and the valid symbol type indication.

	Proposal 8: RAN4 further discuss how to specify CSI-RS based RLM/BFD/CBD test case and test requirement due to the CSI-RS resource punctuation for SBFD symbols.

	Proposal 9: For 1st release of SBFD, the test case shall focus on single cell operation.

# References

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024

- R4-2512133 ‘WF on RRM requirements for NR_Duplex_evo’, Huawei, August,2025