3GPP TSG-RAN4 Meeting #116bis	R4-2514259

# Prague, CZ, 13th - 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 19.2.0 (Draft) |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR MTTD requirements for UL only TRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correction on MTTD requirements for UL only TRP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct RAN2 UE capability is now used in the MTTD requirements for UL only TRP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear to which UE capability the requirements applies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

### 7.5.7	Minimum requirements for multi-TRP

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 shall be capable of handling at least a relative transmission timing difference between slot timing of a pair of TAGs configured on the same carrier as shown in tables 7.5.7-1, if the UE does not indicate support of rxTimingDiff-r18.

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 shall be capable of handling at least a relative transmission timing difference between slot timing of the pair of TAGs configured on the same carrier as shown in table 7.5.7-2, provided that the UE indicates that it is capable of rxTimingDiff-r18.

A UE supporting twoTA-InterCellBM-r19 or twoTA-IntraCellBM-r19 shall be capable of handling at least a relative transmission timing difference between slot timing of a pair of TAGs configured on the same carrier with single DCI as shown in Table 7.5.7-1, if the UE is configured with PL_offset or if the UE does not indicate support of rxTimingDiff-r18, and in Table 7.5.7-2, if the UE indicates that it is capable of rxTimingDiff-r18 and PL_offset is not configured.

Table 7.5.7-1: Maximum uplink transmission timing difference requirement for multi-TRP for UE not supporting rxTimingDiff-r18

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | CP lengthnote 1+1.6us |
| FR2-1 | CP lengthnote 1+0.5us |
| NOTE 1: CP length of the maximum SCS on the carrier. NOTE 2: This requirement applies to the UE capable of [STxMP]. |  |

Table 7.5.7-2: Maximum uplink transmission timing difference requirement for multi-TRP for UE supporting rxTimingDiff-r18

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 |

# <End of Change #1>