3GPP TSG-RAN WG4 Meeting#116bis                                R4-2514267

Prague, CZ, 13th – 17th, Oct. 2025

Agenda item:	6.14.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on conformance testing for Rel-19 Ambient-IoT BS and CW

Document for:	Approval

Introduction

In the last RAN4 meeting, the core requirement for A-IoT BS and CW is declared as completed. In this contribution, we would like to share our initial understanding on the conformance testing for A-IoT BS and CW node.

Discussion

## 2.1. Test signal and Test model

Regarding the test signal for A-IoT BS conformance testing, since we specified 4 different carrier bandwidth varying from 1PRB to 4 PRB, we propose to consider narrowest carrier bandwidth for the conformance testing following the similar logic as 5G NR BS conformance testing with typical narrowest bandwidth.

Proposal 1: For test signal for A-IoT BS conformance testing, propose to use 200kHz as default carrier bandwidth for conformance testing otherwise the supported narrow bandwidth should be used for the conformance testing.

Another important issues is to design the test model for the A-IoT BS conformance testing. In the following Figure 2.1-1/2, we provide the illustration of baseband signal generation and MAC PDU for CFA message.

Figure 2.1-1. Illustration of baseband signal generation for R2D

Figure 2.1-2. Illustration of R2D MAC PDU message for CFA

Table B.2.1-1: Fixed Reference Channels for reference sensitivity level (OOK) [extracted from TS 38.191]

| Reference channel | Configuration | A-FR1-B1-1 | A-FR1-B1-2 | A-FR1-B1-3 | A-FR1-B1-4 |
| --- | --- | --- | --- | --- | --- |
| SIP | SCS | 15 | 15 | 15 | 15 |
|  | PRB | 1 | 2 | 3 | 4 |
|  | Bit length | 8 | 8 | 8 | 8 |
|  | M_SIP | 4 | 4 | 4 | 4 |
|  | OFDM | 2 | 2 | 2 | 2 |
| CAP | Bit length | 4 | 4 | 4 | 4 |
|  | M | 2 | 2 | 2 | 2 |
|  | OFDM | 2 | 2 | 2 | 2 |
| PRDCH | TBS | 96 | 96 | 96 | 96 |
|  | CRC | 16 | 16 | 16 | 16 |
|  | Line encoding |  |  |  |  |
|  | M | 2 | 2 | 2 | 2 |
| postamble | Bit length | 4 | 4 | 4 | 4 |
|  | M | 6 | 12 | 2 | 2 |
|  | chip number except for SIP,padding | 228 | 228 | 228 | 228 |
|  | Padding | 6 | 12 | 2 | 2 |

Proposal 2: For test model for A-IoT BS conformance testing, propose to use the above configuration for the conformance testing.

## 2.2. Test configuration

Similar as the standardization of other new RAT, RAN4 need to discuss the test configuration for A-IoT BS. As shown in Figure 2.2-1, there are 8 different test configuration specified in past for the NR BS. However for A-IoT BS, we could only consider the single carrier operation without any multiple-carrier or multi-band operation due to the limited device1 capability which cannot distinguish the signals from multiple-carrier or multi-band transmission.

- Figure 2.2-1. Illustration of the existing test configuration from TS 38.141-1

Proposal 3: For test configuration for A-IoT BS conformance testing, propose to consider the single carrier operation only for A-IoT BS conformance testing.

## 2.3. Test procedure

Regarding the test procedures for A-IoT BS and CW node, since most of requirement items are aligned with the legacy BS just with different RF requirement for it except for REFSENS requirement;

Proposal 4: For test procedure for A-IoT BS conformance testing, propose to use the existing test procedure of NR BS type 1-C as baseline with the necessary modification except for REFSENS requirement;

Proposal 5: For the A-IoT BS REFSENS requirement, propose to add the CW signal as input for test procedure.

## 2.4. MU for A-IoT BS/ CW

Regarding the MU and test tolerance requirement for A-IoT BS and CW node, we propose to apply the same MU/Test tolerance from less than 3GHz in TS 38.141 unless any special issues should be taken into account.

Proposal 6: For MU and est tolerance for A-IoT BS and CW node, propose to apply the same MU/TT of BS type 1-C from less than 3GHz in TS 38.141 as baseline;

## 2.5. Test setup for A-IoT BS and CW

Regarding the test setup for A-IoT BS and CW node, we propose to apply the existing test setup for BS type 1-C for it except the REFSENS conformance testing as shown in Figure 2.5-1.

- Figure 2.5-1. Illustration of the test setup for A-IoT BS REFSENS requirement

Proposal 7: For test setup for A-IoT BS and CW node, propose to apply the existing test setup for BS type 1-C for it except the REFSENS conformance testing as shown in Figure 2.5-1.

Conclusions

In this contribution, we want to share some initial views on Rel-19 A-IoT/CW conformance testing and proposals/observations are made as following:

Proposal 1: For test signal for A-IoT BS conformance testing, propose to use 200kHz as default carrier bandwidth for conformance testing otherwise the supported narrow bandwidth should be used for the conformance testing.

Proposal 2: For test model for A-IoT BS conformance testing, propose to use the above configuration for the conformance testing.

| Reference channel | Configuration | A-FR1-B1-1 | A-FR1-B1-2 | A-FR1-B1-3 | A-FR1-B1-4 |
| --- | --- | --- | --- | --- | --- |
| SIP | SCS | 15 | 15 | 15 | 15 |
|  | PRB | 1 | 2 | 3 | 4 |
|  | Bit length | 8 | 8 | 8 | 8 |
|  | M_SIP | 4 | 4 | 4 | 4 |
|  | OFDM | 2 | 2 | 2 | 2 |
| CAP | Bit length | 4 | 4 | 4 | 4 |
|  | M | 2 | 2 | 2 | 2 |
|  | OFDM | 2 | 2 | 2 | 2 |
| PRDCH | TBS | 96 | 96 | 96 | 96 |
|  | CRC | 16 | 16 | 16 | 16 |
|  | Line encoding |  |  |  |  |
|  | M | 2 | 2 | 2 | 2 |
| postamble | Bit length | 4 | 4 | 4 | 4 |
|  | M | 6 | 12 | 2 | 2 |
|  | chip number except for SIP,padding | 228 | 228 | 228 | 228 |
|  | Padding | 6 | 12 | 2 | 2 |

Proposal 3: For test configuration for A-IoT BS conformance testing, propose to consider the single carrier operation only for A-IoT BS conformance testing.

Proposal 4: For test procedure for A-IoT BS conformance testing, propose to use the existing test procedure of NR BS type 1-C as baseline with the necessary modification except for REFSENS requirement;

Proposal 5: For the A-IoT BS REFSENS requirement, propose to add the CW signal as input for test procedure.

Proposal 6: For MU and est tolerance for A-IoT BS and CW node, propose to apply the same MU/TT of BS type 1-C from less than 3GHz in TS 38.141 as baseline;

Proposal 7: For test setup for A-IoT BS and CW node, propose to apply the existing test setup for BS type 1-C for it except the REFSENS conformance testing as shown in Figure 2.5-1.

References

- RP-243326, New WID on Rel-19 Ambient IoT, Approved.

- R2-2506559, A-IoT MAC layer specification