3GPP TSG-RAN WG4 Meeting #116bis	R4-2514371

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	7.8.3

Source:            	Google

Title:                	Discussion on enhancement of low band carrier aggregation via switching

Document for:	Approval

# Introduction

In RAN#109, a new WID on enhancement of low NR band carrier aggregation (LB CA) via switching was approved as one of Release 20 work items [1]. The objective in the WID is to specify the requirements for the LB CA via switching between Case#1, i.e., Tx/Rx on FDD carrier 1 and no RX on SDL carrier 2, and Case#3, i.e., Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1, which is shown as follows. According to the operator request, the CA_n12-n29A and CA_n28A-n67A are proposed as the example band combinations in this WID. In this contribution, we will focus on the RAN4 RF requirements part in the following discussion.

### 4.1 Objective of SI or Core part WI or Testing part WI The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

# Discussion

In Rel-20 LBCA_sw WID [1], two challenging example band combinations, i.e., CA_n12A-n29A and CA_n28A-n67A, for this feature are provided. The justification is that to enabling dual downlink is beneficial to improve downlink performance for low band CA combinations. Compared to Rel-19 LB_CA_sw WID, Case#3 is the new scenario which needs further discussion. Different from Rel-19, the switching scenario is the switching from Case#1 Tx/RX on FDD carrier 1 and no Rx on SDL carrier 2 to Case#3 RX on FDD carrier 1 and Rx on SDL carrier 2. In other words, Case#3 needs to keep Rx work on FDD carrier 1 but Case#2 does not need to do that.

Table 4-1: Operator-provided scenarios for NR low band aggregation via switching [3]

| Configuration | Illustration | Notes |
| --- | --- | --- |
| CA_n12A-n29A |  | (3) |
| CA_n28A-n67A |  | (4), (5) |
| NOTE 1: CA_n5A-n29A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 2: Fractional bandwidth of a single antenna to support this combination is 22% and represents a practical implementation challenge NOTE 3: There is no incumbent narrowband service in the band gap between n29 and n12 DL (728~729 MHz) in Canada NOTE 4: In Europe band n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz DL NOTE 5: 3GPP will introduce requirements for band n28 assuming full band duplexer architecture in Rel-19 NOTE 6: CA_n29A-n71A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 7: Fractional bandwidth of a single antenna to support this combination is 16.5% and represents a practical implementation challenge |  |  |

Assuming antenna implementation is usually shared among low bands for the current handheld UE design, the switching delay can be dominated by the RF hardware switching time, which may contain LO retuning time, RFFE SPxT switching time, LNA switching time and PA ON/FF time. In our understanding, the LO retuning time would be hundred microseconds (us) and RFFE SPxT switching time would be around several microseconds. As for LNA switching time and PA ON/OFF time would be around ~10 us. Hence, the total switching delay for LB CA via switching may depends on different implementation from antenna and RF hardware perspective.

For Case#1 to Case#3, n12/n28 Tx on FDD carrier 1 should be turned off and n29/n67 Rx on SDL carrier 2 should be turn on. For Case#3 to Case#1, n29/n67 Rx on SDL carrier 2 should be turned off and n12/n28 Tx on FDD carrier 1 should be turned on. For both cases, FDD carrier 1 are always ON and the switching can be equivalent seen as between FDD carrier 1 Tx ON/OFF and SDL carrier 2 Rx OFF/ON. This switching also includes n12/n28 PA ON/OFF time, n29/n67 LNA ON/OFF time and n29/n67 LO retuning time. In TR 38.768 [3], RAN4 has reached the agreement in RAN4#114 on the applicable switching periods of 35μs, 70μs, and 140μs for switching between Case 1 and Case 2 as an optional UE capability to accommodate different UE implementations. In our view, these switching time between Case#1 and Case#3 can be also covered by the switching period between Case#1 to/from Case#2. Hence, it is proposed to reuse switching periods 35μs, 70μs, and 140μs as for switching between Case 1 and Case 3 as an optional UE capability.

Proposal 1: It is proposed to reuse switching periods 35μs, 70μs, and 140μs for switching between Case 1 and Case 3 as an optional UE capability.

The other primary challenge in implementing the new Case 3 for the example band combinations is the extreme spectral proximity between the two simultaneously received inter-band carriers. This may pose challenges to the UE's RF Front-End (RFFE) design, particularly concerning filter performance and isolation.

- For CA_n12A-n29A, the downlink bands are separated by an only 1 MHz guard band.

- For CA_n28A-n67A, the downlink bands are directly adjacent with a 0 MHz guard band.

Since the gap between FDD carrier 1 and SDL carrier 2 is very close, e.g., 1MHz or 0MHz, achieving the level of acceptable filter performance by using separate two filters may have a big impact to cost, size, and power consumption. It seems that the common filter with the common LNA to accommodate these adjacent two carriers may be a possible RFFE implementation for Case#3. In addition, it is also inevitable that the receiver on one carrier will experience desensitization issue from the other. In our view, ΔRIB,c (Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c) for each band carrier in Case#3 may need to be introduced. Hence, it is proposed that ΔRIB,c relaxation may be needed for Case#3 and the detailed value can be further discussed.

Proposal 2: It is proposed that ΔRIB,c relaxation may be needed for Case#3 and the detailed values can be further discussed.

Considering the scenario for Case#3 is similar to non-collocated intra-band non-contiguous CA scenario if the common filter and the common LNA are used for DL receiving, the Rx power imbalance may be needed to consider. In our view, Rx power imbalance between each band carrier in the common LNA for Case#3 may impact overall throughput performance. Hence, it is proposed that Rx power imbalance requirements may needed for Case#3 and the detailed value can be further discussed.

Proposal 3: It is proposed that Rx power imbalance requirements may needed for Case#3 and the detailed values can be further discussed.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: It is proposed to reuse switching periods 35μs, 70μs, and 140μs for switching between Case 1 and Case 3 as an optional UE capability.

Proposal 2: It is proposed that ΔRIB,c relaxation may be needed for Case#3 and the detailed values can be further discussed.

Proposal 3: It is proposed that Rx power imbalance requirements may needed for Case#3 and the detailed values can be further discussed.

# Reference

- RP-252953, “New WID on enhancement of low band carrier aggregation via switching”, Huawei, RAN#109

- RP-252702, “Motivation for low-band carrier aggregation via switching enhancement in Rel-20”, TELUS, Bell Mobility, Apple, BT plc, Skyworks Solutions, Inc, Murata, AT&T, RAN#109

- TR 38.768, “Low NR Band Carrier Aggregation via Switching”, V19.0.0