3GPP TSG-RAN WG4 Meeting #116bis	R4-2514390

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.18.2

Source:	Qualcomm Incorporated

Title: 		RRM performance requirements for XR enhancements

Document for:	Discussion

# Introduction

RAN4 completed the core part of the Work Item (WI) on XR enhancements during the last meeting [1]. The key enhancement introduced in this WI is the extension of cell detection and measurement delay requirements, facilitated by DCI-based Measurement Gap (MG) cancellation.

When specifying performance-related test cases, the following aspects should be considered:

- Measurement delay impacts,

- Frequency ranges (FR1 and FR2-1),

- Operating modes (NR Standalone and Carrier Aggregation).

In addition to defining the test cases, RAN4 should also discuss relevant test parameters, including:

- Number of MGs to be cancelled,

- Types of measurement gaps (e.g., per UE, per FR, Rel-15, Rel-17, concurrent gaps).

This contribution provides initial considerations on the above aspects to support the development of performance requirements under the XR enhancements WI.

# Test-cases list

To define the test case list, it is necessary to identify which RRM measurement requirements are impacted by the MG cancellation feature. Broadly, the following three categories of measurement delay requirements are affected:

- Intra-frequency measurements – cell identification and measurement requirements,

- Inter-frequency measurements – cell identification and measurement requirements,

- Inter-frequency L1-RSRP measurements – performed with measurement gaps.

The underlying feature under evaluation is the UE’s capability to receive and process the MG-cancelling DCI and subsequently perform measurements with extended delay while still meeting the existing accuracy requirements for the respective measurement type.

Given this, we propose that it is sufficient to evaluate the feature using a limited subset of measurement scenarios. For example, in FR1, a representative set of test cases can be selected from the categories listed above. A similar approach can be applied for FR2.

Proposal 1: RAN4 to consider shortlisting test cases for DCI-based Measurement Gap (MG) cancellation from the table below for FR1, aiming to cover representative scenarios across the impacted measurement types, while maintaining sufficient test coverage.

| Type | Title | Comments |
| --- | --- | --- |
| Intra-frequency | SA event triggered reporting tests with per-UE gaps under non-DRX (e.g., A.6.6.1.3) |  |
|  | SA event triggered reporting tests with per-UE gaps under DRX (e.g., A.6.6.1.4) | Maybe |
|  | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (e.g., A.6.6.1.6) | Yes |
| Inter-frequency | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used (e.g., A.6.6.2.1) | Maybe |
|  | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used (e.g., A.6.6.2.2) |  |
|  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used (e.g., A.6.6.2.5) |  |
|  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used (e.g., A.6.6.2.6) | Yes |
| Inter-frequency L1-RSRP | Inter-frequency SSB based L1-RSRP measurement with measurement gap (e.g., A.6.6.27.1) | Yes |

# Test-case parameters

For the evaluation of DCI-based MG cancellation, it is proposed to use per-UE measurement gaps as the baseline configuration, as these are the most commonly deployed and standardized gap types. Test scenarios should include cases where one (or maximum two) measurement gap is cancelled. This limitation is intended to balance test coverage with practical feasibility, as scenarios involving more than two cancellations would significantly increase test duration and complexity. Additionally, concurrent MG configurations may also be considered.

Proposal 2: RAN4 to adopt per-UE measurement gaps as the baseline configuration, with a single MG cancellation.

Proposal 3: One additional test case for concurrent MG configurations may also be introduced.

# Test-case design

Use incremental test-case design approach to specify the new test cases, where only additional parameters and configurations relative to the baseline test case are defined, along with the corresponding test requirements. This approach has been adopted in other WIs and has proven to be effective in minimizing redundancy, simplifying documentation, and facilitating easier interpretation and implementation of test cases.

Proposal 4: RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements.

# Conclusion

Proposal 1: RAN4 to consider shortlisting test cases for DCI-based Measurement Gap (MG) cancellation from the table below for FR1, aiming to cover representative scenarios across the impacted measurement types, while maintaining sufficient test coverage.

| Type | Title | Comments |
| --- | --- | --- |
| Intra-frequency | SA event triggered reporting tests with per-UE gaps under non-DRX (e.g., A.6.6.1.3) |  |
|  | SA event triggered reporting tests with per-UE gaps under DRX (e.g., A.6.6.1.4) | Maybe |
|  | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (e.g., A.6.6.1.6) | Yes |
| Inter-frequency | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used (e.g., A.6.6.2.1) | Maybe |
|  | SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used (e.g., A.6.6.2.2) |  |
|  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used (e.g., A.6.6.2.5) |  |
|  | SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used (e.g., A.6.6.2.6) | Yes |
| Inter-frequency L1-RSRP | Inter-frequency SSB based L1-RSRP measurement with measurement gap (e.g., A.6.6.27.1) | Yes |

Proposal 2: RAN4 to adopt per-UE measurement gaps as the baseline configuration, with a single MG cancellation.

Proposal 3: One additional test case for concurrent MG configurations may also be introduced.

Proposal 4: RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements.

# References

[1] R4-2512144, “WF on RRM requirements for XR_Ph3”, Nokia, RAN4#116