3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514401

Prague, CZ, October 13 – 17, 2025

Agenda Item:	4.15.1

Source:	MediaTek Inc.

Title:	Discussion on core part for FR2-1 SSB based L3 measurement delay reduction

Document for:	Discussion

# Introduction

In the last meeting RAN4 has closed the core part for R19 RRM phase 5 WI. In this paper we discuss the remaining issues in the maintenance for both part 1 (i.e., optimizing CSSF) and part 2 (I.e., optimizing Rx BSF) based on the WF [1][2].

# Discussion

For part 1 (optimizing CSSF), RAN4 made the following agreement as captured in the WF [1]

Issue 1-2-1: applicability limitation for solution 3 Agreement: For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG. FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

For Issue 1-2-1, RAN4 agreed to exclude enhancing CSSF based on 3 searchers for Inter-RAT measurement without MG scenario. However, some companies also proposed to exclude this enhancement for inter-frequency measurement without MG scenario as in the FFS above. In our understanding, the use of the searcher for inter-frequency w/o MG and intra-frequency w/o MG is not different (unlike the searcher for inter-RAT). However, we are open to discuss if any issue is identified for using the 3 searchers for inter-frequency w/o MG scenario.

Proposal 1: Discuss whether to exclude the 3-searcher enhancement for inter-frequency measurement without MG scenario.

For part 2 (I.e., optimizing Rx BSF), RAN4 also discussed the following issue as captured in the WF [2].

### Issue 1-1-5: Revision of the agreement on applicability requirement Agreement in Tuesday ad-hoc: - The FBS enhancement applies for the case when the target and serving carrier frequency are same. - FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different. Agreement in Thursday online: When the target and serving carrier frequency are different, - FBS requirement does not apply to inter-frequency measurement without measurement gap. - FFS inter-frequency measurement with per-FR measurement gap. - Note: inter-RAT is not in the scope.

For Issue 1-1-5, RAN4 agreed that FBS enhancement applies for the case when the target and serving carrier frequency are same (e.g., intra-frequency measurement with or without MG). If they are different carriers, the enhancement does not apply to inter-frequency measurement without MG. The remaining case to be discussed is inter-frequency measurement with per-FR MG as captured in the FFS above. After checking this, we realized that activating FBS is limited to serving cell in FR2 carrier anyway, as agreed in RAN4#114-bis and captured below. Therefore, even when UE is configured with FR2 inter-frequency measurement with per-FR gap, this enhancement can still be applied when both target and serving carrier are in FR2. Therefore, in our view we can close this issue without spec impact.

### Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in RRM session (Monday): - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell. - Leave the decision to RAN2. Further discuss the details for the LS to RAN2, and the LS can only be approved if the content is agreeable. - Details of the LS can refer to R4-2504962.

Proposal 2: FBS enhancement can apply to inter-frequency measurement with per-FR MG and there is no spec impact.

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: Discuss whether to exclude the 3-searcher enhancement for inter-frequency measurement without MG scenario.

Proposal 2: FBS enhancement can apply to inter-frequency measurement with per-FR MG and there is no spec impact.

# References

- R4-2512124, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.

- R4-2512125, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT.