3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2514410

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Way Forward on HPUE notes in 38.101-1 Clause 5 tables

Source: 	Skyworks Solutions, Inc.

Agenda Item:	5.2.6	RAN4 PRD

Document for:	Approval

# 1	Background

In RAN4#116, the HPUE MSD simplification [1] and Clause 5 and Clause 6 inter-band power class configuration [2] CRs were agreed. In this contribution, we discuss the work need to further simplify the clause 5 HPUE notes in preparation for Release 20 baskets.

# Discussion

## 2.1 Current Status after the HPUE MSD simplification

Based on the CRs [1,2] agreed in RAN4#116, the situation is the following:

- HPUE MSD simplification rules and LUT are in place for:

  - 1UL band 1CC MSDs (UL harmonic, harmonic mixing and cross-band)

  - 2UL bands 2CC MSDs (IMD2 to 5) UL configurations

- No HPUE MSD simplification for:

  - 1UL band 2CC IMD (IMD3 to IMD6 for TDD) related MSDs

  - 2UL bands 3CC triple beat (IMD3) related MSD

  - A proposal [3] is done in this meeting to simplify these in either in the PRD or in the TS HPUE MSD simplification clause.

- The inter-band 2Tx and 3Tx power classes have a table of the applicable per-band power class cases.

- The clause 5 table HPUE notes for 2UL 2CC UL configurations have been simplified and point to the Clause 6 tables.

Unless the proposals in [3] are adopted in the TS (as-is or are revised based on companies input), the HPUE notes are needed for the UL configuration that include an intra-band ULCA part: CA_nXXB/C/(2A) or CA_nXXA-nYYB/C. If adopted, it could be feasible for the Clause 5 HPUE notes to point to the Clause 6 HPUE power class tables for intra-band contiguous and non-contiguous ULCA.

Even for the cases that now have a HPUE MSD simplification in place, there has been no cross checking of all the legacy cases whether a PC3 case with no MSD may need a PC2 or PC1.5 MSD:

- For example, DL CA_n1-n3 only has n1 as HPUE, but it was not verified if n3 HPUE would require and exception for PC2 or PC1.5 (no PC3 MSD but PC2 or PC1.5 MSD needed) => single band notes must stay until this has been cross-checked

- Similarly, it was not verified whether all 3 band DL cases that have a specified 2UL band HPUE might need an exception for the third band for PC2 or PC1.5 (no PC3 MSD but PC2 or PC1.5 MSD needed) => 2UL band notes must stay until this has been cross-checked.

## 2.2 Way Forward

We understand that RAN4 is eager to simplify completely the inter-band HPUE introduction by fully using the MSD simplification LUTs and consequently remove the notes in Clause 5. As a company, we have worked hard to make this happen. However, for Release 19 the work is not finished:

- 1UL band 2CC and 2UL band 3CC cases have not (yet) and HPUE MSD simplification in place

- There is the need to at least cross-check that there is no exception for 3Band DL with 2 band UL 2CC cases in the legacy combinations and the existing 2UL HPUE cases.

  - With lower delta MSD the 1UL HPUE MSD is less critical.

- For new inter-band (2DL bands and 3DL bands with 1UL1CC or 2UL2CC UL configurations), we recommend that the HPUE MSD LUTs are fully used to specify MSDs directly with CRs.

Proposal:

- Until February 2026 RAN4 Meeting, the R19 Clause 5 HPUE Notes are kept (1UL, 2UL, intra-band ULCA).

- In the meantime:

  - RAN4 further work at establishing rules for the missing cases (1UL2CC and 2UL3CC)

  - RAN4 cross-checks PC2 or PC1.5 exceptions (no PC3 MSD but HPUE MSD needed) for at least the 3DL band 2UL2CC MSD cases for the third band.

- In March 2026 a Release 19 clean-up CR is submitted to RAN to serve as the basis for the Release 20 RAN4 specification.

  - FFS if all of Clause 5 HPUE notes can all be removed.

# Conclusion

In this contribution, we discuss the status after the HPUE MSD simplification CRs and make the following recommendations.

Proposal:

- Until February 2026 RAN4 Meeting, the R19 Clause 5 HPUE Notes are kept (1UL, 2UL, intra-band ULCA).

- In the meantime:

  - RAN4 further work at establishing rules for the missing cases (1UL2CC and 2UL3CC)

  - RAN4 cross-checks PC2 or PC1.5 exceptions (no PC3 MSD but HPUE MSD needed) for at least the 3DL band 2UL2CC MSD cases for the third band.

- In March 2026 a Release 19 clean-up CR is submitted to RAN to serve as the basis for the Release 20 RAN4 specification.

  - FFS if all of Clause 5 HPUE notes can all be removed.

# References

[1] R4-2511752 [NR_ENDC_RF_Ph4-Core] Draft CR to 38101-1-j20 on introducing the HPUE MSD requirements simplification, Skyworks Solutions Inc., Nokia, T-Mobile USA, ZTE, Oppo, Samsung, Murata Manufacturing Co Ltd., MediaTek, vivo, Huawei, Apple, Qualcomm France, RAN4#116

[2] R4-2511897 Draft CR to R19 38101-1 to introduce power class definitions for inter band CA wo-w MIMO, Skyworks Solutions Inc., Murata, T-Mobile USA, Qualcomm, RAN4#116

[3] R4-2514408 Proposal on 1UL band 2CC and 2UL band 3CC HPUE MSD, Skyworks Solutions Inc.,RAN4#116b