3GPP TSG-RAN WG4 Meeting #116bis	R4-2514487

Prague, Czech Republic, October 13th – 17th, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance requirements for SSB adaptation

Agenda item:	6.15.2.2

Document for:	Discussion

- 1. 	Introduction

Performance aspects for Rel-19 network energy saving were discussed in last meeting. In this contribution we continue discussing the test cases for SSB adaptation based on latest agreements and recommended WF [1]. Further discussion details can be found in the topic summary [2].

- Discussion

The following progress was achieved at RAN4 #116 on performance requirements for SSB adaptation [1].

- Sub-topic 3-3: SSB adaptation performance part Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement RAN4 to discuss whether the following SSB adaptation test case list can be defined. - L1-RSRP - L1-SINR - CBD Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection.

Test case list

Regarding the test case list in issue 3-3-1, we support all of the above listed test cases for L1-RSRP, L1-SINR and CBD, since the scope of SSB adaptation was reduced at RAN4 #116 to activated SCell operation anyway.

Proposal 1: 	Test cases for SSB adaptation should be introduced also for L1-RSRP, L1-SINR and CBD.

General testing principle

Regarding the general principle in issue 3-3-2, the above list can serve as a baseline. However, given the restricted scope of SSB adaptation for activated SCell operation, as agreed at RAN4 #116, we believe that following two bullets need further consideration:

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- without SSB time index detection.

The first one is an essential part of SSN adaptation and hence should be verified in a test case. For the second one, further study is needed on the additional impact if SSB index detection is included in the cell detection and

Proposal 2: 	For general testing principle, include test case for verifying L1/L3 transition requirement and study the additional impact for including a test case with SSB time index detection.

SMTC and SSB configuration

The proposals and the recommended WF from RAN4#116 [2] are shown below.

Issue 3-3-3: SMTC and SSB configuration - Proposals - Option 1(Samsung): - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation. - Default SSB periodicity: 20ms - Larger SSB periodicity: 80/160ms - Denser SSB periodicity: 5/10ms - Option 2(Nokia, Ericsson): - RAN4 to use SMTC periodicity change from 80ms to 20ms and SSB periodicity change from 80ms to 20ms, and vice versa. - Recommended WF - Further discussion

We support option 2. It covers very well the SSB periodicity range up to 160ms.

Proposal 3: 	RAN4 to use SMTC periodicity change from 80ms to 20ms and SSB periodicity change from 80ms to 20ms, and vice versa, for test case design.

NCD-SSB configuration

The proposals and the recommended WF from RAN4#116 [2] are shown below.

Issue 3-3-4: NCD-SSB configuration - Proposals - Proposal 1(Ericsson): - RAN4 to introduce new NCD-SSB configuration with 20ms and 80ms. - Recommended WF - Further discussion.

The issue has not been further discussed at RAN4 #116.

It is noted that this issue is related both to RRM core requirements and RRM performance requirements. It is also being discussed under RRM core requirements maintenance in [3]. Nevertheless, the work on specifying a test case for NCD-SSB adaptation should proceed in our view.

Proposal 4: 	RAN4 to continue work on introducing a test case for applying SSB adaptation to NCD-SSB.

Conclusion

In this contribution, we discussed the RRM performance requirements for Rel-19 network energy saving for the feature SSB adaptation.

The following proposals are made.

Proposal 1: 	Test cases for SSB adaptation should be introduced also for L1-RSRP, L1-SINR and CBD.

Proposal 2: 	For general testing principle, include test case for verifying L1/L3 transition requirement and study the additional impact for including a test case with SSB time index detection.

Proposal 3: 	RAN4 to use SMTC periodicity change from 80ms to 20ms and SSB periodicity change from 80ms to 20ms, and vice versa, for test case design.

Proposal 4: 	RAN4 to continue work on introducing a test case for applying SSB adaptation to NCD-SSB.

- References

[1]	R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4 #116

[2]	R4-2509063, Topic summary for [116][220] Netw_Energy_NR_enh_Part1, Ericsson, RAN4 #116

[3]	R4-2513721, Core maintenance for network energy saving, Nokia, Nokia Shanghai Bell, RAN4 #116bis