3GPP TSG-RAN4 Meeting #116bis	R4-251xxxx

# Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:			8.12

Source:	Feature lead (vivo)

Title:	WF for [116bis][110] 6G testability and OTA

Document for:	Approval

# Introduction

This Way Forward captures the agreements of 6G OTA in RAN4.

# Topic #1: Improved testability of Conducted Requirements

### Sub-topic 1-1 Antenna assumption of FR1 transmitter and receiver

Issue 1-1-1: Whether RAN4 should consider a More realistic antenna efficiency assumption for FR1

- Proposals

  - Proposal 1: Discuss a more realistic assumption on antenna efficiency for FR1 band tests, e.g. -5.5 dB.

Agreements:

- move this discussion to general RF/UE RF from next meeting.

### Sub-topic 1-2 Improve traditional conducted test to OTA

Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach

Agreements:

  - RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g.,

    - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD

    - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission

    - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern.

# Topic #2: New test methodologies for new 6GR frequencies

### Sub-topic 2-1 Testability for new 6GR frequency

Issue 2-1-1: Testability for New Frequencies between FR1 and FR2

Agreements:

RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered:

- The frequency range definition will be discussed under the spectrum agenda.

- For conducted test, use existing test method as a starting point

- For OTA test,

  - Further study potential test methodologies including the full-package, i.e., supported frequency range, test setup, configuration, positioning, procedure, validation/calibration, quiet-zone/test zone, MU, testing time reduction

# Topic #3: OTA test methods for Multi-Tx and CA

### Sub-topic 3-1 OTA for 6GR multi-carrier

Issue 3-1-1: OTA test methodology for FR1 CA

Agreements:

RAN4 consider the study of testability for single carrier (with single or multi-Tx/Rx) as 1st priority.

# Topic #4: Testability for different Device types

### Sub-topic 4-1 OTA testability for different device types

Issue 4-1-1: OTA testability applicability for different UE types in 6GR day-1

Agreements:

RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases.

- The UE types and form factor considered in testability can be further discussed in this SI.

- RAN4 study the feasibility of developing a single system to cover above aspects.

# Topic #5: AI/ML OTA testability

### Sub-topic 5-1 Enhanced OTA test method for 6GR AI/ML cases

Issue 5-1-1: RAN4 study on enhancement of AI/ML testability for 6GR

Agreements:

The corresponding test method including both OTA and conducted, to verify the AI/ML features (FFS details) should be studied in RAN4 in 6G SI.

- The methodologies identified in 5G will be considered as the starting point for both conducted and OTA.

# Topic #6: Harmonized testing for TN and NTN

### Sub-topic 6-1 Harmonized OTA test methodologies for TN and NTN

Issue 6-1-1: Harmonized OTA test methodologies for TN and NTN

Way Forward

  - Targeting a harmonized test system/test method for TN and NTN devices.

# Topic #7: Improved test methods and metric for SISO OTA

### Sub-topic 7-1 Enhanced OTA test method for TRP/TRS

Issue 7-1-1: Enhanced OTA test method for TRP/TRS

Agreements:

- RAN4 further study the feasibility of incorporating real-world usage scenarios more closely into the 6GR OTA testing methodology.

  - FFS on OTA configurations/parameters to reflect real-world scenarios, e.g., support antenna switching, and other aspects.

# Topic #8: Improved test methods and metric for DL MIMO OTA

### Sub-topic 8-1 MIMO OTA for 6GR

Issue 8-1-1: RAN4 consider MIMO OTA for 6GR day-1

Agreements：

RAN4 will study the dynamic MIMO OTA (at least dynamic channel model and link adaptation) for 6G. static MIMO is not precluded. The following can be considered as starting point:

- 2D or 3D channel model

- FFS phantom involved testing

- FFS multi-TRP

- FFS focus on FR1 as first priority

  - FR2 and new frequency range may also be considered

# Efficiency improvement (Test and requirements)

### Sub-topic 9-1 Improvement of OTA testing efficiency

Issue 9-1-1: Improve OTA test efficiency in 6GR

Agreements:

6G OTA testability study should consider system complexity and test time reduction.

# Topic #10: Testability for BS requirements

### Sub-topic 10-1 BS related testability

Issue 10-1-1: Study on BS testability

- Proposals

  - Proposal 1: RAN4 should study the improvement of BS OTA test methods for FR3. (vivo)

- Recommended WF

  - TBA.

FL: check with BS session whether any issue of testability should be discussed in 6G SI.

# Topic #11: Others

### Sub-topic 11-1 New requirements/metric for OTA

Issue 11-1-1: new Energy Efficiency performance under OTA metric

- Proposals

  - Proposal 1: Discuss Energy Efficiency evaluation under OTA performance metric. (Xiaomi)

- Recommended WF

  - TBA.

FL: keep it Open for further discussion.

Issue 11-1-2: new OTA metric for SBFD

- Proposals

  - Proposal 1: Study OTA testing for SBFD. (E///)

- Recommended WF

  - TBA.

FL: keep it Open for further discussion.

### Sub-topic 11-3 General procedure on introducing alternative test method in RAN4

Issue 11-3-1: General procedure on introducing alternative test method in RAN4

Agreements:

- RAN4 could work on a general principle on introducing alternative methods.