3GPP TSG-RAN WG4 Meeting #116bis		R4-25xxxxx
Prague, Czech Republic, 13 – 17 Oct 2025

Agenda item:			4.1.1

Source:	Moderator (OPPO)

Title:	Topic summary for [116bis][113] R19_UERF_maintenance

Document for:	Information

Introduction

This is the summary for R19_UERF_maintenance under agenda 4.2 and except 4.2.7.

List of topics below:

- Discussion papers and corresponding CRs (22)

  - Draft CRs for 38.101-1 (2)

  - Draft CRs for 38.101-2 (5)

  - Draft CRs for 38.101-3 (1)

  - Draft CRs for 38.101-5 (4)

  - Draft CRs for 36.102 (2)

  - Draft CRs for 38.863 (1)

Topic #1: R19 topics exclude Ku-Band for NR NTN

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2514002 (R15) CAT-A: R4-2514003 R4-2514004 R4-2514005 R4-2514006 | ZTE Corporation,Sanechips | Draft CR for 38.101-2 Correct the descriptions for BWintraCA and A-MPR NS_202 Moderator: The title in the cover page has a typo, it said the draft CR is for 38.101-1. |
| R4-2513845 (R19) | Apple | For 38.101-5 Correction of the NTN NR band n250 UL configuration for REFSENS testing |
| R4-2513846 (R19) | Apple | For 38.863 Addition of power back-off simulation results for the NTN L-bands |
| R4-2513295 | Murata Manufacturing Co Ltd. | Proposal 1: RAN4 to use MSD tables for 10MHz of n41 in draft CRs (R4-2513296 and R4-2513297) for 38.101-1 and 38.101-3. |
| R4-2513296 (R19) | Murata Manufacturing Co Ltd. | Draft CR 38.101-1 to change MSD for n41 10MHz CBW |
| R4-2513297 (R19) | Murata Manufacturing Co Ltd. | Draft CR 38.101-3 to change MSD for n41 10MHz CBW |
| R4-2513322 | Murata Manufacturing Co Ltd. | Observation: The Tx noise of n71 falling into n5 DL do not increase even change the n71 UL CBW from 20MHz to 25MHz. Proposal 1: RAN4 to consider the MSD value highlighted in the following table 1 for PC3 CA_n5A-n71A and PC3 CA_n71A-n85A for n71 UL CBW 25MHz. Proposal 2: RAN4 to consider the MSD value highlighted in the following table 2 PC2 CA_n71A-n85A for n71 UL CBW 25MHz. Table 1. PC3 MSD for CA_n5A-n71A and CA_n71A-n85A UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n5; 685.5; 25; 15; 20 (RBstart=113); 871.5; 5; 2.0; >ACLR2. …; …; …; …; …; …; …; …; …; …. n71; n85; 685.5; 25; 15; 20 (RBstart=113); 730.5; 5; 10.16; ACLR2. n71; n85; 680.5; 35; 15; 20 (Rbstart=168); 730.5; 5; 237; ACLR1. NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 8: Applicable when n41 spectrum is restricted to 2515-2675MHz. Table 2. PC2 MSD for CA_n71A-n85A UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n85; 685.5; 25; 15; 20 (RBstart=113); 730.5; 5; 13.14,6 16.14,7; ACLR2. n71; n85; 680.5; 35; 15; 20 (Rbstart=168); 730.5; 5; 265,6 32.35,7; ACLR1. NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx. |
| R4-2513325 (R19) | Murata Manufacturing Co Ltd. | Draft CR 38.101-1 to change MSD for n71 25MHz CBW |
| R4-2513841 | ViaSat Satellite Holdings Ltd | Observation 1: Two sets of requirements have been specified for NR NTN bands n250, n251 and n253 (Annex B) – set 1 based on current ETSI requirements in EN 301 681 [4, Annex A] and set 2 based on requirements likely to be included in the new ETSI standard inferred from [3]. Observation 2: RAN4 has agreed that given that the new ETSI standard has not been completed, the emissions requirements and the NS flags and A-MPR values associated with set 1 requirements (current ETSI requirements in EN 301 681) are in force. Observation 3: The ETSI emissions requirements specified for bands n251 also apply to NR NTN band n255 but have not been incorporated in TS 38.101-5. Observation 4: The ETSI emissions requirements specified for bands n251 and n253 also apply to IoT NTN bands 255 and 253 respectively but have not been incorporated in TS 36.102. Observation 5: The A-MPR assessment for PC2 n255 NR NTN UE was performed without considering the ETSI requirements. Observation 6: The A-MPR assessment for PC2 and PC1 band 255 IoT NTN UEs was performed without considering the ETSI requirements. Observation 7: If the ETSI requirements are not specified for n255/b253/b255 operators (like Viasat, Space42) will be unable to signal that the UE needs to comply with additional emission requirements in Region 1 and any other country where ETSI emissions requirements are required to be met. Observation 8: If proprietary power back-off implementations are implemented to pass regulatory certification, operators will be unable to plan its deployment such that the UE can close the link using the allotted/assigned RBs – this will likely result in severe degradation and/or connectivity failure for its customers. Proposal 1: Update the n255 specifications in TS 38.101-5 to capture the two sets of requirements and associated NS flags/A-MPR for PC3 NR NTN UE from Rel-17 specifications onwards. Proposal 2: Update the Rel-19 n255 specifications to specify the A-MPR for PC2 NR NTN UE for the flags associated with the two sets of requirements. Proposal 3: Update the band 253 and 255 specifications in TS 36.102 to capture the two sets of requirements and associated NS flags/A-MPR for PC3 IoT NTN UE from Rel-18 specifications onwards. Proposal 4: Update the Rel-19 specifications for b255 to specify the A-MPR for PC2 and PC1 IoT NTN UEs for the flags associated with the two sets of requirements. |
| R4-2513842 (R19) | ViaSat Satellite Holdings Ltd | For 38.101-5 Updates to unwanted emissions specifications for NR NTN band n255 to align with ETSI emissions requirements |
| R4-2513843 (R18) | ViaSat Satellite Holdings Ltd | For 38.101-5 Updates to unwanted emissions specifications for NR NTN band n255 to align with ETSI emissions requirements |
| R4-2513844 (R17) | ViaSat Satellite Holdings Ltd | For 38.101-5 Updates to unwanted emissions specifications for NR NTN band n255 to align with ETSI emissions requirements |
| R4-2513858 (R19) | ViaSat Satellite Holdings Ltd | For 36.102 Updates to unwanted emissions specifications for IoT NTN bands 253 and 255 to align with ETSI emissions regulations |
| R4-2513859 (R18) | ViaSat Satellite Holdings Ltd | For 36.102 Updates to unwanted emissions specifications for IoT NTN bands 253 and 255 to align with ETSI emissions regulations |
| R4-2514315 | Qualcomm | Proposal 1: Modify n71 Channel Bandwidths, REFSENS, and UL RB allocation for REFSENS as follows for Rel-19 NR Band; SCS (kHz); UE Channel bandwidth (MHz). 3; 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. n71; 15; 5; 10; 15; 20; 2512; 3012; 3512. 30; 10; 15; 20; 2512; 3012; 3512. NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification.. Operating band / SCS / Channel bandwidth. Operating Band; SCS kHz; 3 MHz (dBm); 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); 25 MHz (dBm); 30 MHz (dBm); 35 MHz (dBm); 40 MHz (dBm); 45 MHz (dBm); 50 MHz (dBm). n71; 15; -97.2; -94.0; -91.6; -86.0; -84.19 -74.810; -82.59 -67.110; -80.79 -64.010. 30; -94.3; -91.9; -87.4; -84.29 -74.910; -82.69 -67.210; -80.89 -64.110. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW.. Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode. Operating Band; SCS; 3; 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100; Duplex Mode. n71; 15; 25; 251; 201; 201; 201,6; 201,6; 201,6; FDD. 30; 121; 101; 101; 101,6; 101,6; 101,6. Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply.. Proposal 2: Modify CA_n5A-n71A MSD with n71 UL as follows for Rel-19 UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n5; 688685.5; 2025; 15; 20 (RBstart=86); 871.5; 5; 2.0; >ACLR2. Proposal 3: Modify CA_n71A-n85A MSD with n71 UL as follows UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n85; 688; 20; 15; 20 (RBstart=86); 730.5; 5; 8.2116; ACLR2. n71; n85; 680.5; 35; 15; 20 (Rbstart=168); 730.5; 5; 237; ACLR1. |
| R4-2514409 | ViaSat Satellite Holdings Ltd | Repeat with R4-2513841 |
| R4-2514494 | Skyworks Solutions Inc. | # Proposal 1: Remove note 12 from Table 5.3.5-1 for 25MHz CBW as highlighted in yellow in the table below. NR Band; SCS (kHz); UE Channel bandwidth (MHz). 3; 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. n71; 15; 5; 10; 15; 20; 25; 3012; 3512. 30; 10; 15; 20; 25; 3012; 3512. NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification.. Proposal 2: For 25MHz CBW, remove optional CBW REFSENS requirement and Note 10 from the REFSENS Table 7.3.2-1a, and remove Note 6 from the UL configuration requirement Table 7.3.2-3 as highlighted in yellow in the tables below. Operating band / SCS / Channel bandwidth. Operating Band; SCS kHz; 3 MHz (dBm); 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); 25 MHz (dBm); 30 MHz (dBm); 35 MHz (dBm); 40 MHz (dBm); 45 MHz (dBm); 50 MHz (dBm). n71; 15; -97.2; -94.0; -91.6; -86.0; -74.8; -82.59 -67.110; -80.79 -64.010. 30; -94.3; -91.9; -87.4; -74.9; -82.69 -67.210; -80.89 -64.110. NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW.. Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode. Operating Band; SCS; 3; 5; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100; Duplex Mode. n71; 15; 25; 251; 201; 201; 201; 201,6; 201,6; FDD. 30; 121; 101; 101; 101; 101,6; 101,6. Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply.. Observation: The MSD requirements for CA_n71B are not impacted by mandating support of symmetrical UL/DL 25MHz CBW. Proposal 3: In Table 7.3A.6-1 and for CA_n5-n71, consider replacing the Band n71 20MHz UL CBW MSD test point with the changes highlighted in yellow for the Band n5 PC3 MSD. Due to BCS4/5, attach Note 6 to this requirement. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n5; 685.5; 25; 15; 20 (RBstart=113); 871.5; 5; 2.06; >ACLR2. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth. Proposal 4: In Table 7.3A.6-1 and for CA_n71-n85, consider adopting the changes highlighted in yellow for the Band n85 PC3 MSD due to Band n71 25MHz UL CBW cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n85; 685.5; 25; 15; 20 (RBstart=113); 730.5; 5; 9.46; ACLR2. n71; n85; 680.5; 35; 15; 20 (Rbstart=168); 730.5; 5; 237; ACLR1. NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. Proposal 5: In Table 7.3A.6-1a-1 and for CA_n71-n85, consider adopting the changes highlighted in yellow for the Band n85 MSD due to Band n71 25MHz UL CBW PC2 cross-band isolation interference. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n85; 685.5; 25; 15; 20 (RBstart=113); 730.5; 5; 12.44,6 15.44,7; ACLR2. n71; n85; 680.5; 35; 15; 20 (Rbstart=168); 730.5; 5; 265,6 32.35,7; ACLR1. NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx. Proposal 6: In Table 7.3A.6-1a-1 and for CA_n71-n85, consider correcting the Band n85 5MHz CBW MSD due to Band n71 35MHz UL CBW PC22Tx cross-band isolation interference to 29dB. |
| R4-2514499 | Skyworks Solutions Inc. | Observation: Based on the partial parsing of TS 38.101-1 (cf. Annex), we find that the number of Band n41 MSD requirements that need to be re-evaluated may become significant. The partial parsing of TS 38.101-1 indicates that the band n41 5 MHz CBW MSD ranges from 5.2 dB to ~ 30 dB. Proposal: To simplify the re-evaluation of a significant number of Band n41 5MHz CBW MSD requirements, application of the following MSD correction factors is proposed: - 3 dB for n41 5 MHz CBW MSD >10 dB - 2.5 dB for 8 dB ≤ n41 5 MHz CBW MSD ≤ 10 dB - 2.0 dB for 5 dB ≤ n41 5 MHz CBW MSD ≤ 7 dB The correction factor could be easily extended to MSD less than 5 dB should such a case be found after all MSD tables are parsed. It is proposed that these changes are filed in the same CR that removes the redundant dual-UL IMD MSD requirements. |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: 10MHz test points for band n41

Sub-topic description:

- Band n41/n90:

  - 5MHz CBW remains optional. Update 38.101-1 and 38.101-3 MSD test points for 10MHz CBW with CR at next meeting to ensure that the band n41/n90 MSD requirements are no longer specified for 5MHz CBW.

In RAN4 #116 meeting, it was agreed in WF R4-2511779

Issue 1-1: Update MSD tables for 10MHz of Band n41

- Proposals

  - Proposal 1: Check the MSD values for 10MHz of n41 in draft CRs (R4-2513296 and R4-2513297) for 38.101-1 and 38.101-3 (Murata)

  - Proposal 2: To simplify the re-evaluation of a significant number of Band n41 5MHz CBW MSD requirements, application of the following MSD correction factors is proposed: (Skyworks)

    - 3 dB for n41 5 MHz CBW MSD >10 dB

    - 2.5 dB for 8 dB ≤ n41 5 MHz CBW MSD ≤ 10 dB

    - 2.0 dB for 5 dB ≤ n41 5 MHz CBW MSD ≤ 7 dB

- Recommended WF

  - TBA

Related CRs

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2513296 (R19) | Murata Manufacturing Co Ltd. | Draft CR 38.101-1 to change MSD for n41 10MHz CBW |  |
| R4-2513297 (R19) | Murata Manufacturing Co Ltd. | Draft CR 38.101-3 to change MSD for n41 10MHz CBW |  |

Sub-topic 1-2: n71 UL CBW 25MHz

Sub-topic description

- Band n71:

  - Companies are invited to study how to mandate symmetrical UL 25MHz / DL 25MHz CBW for band n71 as mandatory support, in particular

    - Study removing Note 10 from the DL 25MHz REFSENS requirement,

    - Study the impact on PC3 CA_n5A-n71A and PC3 PC2 CA_n71A-n85A cross-band isolation MSD due to BCS4/5 new greatest CBW,

  - UL 30MHz and UL 35MHz remain optional CBW,

  - No change to the band n71 asymmetric channel bandwidths requirements, i.e.

    - No change to the UL 20MHz / DL 30MHz and UL 20MHz / DL 35MHz REFSENS

    - No change to the UL 30MHz / DL 30MHz and UL 35MHz / DL 35MHz REFSENS

    - No need to study UL 25MHz / DL 30MHz and UL 25MHz / DL 35MHz REFSENS

In RAN4 #116 meeting, it was agreed in WF R4-2511779

Issue 1-2-1: n71

- Proposals

  - Proposal 1: Modify n71 Channel Bandwidths, REFSENS, and UL RB allocation for REFSENS as follows for Rel-19 (Qualcomm, Skyworks)

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 |  | 5 | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |
| NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -84.19 -74.810 | -82.59 -67.110 | -80.79 -64.010 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.4 | -84.29 -74.910 | -82.69 -67.210 | -80.89 -64.110 |  |  |  |
| NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n71 | 15 |  | 25 | 251 | 201 | 201 | 201,6 | 201,6 | 201,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 | 101,6 | 101,6 | 101,6 |  |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - Check whether proposal 1 is agreeable.

Issue 1-2-2: PC3 CA including n71

- Proposals

  - Proposal 1: MSD values for PC3 CA_n5A-n71A and PC3 CA_n71A-n85A when n71 UL CBW is 25MHz. (Murata)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 685.5 | 25 | 15 | 20 (RBstart=113) | 871.5 | 5 | 2.0 | >ACLR2 |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 10.16 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |

  - Proposal 2: Modify CA_n71A-n85A MSD with n71 UL as follows (Qualcomm)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 688685.5 | 2025 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.0 | >ACLR2 |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2116 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |

  - Proposal 3: Modify CA_n71A-n85A MSD with n71 UL as follows (Skyworks)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 685.5 | 25 | 15 | 20 (RBstart=113) | 871.5 | 5 | 2.06 | >ACLR2 |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 9.46 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |

- Recommended WF

  - Introduce superscript 6 for the MSD value of CA_n5-n71;

  - Further discuss the MSD value for CA_n71-n85 based on below table, whether the average value can adopt?

| UL band | DL band |  | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD (dB) |  |  |  | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | Murata | Qualcomm | Skyworks | Average |  |
| n71 | n5 |  | 685.5 | 25 | 15 | 20 (RBstart=113) | 871.5 | 5 | 2.0 | 2.0 | 2.06 |  | >ACLR2 |
| n71 | n85 |  | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 10.16 | 116 | 9.46 | 10.2 | ACLR2 |
|  |  | NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |

Issue 1-2-3: PC2 CA including n71

- Proposals

  - Proposal 1: MSD value for PC2 CA_n71A-n85A when n71 UL CBW is 25MHz. (Murata)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 13.14,6 16.14,7 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.35,7 | ACLR1 |

  - for CA_n71-n85, consider adopting the changes highlighted in yellow for the Band n85 MSD due to Band n71 25MHz UL CBW PC2 cross-band isolation interference. (Skyworks)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 12.44,6 15.44,7 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.3295,7 | ACLR1 |
| NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - Further discuss the MSD value for PC2 CA_n71A-n85A when n71 UL CBW is 25MHz, whether the average values can adopt

  - Check whether MSD value need update for PC2 with 2Tx CA_n71A-n85A when n71 UL CBW is 35MHz

| UL band | DL band | UL Fc |  | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD (dB) |  |  | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) |  | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | Murata | Skyworks | Average |  |
| n71 | n85 | 685.5 |  | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 13.14,6 16.14,7 | 12.44,6 15.44,7 | 12.8 15.8 | ACLR2 |
| n71 | n85 | 680.5 |  | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.35,7 | 265,6 32.3295,7 |  | ACLR1 |
|  |  |  | NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

Related CRs

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2513325 (R19) | Murata Manufacturing Co Ltd. | Draft CR 38.101-1 to change MSD for n71 25MHz CBW |  |

Sub-topic 1-3: ETSI emission requirements for IoT NTN bands 253 and 255 and NR NTN band n255

Sub-topic description

In RAN4 #116 meeting, NS_09N/NS_13N for new bands n253 and n250, NS_10N/NS_14N for new band n251 had been introduced in Rel-19 CR R4-2511961.

Open issues and candidate options before meeting:

Issue 1-3-1: n255

- Proposals

  - Proposal 1: Update the n255 specifications in TS 38.101-5 to capture the two sets of requirements and associated NS flags/A-MPR for PC3 NR NTN UE from Rel-17 specifications onwards. (ViaSat)

  - Proposal 2: Update the Rel-19 n255 specifications to specify the A-MPR for PC2 NR NTN UE for the flags associated with the two sets of requirements. (ViaSat)

- Recommended WF

  - TBA

Issue 1-3-2: 253 and 255

- Proposals

  - Proposal 1: Update the band 253 and 255 specifications in TS 36.102 to capture the two sets of requirements and associated NS flags/A-MPR for PC3 IoT NTN UE from Rel-18 specifications onwards. (ViaSat)

  - Proposal 2: Update the Rel-19 specifications for b255 to specify the A-MPR for PC2 and PC1 IoT NTN UEs for the flags associated with the two sets of requirements. (ViaSat)

- Recommended WF

  - TBA

Related CRs

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2513844 (R17) CAT-A: (had submitted) R4-2513843 (R18) R4-2513842 (R19) | ViaSat Satellite Holdings Ltd | For 38.101-5 Updates to unwanted emissions specifications for NR NTN band n255 to align with ETSI emissions requirements |  |
| R4-2513859 (R18) CAT-A: (had submitted) R4-2513858 (R19) | ViaSat Satellite Holdings Ltd | For 36.102 Updates to unwanted emissions specifications for IoT NTN bands 253 and 255 to align with ETSI emissions regulations |  |

Sub-topic 1-4: CRs for mmW in NR

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2514002 (R15) CAT-A: R4-2514003 R4-2514004 R4-2514005 R4-2514006 | ZTE Corporation,Sanechips | Draft CR for 38.101-2 Correct the descriptions for BWintraCA and A-MPR NS_202 Moderator: The title in the cover page has a typo, it said the draft CR is for 38.101-1. |  |

Sub-topic 1-5: CRs for NR-NTN L-bands

| T-doc | Company | Comments collection | Recommend |
| --- | --- | --- | --- |
| R4-2513845 (R19) | Apple | For 38.101-5 Correction of the NTN NR band n250 UL configuration for REFSENS testing |  |
| R4-2513846 (R19) | Apple | For 38.863 Addition of power back-off simulation results for the NTN L-bands |  |