3GPP TSG-RAN WG4 Meeting #116bis									                      R4-2514521

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:			4.1.1

Source:	Moderator (Huawei)

Title:	Topic summary for [116bis][114] R19_UERF_maintenance_Part1

Document for:	Information

# Introduction

This summary handles Rel-19 maintenance t-docs under agenda 4.3,4.4.1, 4.5, 4.6(except4.6.2), 4.7.1 and 4.8.1.

Deadline for flag is by 14.10.2025 (Tuesday), 18:00 (local time).

NOTE:

The content of R4-2514190 by Anritsu is not specific to 6Rx agenda so that somewhere else to get more UE RF people’s attention or it would be handled directly under this agenda if not.

# Topic #1: HPUE handling

## Companies’ contributions summary

| T-doc number | Company | Title |
| --- | --- | --- |
| R4-2514495 | Skyworks Solutions Inc. | # Proposal: In clause 7.3C.2 - Introduce missing PC2 MSD requirements for harmonic, cross band isolation and harmonic mixing in new Table 7.3C.2-2a, Table 7.3C.2-4a and Table 7.3C.2-6a respectively based on [2,3,4], - Introduce 1UL HPUE MSD requirements with LUTs in a new sub-clause 7.3C.2.1 based on [6]. # Moderator note: directly check if R4-2514492 is agreeable or not via NWM. |
| R4-2514497 | Skyworks Solutions Inc. | Proposal: Bring clean-up CR to remove the redundant 2UL IMD MSD requirements from MSD tables in TS 38.101-1 and TS 38.101-3. Exceptions to these clean-ups are MSD requirements which cannot be verified due to regional frequency range restrictions – see the Annex for a detailed list of changes. |
| R4-2514410 | Skyworks Solutions Inc. | Proposal: - Until February 2026 RAN4 Meeting, the R19 Clause 5 HPUE Notes are kept (1UL, 2UL, intra-band ULCA). - In the meantime: - RAN4 further work at establishing rules for the missing cases (1UL2CC and 2UL3CC) - RAN4 cross-checks PC2 or PC1.5 exceptions (no PC3 MSD but HPUE MSD needed) for at least the 3DL band 2UL2CC MSD cases for the third band. - In March 2026 a Release 19 clean-up CR is submitted to RAN to serve as the basis for the Release 20 RAN4 specification. FFS if all of Clause 5 HPUE notes can all be removed. |
| R4-2513851 | Apple | Observation 1: One of the main reasons the PC2 and PC1.5 applicability notes in band combination configuration tables of the RAN4 UE RF specifications are necessitated is to ensure the corresponding MSD requirements are specified. Observation 2: RAN task on HPUE support for band combinations without HPUE applicability notes only mitigated the issue for DL CA combinations with single UL configured. Observation 3: The introduction of HPUE MSD LUTs is intended to resolve all the issues due to the lack of HPUE MSD requirements which has been hindering the HPUE support for band combinations without the HPUE applicability notes in clause 5. Observation 4: With HPUE MSD LUTs, the existing HPUE applicability notes in band combination configuration tables in clause 5 of the RAN4 UE RF specifications in principle are no longer needed. Observation 5: RAN4 seemed not ready to move away from the existing HPUE band combination introduction process (at least in Rel-19) despite the HPUE LUTs are already developed. Observation 6: With HPUE MSD LUTs specified, the current HPUE band combination introduction process is essentially redundant since the MSD can already be referred to the LUTs and there is no need to further add them to the legacy MSD tables. Observation 7: The HPUE MSD LUTs are supposed to resolve the disenabled HPUE support issue for band combinations without the HPUE applicability notes in clause 5 which is still left open even after the LUTs introduction. Proposal 1a: Add two general notes below Table 5.5A.3.1-1o (last CA configuration table for inter-band CA with 2 bands): NOTE 17: For single uplink carrier with up to 2Tx antenna connectors without NOTE 8, minimum requirements for Power Class 2 are applicable provided the said power class has been specified in Table 6.2.1-1 or Table 6.2D.1-1 respectively and the corresponding PC2 ΔMSD is specified in Table 7.3A.2.3.1-1 and Table 7.3A.2.3.1-2 for this downlink/uplink combination. NOTE 18: For single uplink carrier with up to 2Tx antenna connectors without NOTE 9, minimum requirements for Power Class 1.5 are applicable provided the said power class has been specified in Table 6.2.1-1 and the corresponding PC1.5 ΔMSD is specified in Table 7.3A.2.3.1-1 and Table 7.3A.2.3.1-2 for this downlink/uplink combination. Proposal 1b: Remove the texts in clause 7.3A.1 introduced via RAN task for CA combinations with single uplink as they are no longer needed after the HPUE MSD LUT introduction. Proposal 2: Add two general notes below Table 5.5A.3.2-1c (last CA configuration table for inter-band CA with 3 bands): NOTE 15: For a 2-band UL configuration without NOTE 7 or NOTE 13, minimum requirements for Power Class 2 are applicable provided the said power class has been specified in Table 6.2A.1.3-1 or Table 6.2H.3.1-1 respectively and the corresponding PC2 ΔMSD is specified in Table 7.3A.2.3.2-1 and Table 7.3A.2.3.2-2 for this downlink/uplink combination. NOTE 16: For a 2-band UL configuration without NOTE 14, minimum requirements for Power Class 1.5 are applicable provided the said power class has been specified in Table 6.2H.3.1-1 and the corresponding PC1.5 ΔMSD is specified in Table 7.3A.2.3.2-1 and Table 7.3A.2.3.2-2 for this downlink/uplink combination. Proposal 3: For the extended Rel-19 HPUE basket WIDs, proponents are encouraged to only introduce new 2UL/2DL combinations and not to capture MSD requirements in legacy MSD tables if they are covered by the HPUE MSD LUTs nor to add HPUE applicability notes in clause 5 to reduce the RAN4 workloads. Proposal 4: Remove HPUE applicability notes in band combination configuration tables in clause 5 in Rel-20 RAN4 specifications. Moderator note: This can be discussed once Rel-19 HPUE applicability notes handling become clear. Proposal 5: Do not continue HPUE basket WIDs for band combinations in Rel-20. The 2-band HPUE support can be requested in generic 2UL/2DL basket WIDs. |

## Open issues summary

### Sub-topic 1-1: Redundant dual-UL IMD MSD handling

Issue 1-1: Redundant dual-UL IMD MSD handling

- Proposal(R4-2514497 by SKY)

  - Bring clean-up CR to remove the redundant 2UL IMD MSD requirements from MSD tables in TS 38.101-1 and TS 38.101-3. Exceptions to these clean-ups are MSD requirements which cannot be verified due to regional frequency range restrictions – see the Annex for a detailed list of changes.

- Recommended WF

  - Check if the proposal is agreeable or not.

### Sub-topic 1-2: Clause 5 HPUE Notes

Issue 1-2-1: Work plan for Rel-19

- Proposal(R4-2514410 by SKY)

  - Until February 2026 RAN4 Meeting, the R19 Clause 5 HPUE Notes are kept (1UL, 2UL, intra-band ULCA).

  - In the meantime:

    - RAN4 further work at establishing rules for the missing cases (1UL2CC and 2UL3CC)

    - RAN4 cross-checks PC2 or PC1.5 exceptions (no PC3 MSD but HPUE MSD needed) for at least the 3DL band 2UL2CC MSD cases for the third band.

  - In March 2026 a Release 19 clean-up CR is submitted to RAN to serve as the basis for the Release 20 RAN4 specification.

  - FFS if all of Clause 5 HPUE notes can all be removed.

- Recommended WF

  - Check if the proposal is agreeable or not.

Issue 1-2-2: Work process for Rel-19

- Proposal(R4-2513851 by Apple)

  - For the extended Rel-19 HPUE basket WIDs, proponents are encouraged to only introduce new 2UL/2DL combinations and not to capture MSD requirements in legacy MSD tables if they are covered by the HPUE MSD LUTs nor to add HPUE applicability notes in clause 5 to reduce the RAN4 workloads

- Recommended WF

  - Check if the proposal is agreeable or not.

Issue 1-2-3: HPUE applicability notes handling

- Proposal(R4-2513851 by Apple)

  - For clause 5, add

    - two general notes below Table 5.5A.3.1-1o (last CA configuration table for inter-band CA with 2 bands):

    - two general notes below Table 5.5A.3.2-1c (last CA configuration table for inter-band CA with 3 bands)

  - For clause 7, remove:

    - the texts in clause 7.3A.1 introduced via RAN task for CA combinations with single uplink as they are no longer needed after the HPUE MSD LUT introduction

- Recommended WF

  - Check if the proposal is agreeable or not.

Issue 1-2-4: Work plan for Rel-20

- Proposal(R4-2513851 by Apple)

  - Do not continue HPUE basket WIDs for band combinations in Rel-20. The 2-band HPUE support can be requested in generic 2UL/2DL basket WIDs.

- Recommended WF

  - This is TSG RAN level discussion. Hence, we just check if we can see consensus in RAN4 level.

## Recommendation for T-docs

| T-doc | Comments collection | Recommendation |
| --- | --- | --- |
| R4-2513171 CR for -1 by CATT |  |  |
| R4-2513172 CR for -3 by CATT |  |  |
| R4-2513361 CR for -1 by Samsung |  |  |
| R4-2513852 CR for -1 by Apple |  |  |
| R4-2514000 CR for -1 by ZTE |  |  |
| R4-2514114 CR for -1 by CHTTL |  |  |
| R4-2514316 CR for -3 by Qualcomm |  |  |
| R4-2514492 CR for -1 by Skyworks |  | NOTE: A companion CR of R4-2514495 |

# Topic #2: Power domain enhancement(MPR reduction)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513742 | Skyworks Solutions Inc. | Proposal on modification to enable PC2 and PC1.5 2Tx: - RB/BW extension framework should be enabled for PC2 2Tx and PC1.5 2Tx: - PC3 and PC2 restriction highlighted in yellow below in following 38.101-1 clause 6.2.2 is modified to include PC1.5 and 1Tx/2Tx as highlighted in yellow below: If all of the following conditions are simultaneously met: - The UE is a PC3 or PC2 PC3, PC2 or PC1.5 UE supporting 1Tx or 2Tx operating in any channel bandwidth except 3MHz - The UE indicates support for MPR reduction for single carrier with single value or MPR reduction for single carrier with multiple values, and is configured with the IE Extension UE-Rel19 - The uplink modulation order is either QPSK or 16QAM - When no A-MPR is applicable - 38.101-1 clause 6.2D.2 text below Table 6.2D.2-5 is modified for PC1.5 edge allocation as highlighted in yellow below. Inner, outer and edge allocations are as defined in section 6.2.2 except for PC1.5 edge allocations which is for LCRB ≤ 4 RBs instead of LCRB ≤ 2 RBs for other power classes. When a UE indicates support for MPR reduction for single carrier with single value or MPR reduction for single carrier with multiple values, and is configured with the IE Extension UE-Rel19, an Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost edge of the channel LCRB ≤ 4 RBs with Rext_low = 0, or the one for which the RB(s) is (are) allocated at the uppermost edge of the channel LCRB ≤ 4 RBs with Rext_high = 0. Proposal on enabling R18 power boosting: Although our data shows margin for new inners to enable boosting, further verification is needed for the few PC3 and PC2 cases with less than 3dB and 1.5dB margin respectively. Proposal on min CBW: - On top of 3MHz, 7MHz is excluded from the RB/BW extension framework. 38.101-1 clause 6.2.2 is modified to exclude 7MHz CBW - Further checks for the 250% BW criteria can be performed for 5MHz CBW and if necessary additional testing can be performed for this CBW. |
| R4-2513850 | Apple | Observation 1: For intra-band CA MPR scenarios with single CC UL the single CC MPR is specified by simply referencing the MPR tables. However, intra-band CA calculation for inner/outer still applies. Since intra-band CA does not specify edge RB allocations this raises the question whether edge RB definitions from specific clauses (6.2.2 or 6.2D.2) should be directly referenced or not. Observation 2: The option of referencing to the single CC MPR tables is one approach it might also be considered to directly refer to the single CC clauses itself (either 6.2.2 or 6.2D.2) Proposal 1: RAN4 should discuss whether applicability of edge RB allocations should be directly specified to avoid confusion and whether the approach of directly referencing to the requirements of clauses 6.2.2 and 6.2D.2 would be a better option. Observation 3: One result of RAN4 discussion was that any channel which is subject to A-MPR is not subject to single carrier MPR enhancement . The reason for introducing this limitation has been explained in. The Big CR features the A-MPR aspect as by stating “When no A-MPR is applicable” then the enhancement can be applicable. While this statement reflects RAN4 understanding it might be slightly confusion to the reader as it is not explicitly specified that it refers to the channel. Proposal 2: Add the channel context to the A-MPR condition. If all of the following conditions are simultaneously met: - The UE is a PC3 or PC2 UE operating in any channel bandwidth except 3MHz - The UE indicates support for MPR reduction for single carrier with single value or MPR reduction for single carrier with multiple values, and is configured with the IE Extension UE-Rel19 - The uplink modulation order is either QPSK or 16QAM - When no A-MPR is applicable to the configured channel. Observation 4: To enable boosting with respect to the extended channel the equation defining the ‘enhanced power inner allocation region’ needs to be modified to use the extended RBs. Proposal 3: If RAN4 decides to enable simultaneous use of Rel-19 MPR enhancement and Rel-18 power boost feature then there needs to be an indication for the UE that it can support both features simultaneous. |
| R4-2514304 | Qualcomm Incorporated | Proposal 1: For a UE that supports MPR reduction through bandwidth expansion allow the same level of power boost in the expanded inner and expanded enhanced inner regions as in the original inner and enhanced inner regions. Proposal 2: If a UE supports MPR reduction through bandwidth expansion then the expanded enhanced power inner allocation region within the expanded inner region is defined so any waveform it contains satisfies the following condition: RBStart,Low_new + P1 ≤ RBStart ≤ RBStart,High_new - P1 Where RBStart,Low_new = max(1, floor((LCRB)/2))- RBShift_low) RBStart,High_new = = min(NRB-LCRB, NRB + RBShift_high - max(1, floor((LCRB)/2))- LCRB) P1 = MIN{12,CEIL(2+NRB/25)} Where RBShift_low and RBShift_high are the RB extensions corresponding to CBW/2 or CBW/4 bandwidth extensions |

## Open issues summary

### Sub-topic 2-1: Spec clarification

Issue 2-1-1: Reference from intra band CA to single carrier MPR

- Proposal(R4-2513850 by Apple)

  - RAN4 should discuss whether applicability of edge RB allocations should be directly specified to avoid confusion and whether the approach of directly referencing to the requirements of clauses 6.2.2 and 6.2D.2 would be a better option.

- Recommended WF

  - Check views from other companies

Issue 2-1-2: Clarification on no applicability of MRP reduction to configured UE CBW with A-MPR

- Proposal(R4-2513850 by Apple)

  - Add the configured channel context to the A-MPR condition

- Recommended WF

  - Check if the proposal is agreeable or not

### Sub-topic 2-1: Applicability of Rel-19 MPR reduction

Issue 2-1: Extension of MPR reduction to 2Tx PC2 and PC1.5

- Proposal(R4-2513742)

  - Enable RB/BW extension framework for PC2 2Tx and PC1.5 2Tx

- Recommended WF

  - Check if the proposal is agreeable. If agreed, how to do it can be further discussed.

Issue 2-2: Concurrent usage of Rel-18 power boosting and Rel-19 MPR reduction

- Proposal 1 (R4-2513742 by SKY)

  - Further verification is needed for the few PC3 and PC2 cases with less than 3dB and 1.5dB margin respectively

- Proposal 2 (R4-2513850 by Apple)

  - If RAN4 decides to enable simultaneous use of Rel-19 MPR enhancement and Rel-18 power boost feature then there needs to be an indication for the UE that it can support both features simultaneous

- Proposal 3 (R4-2514304 by Qualcomm)

  - For a UE that supports MPR reduction through bandwidth expansion allow the same level of power boost in the expanded inner and expanded enhanced  inner regions as in the original inner and enhanced inner regions.

- Recommended WF

  - Check proposal 1, and if agreeable, we may not have to take time to discuss proposal 2 and 3 in this meeting.

## Recommendation for CRs and TPs

| T-doc | Comments collection | Recommendation |
| --- | --- | --- |
| R4-2513246 CR for -1 by CATT |  |  |
| R4-2513918 CR for -1 by Huawei |  |  |
| R4-2513919 CR for -3 by Huawei |  |  |

## Open issues summary

# Topic#3: 6Rx

## Companies’ contributions summary

| T-doc number | Company | Title |
| --- | --- | --- |
| R4-2513762 | T-Mobile USA, BT, SK Telecom, TELUS, Verizon, Deutsche Telekom, Bell Mobility. Boost Mobile, Southern Linc, Telecom Italia, Telstra, Vodafone | Proposal: Because of the large commercial interest for 6Rx for FWA, we propose to support option 3, i.e., release independence of 6Rx from Rel-18 for FWA both for 4 and 6 MIMO layers |
| R4-2514481 | Qualcomm | Observation 1: There is significant interest from operators globally in defining 6Rx requirements in a release-independent manner starting from Rel-18. Observation 2: While signaling support for 6Rx devices has been recently introduced with Rel-19 in mind, it could be early implementable, thereby allowing support from previous releases. Proposal 1: Define RF and demodulation requirements for 6Rx devices, along with 6L signaling, in a release-independent manner starting from Rel-18. |
| R4-2514001 | ZTE Corporation,Sanechips | draft CR to TS38.101-1_Remove square bracket for the 3T6R IE |
| R4-2514190 | Anritsu Limited | This should be not discussed here. |

## Open issues summary

### Sub-topic 3-1: 6Rx release independence

Issue 3-1: From which release, release independent for 6Rx with max 6L or 4L applies

- Proposals

  - Option 1(R4-2513762 by T-Mobile USA et al.): Release independence of 6Rx from Rel-18 for FWA both for 4 and 6 MIMO layers (T-Mobile USA et al.)

  - Option 2(R4-2514481 by Qualcomm): Define RF and demodulation requirements for 6Rx devices, along with 6L signaling, in a release-independent manner starting from Rel-18(Qualcomm)

- Recommended WF

  - Better to wait for receiving RAN2 LS reply to LS of R4-2511898 approved by RAN4#116.

## Recommendation for T-docs

| T-doc | Comments collection | Recommendation |
| --- | --- | --- |
| R4-2514001 CR for -1 by ZTE |  |  |

# Topic #4: NonCol_intraB_ENDC_NR_CA_Ph2-Core

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513611 | Apple | Proposal 1: It is proposed to update the requirement to cover the case that “6dB power imbalance apply for type 4 UE when it indicates intraBandNR-CA-non-collocated-r19 and nonCollocatedTypeNR-CA-v1900 is absent.” |

## Open issues summary

### Sub-topic 4-1

Issue 4-1: Clarification of applicability of 6dB power spectrum density

- Proposal

  - Update the requirement to cover the case that “6dB power imbalance apply for type 4 UE when it indicates intraBandNR-CA-non-collocated-r19 and nonCollocatedTypeNR-CA-v1900 is absent.”

- Recommended WF

  - Check if the agreeable or not

## Recommendation for T-docs

| T-doc | Comments collection | Recommendation |
| --- | --- | --- |
| R4-2513612 CR for -1 by Apple |  |  |
| R4-2513613 CR for -3 by Apple |  |  |

# Topic #3: NR_LBCA_Sw

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513165 | CATT | Observation 1: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for Low-Low bands inter-band CA via switching. Observation 2: for Low-Low bands inter-band CA via switching, it’s possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together. Observation 3: the DL RMC for the LBCA-SwitchingPattern should include at least one SSB in PCell and one SSB in SCell and option 1 in the WF R4-2511862 can’t align the RAN1’s understanding on the restrictions for the semi-static switching pattern. Proposal 1: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together. 1-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’. 1-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots. 1-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots. 1-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 1. Moderator note: See the details of the Figure 1 in the contribution Proposal 2: to design the following timing relationship and values according to the updated formulas for switching gap calculations. RTDP2S; µs; 0. RTDS2P; µs; 0. NTA; Tc; 0. NTA,offset; Tc; 25600. swithcingPeriodForFDD-SDL (Tswitch); µs; 35; 70; 140. gapDurationPCelltoSCell-r19; in units of 15KHz SCS symbol; 1; 1; 2. gapDurationSCelltoPCell-r19; in units of 15KHz SCS symbol; 1; 2; 3. Observation 4: For LB-LB CA via switching, a semi-static pattern with 40bits map should be specified for UL requirements testing, especially for the UL on/off time mask. Observation 5: For LB-LB CA via switching, the switching gap should be considered in the UL switch-from slot of the PCell FDD band, which will result that the available PUSCH symbols in the UL switch-from slot of the PCell FDD band are less than 14, i.e. 11, 12 and 13 symbols for one, two and three switching gaps respectively. Proposal 3: to specify the following aspects in order to complete the UL RMC for LB-LB CA via switching: 3-1: to specify LBCA-SwitchingPattern for UL RMC of LB-LB CA via switching. 3-2: to specify the UL RMC in the UL switch-from slot of the PCell FDD band considering the switching gap, which will result that the available PUSCH symbols in the UL switch-from slot of the PCell FDD band are less than 14, i.e. 11, 12 and 13 symbols for one, two and three switching gaps respectively. 3-3: RAN4 explore the test methodology which is used to verify the UL transmitting signal’s performance for UE before and/or after the Switching gap(s). |
| R4-2513166 | CATT | TP to TR 38.768 to modify the formulas for the switching gap calculation |
| R4-2513167 | CATT | draft CR for TS 38.101-1 to introduce RMC for LB-LB CA via switching |
| R4-2513782 | MediaTek Inc. | Proposal 1: Try to reuse most of parameter values in existing TS38.101-1 RMCs when considering LBCA: - PDCCH length: Two symbols PDCCH in DL slots with PDSCH - PDSCH mapping type: Type A - Periodicity of TRS: 20ms - Number of additional DMRS: 2 Proposal 2: By considering the worst case of switching period Tswitch with 140us. For DL RMC, we propose to use 3 symbols for LBCA-SwitchingGap-Duration-PCelltoSCell and LBCA-SwitchingGap-Duration-SCelltoPCell. Proposal 3: To facilitate 20ms transmission of SSB and TRS, we propose to set the LBCA-SwitchingPattern as ‘0000000000, 1111111111, 0000000000, 1111111111’ in DL RMC. Proposal 4: We propose RMC Table 1 and 2 for DL CA switching test configuration Moderator note: See the details of the Table 1 and 2 in the contribution Proposal 5: There’s no need to specify UL RMC Proposal 6: RAN4 need to define the UE-coexistence requirements to consider this coexistence scenarios for CA_n12-n29 and CA_n28-n67 Proposal 7: RAN4 agree on the CR in [2] R4-2513780 capturing proposal 1~6 Moderator note: R4-2513780 would actually intend to refer to R4-2513939. |
| R4-2513939 | MediaTek Inc. | A companion CR for R4-2513782 |
| R4-2513854 | Apple | Observation #1: DL measurement channels for both single-carrier and carrier aggregation as defined in clause A.3.2 are based on one NR frame (10 slots). Proposal #1: For consistency with the current specifications and to easily compared the UE REFSENS performance with and without LBCA via switching, we propose using 1 NR frame (10 slots) to define DL RMCs for low NR ban aggregation via switching. Observation #2: With the switching pattern “0000000000, 1111111111, 0000000000, 111111111” as proposed in option 2, the UE may not be able to receive fallback DCI scheduled PDSCH at the beginning of SDL due to fact that HARQ-ACK cannot be scheduled (Fallback DCI assumes a maximum of 8 slots as captured in clause 9.2.3 of TS38.213). Proposal #2: We prefer using Option 1 which is based on 10ms (10 slots) periodicity as a candidate for DL RMC for for low NR band aggregation via switching. Observation #3: Testing UE ON/OFF UL performance when the UE is subjected to switching is similar to testing the ON/OFF performance for the UL Tx switching feature. Proposal #3: New UL RMCs are not needed to evaluate the UE UL performance for low NR band aggregation via switching. The existing RMCs defined in Annexes A.2 and A.3 can be used to test the UL performance, just as it is the case for evaluating UL Tx switching |

## Open issues summary

### Sub-topic 5-1: DL RMC

Moderator note: Need to confirm which options actually work. Then, we discuss which one is the best among the workable options.

Issue 5-1: Selection of DL RMC options

- Proposals

  - Option 1(R4-2513854 by Apple): 10ms periodicity with switching period placed in slots 4 and 8

  - Option 2(R4-2513165 by CATT): Increase period of RMC to 40 slots and use the LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’(CATT)

  - Option 3(R4-2513782 by MTK): Bitmap (0000000000, 1111111111, 0000000000, 1111111111)

- Recommended WF

  - First, check if all the three options are feasible or not. For example, Apple (R4-2513854) raised an issue on Option 3 in terms of feasibility.

### Sub-topic 5-2: UL side

Issue 5-2-1: Necessity of UL RMC options

- Proposals

  - Option 1: Introduce NR_LBCA_Sw specific UL RMC (CATT)

  - Option 2: No need to introduce NR_LBCA_Sw specific UL RMC (Apple, MTK)

- Recommended WF

  - First, confirm if the existing UL RMC works or not. Then, discuss if the new UL RMC specific to NR_LBCA_Sw is essential or not.

Issue 5-2-2: Necessity of test methodology to verify UL signal’s performance before/after the Switching gap

- Proposals

  - Option 1: RAN4 explore the test methodology which is used to verify the UL transmitting signal’s performance for UE before and/or after the Switching gap(s). (CATT)

  - Option 2: Not needed

- Recommended WF

  - Check if option 1 is agreeable or not.

## Recommendation for T-docs

| T-doc | Comments collection | Recommendation |
| --- | --- | --- |
| R4-2513939 TP for TR by CATT |  |  |
| R4-2513167 CR by CATT |  |  |
| R4-2513166 CR by MTK |  |  |