3GPP TSG-RAN4 Meeting #116bis	 R4-25xxxxx

Prague, Czech Republic, 13rd – 17th October 2025

Agenda item:			4.1.3

Source:	CMCC

Title:	                       WF on [116bis][326] A-IoT_Maintenance

Document for:	Approval

Introduction

This document captures the agreements on A-IoT_Maintenance in RAN4#116bis.

Topic #1: RF requirements for A-IoT device

Issue 1-1: backscatter power

WF:

Further consider these two options:

- Option 1: Use the test configuration in Table 2 for D2R measurement.

Table 2: Test configuration for D2R measurement

| Measurement parameter | Value |
| --- | --- |
| CW frequency (FCW) | According to TS 38.192 |
| Filter passband range for D2R upper sideband | FCW+RSFS-1Tb*0.9 to FCW+RSFS+1Tb*1.1 |
| Filter passband range for D2R lower sideband | FCW-RSFS+1Tb*1.1 to FCW-RSFS-1Tb*0.9 |

- Option 2: The test configuration in Table A.1-1 is defined to be used for testing the backscattering loss requirement defined in section 6.1.1, the filter position is illustrated in Figure A.1-1.

Table A.1-1: Test configuraton

| Test parameter | value |
| --- | --- |
| filter bandwidth (kHz) | 1Tb* 1.1/0.9 |
| CW frequency (MHz) | According to TS 38.194 |
| Filter center frequency offset to CW frequency | 1/Tc |

Figure A.1-1: illustration of measurement filter configuration

Issue 1-2: REFSENSE

WF:

- Single M =6

- RAN4 can only retain 1PRB and delete 2/3/4PRB configurations for measurement channel

Issue 1-3: maximum input level

Agreement:

- Table 4 with M=24 and 3 PRBs.

Table 4: Fixed Reference Channels for maximum input level

| Component | Parameter | Unit | Value |
| --- | --- | --- | --- |
| General | PRB | RBs | 3 |
|  | SCS | kHz | 15 |
| SIP | Bit length | Bits | 8 |
|  | Mapping to OFDM | Chips/Symbol | 4 |
| CAP | Bit length | Bits | 4 |
|  | M | Chips/Symbol | 24 |
| PRDCH | TBS | Bits | Depending on the size of the MAC PDU of A-IoT CFA paging message |
|  | CRC | Bits | 16 |
|  | M | Chips/Symbol | 24 |
| Postamble | Bit length | Bits | 4 |
|  | M | Chips/Symbol | 24 |
| Padding | Padding per OFDM symbol excluding SIP and Postamble | Chips | 2 |
| Padding | Padding for last OFDM symbol | Chips | Depending on the TBS |

Issue 1-4: Other requirements

WF:

- Use the D2R RMC in Table 6 to test spurious emission requirement.

  - FFS Parameters for backscatter power.

Table 6: FRC for [backscatter power,] SEM and spurious emissions

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission BW | kHz | 15 |
| TBS | Bits | Depending on the length of AIoT device ID |
| CRC | Bits | 16 |
| FEC code rate |  | 1/3 |
| Block repetition number |  | 1 |
| Preamble length | Bits | 31 |
| Midamble length | Bits | 31 |
| Interval for midamble insersion | Bits | 48 |
| Additional midamble insertion |  | No |
| Small frequency shift | kHz | 480 |
| Modulation |  | BPSK/OOK (NOTE1) |
| NOTE 1: The modulation scheme used is up to device implementation. |  |  |

Topic #2: RF requirements for A-IoT BS and CW

Issue 2-1: ACLR limit

Agreement:

- No need to define ACLR absolute limit for A-IoT BS.

Issue 2-2: Foffset for A-IoT operation

WF:

- For Tx side of A-IoT operation, A-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset as defined in Table 3 below where CBW is defined in 5.3.1 and 5.3.2 of TS38.194.

Table 3 Foffset for A-IoT operation

| Carrier | Foffset |
| --- | --- |
| Ambient IoT | 100 kHz+CBW/2 |

- FFS on the Rx side.

Issue 2-3: CW phase noise

WF:

- FFS RAN4 to further check whether it is necessary to modify above CW phase noise based on CW’s implementation performance.

Issue 2-4: interfering signal

WF:

- FFS whether to update the interfering signal for ACS, in-band blocking, and narrowband intermodulation from 3 MHz E-UTRA/NR to 5 MHz NR.The motivation is that 5 MHz NR is more widely deployed than 3 MHz NR, making these requirements more representative of real-world conditions.