3GPP TSG-RAN WG4 Meeting #116bis	R4-2515053

Prague, Czech Republic, October 13 ‒ October 17, 2025

Title: 	Way Forward for [116bis][328] Rel-19 Demodulation_Part3

Agenda Item:	6.23

Source: 	Ericsson

Document for:	Approval

# NTN testing for NGSO

Channel models

Agreement: Eccentricity of satellite orbit

- No need to capture “The method specified in this sub clause is applicable to cases that Eccentricity (e) in Step 1-4 is more than zero” in the specification, because e≠0 for the initial ephemeris information developed in Rel-19 RAN4.

Agreement: Additional margin to IoT-NTN UE demodulation requirements when the time-varying Doppler shift and propagation delay model is applied

- For NB-IoT, add 0.5dB to the existing NPDSCH requirements.

- For eMTC, add 1.0dB to the existing PDSCH requirements.

Agreement: Impact of sampling frequency offset

- RAN4 does not need to consider the impact of sampling frequency offset for UE demodulation requirements.

Way forward: How to ensure the satellite elevation angles more than 30 degrees during the test

Background: The time-varying Doppler shift and propagation delay model for LEO-600 ensures the elevation angle of one satellite is more than 30 degrees only for about 250 seconds.

- Interested companies are encouraged to study how to keep that the elevation angle above 30 degrees during the test if the total test time is more than 250 seconds.

Way forward: Applicability of Rel-19 tests with time-varying Doppler shift and propagation delay model

Background In Rel-17 RAN4 defined four PDSCH demodulation requirements for NR NTN (1-1, 1-2, 1-3, and 1-4). In this WI, RAN4 has agreed to add 2 new test cases (1-5 and 1-6) by applying the time-varying Doppler shift and propagation delay model for NGSO to tests 1-1 and 1-2, as follows. Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 0.3. 1-2; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; 7.6. 1-3; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLC5-200; 1x2, ULA Low; 70; -0.4. 1-4; R.PDSCH.1-1.1 FDD(1); 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 1.1. 1-5; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; [0.3+0.5]. 1-6; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; [7.6+0.5]. Note 1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. Note 2: For Tests 1-5 and 1-6, the time-varying Doppler shift and propagation delay model, specified in Annex E, is applied..

For UE declaring Rel-19 UE,

- Option 1

  - Applicable tests are 1-5, 1-6, 1-3 and 1-4.

- Option 2

  - Applicable tests are 1-5, 1-6.

For UE declaring Rel-17/18 UE,

- Option 1a:

  - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6, 1-3 and 1-4.

  - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4.

- Option 1b:

  - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6.

  - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4.

- Option 2:

  - Applicable tests are 1-1, 1-2, 1-3 and 1-4.

Note the same test applicability rule is used for IoT-NTN UE demodulation requirements.