3GPP TSG-RAN WG4 Meeting # 116-bis											R4-2514547

Prague, Oct. 13th ‒ Oct. 17th, 2025

Agenda item:			5.2.2

Source:	Nokia, AT&T

Title:	TP to TR 37.719-11-11 addition of DC_30A_n12A

Document for:	Approval

# Introduction

This is a TP to TR 37.719-11-11 to add DC_30A_n12A.

************************************* Start of TP*****************************************

## 6.x.1	DC_ 30_n12

#### 6.x.1.1 	Configurations for DC

Table 6.x.1.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_30A_n12A | DC_30A_n12A | No |

#### 6.x.2.2	Maximum output power for DC

Table 6.x.2.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_30A_n12A | 23 | +2/-3 |

#### 6.x.2.3	Spurious emission band UE co-existence for DC

Not applicable.

#### 6.x.2.4	Co-existence analysis for DC

Table 6.x.2.4-1 Band 30 and Band n12 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 30 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 2305 | 4610 | 6915 | 9220 | 11525 |  |
| n12 | fLow | fHigh | 2315 | 4630 | 6945 | 9260 | 11575 |  |
| DL1 | 729 | 746 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1458 | 1492 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2187 | 2238 | - | - | N/A | - | N/A |  |
| DL4 | 2916 | 2984 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3645 | 3730 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issues found. |  |  |  |  |  |
| UL/DL harmonics |  | n12 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 699 | 1398 | 2097 | 2796 | 3495 |  |
| 30 | fLow | fHigh | 716 | 1432 | 2148 | 2864 | 3580 |  |
| DL1 | 2350 | 2360 | N/A | - | - | - | - | UL harmonic |
| DL23 | 4700 | 4720 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 7050 | 7080 | - | - | N/A | - | N/A |  |
| DL4 | 9400 | 9440 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 11750 | 11800 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issues found. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Based on Table 6.x.2.4-1, there is no UL harmonic or harmonic mixing issues.

Table 6.x.2.4-2: Band 30 and Band n12 cross-band isolation analysis

| Bands | 30 |  | n12 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 2305 | 2315 | 699 | 716 |
| fDL (MHz) | 2350 | 2360 | 729 | 746 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 10 | 5 | 15 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 2295 | 2325 | 684 | 731 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 2285 | 2335 | 669 | 746 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 2275 | 2345 | 654 | 761 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 2265 | 2355 | 639 | 776 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 2255 | 2365 | 624 | 791 |
| Analysis | No issues found |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1-1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible. NOTE 2: The maximum UL channel bandwidth of the band (noted maxULCBW) is used to calculate the ACLR ranges. |  |  |  |  |

Based on Table 6.x.2.4-2, there is no cross-band issue for this combination.

Table 6.x.2.4-3: Band 30 and Band n12 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1589 - 1616 |  | 3004 - 3031 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3894 - 3931 |  | 873 - 917 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5309 - 5346 |  | 3703 - 3747 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6199 - 6246 |  | 157 - 218 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3178 - 3232 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7614 - 7661 |  | 4402 - 4463 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6008 - 6062 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 481 - 559 |  | 8504 - 8561 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2462 - 2533 |  | 5483 - 5547 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5101 - 5179 |  | 9919 - 9976 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6707 - 6778 |  | 8313 - 8377 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.x.2.4-3, there is no IMD issue for this combination.

#### 6.x.2.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_30_n12 as specified in TS 38.101-3.

Table 6.x.2.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_30_n12 | 0.3 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.x.2.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_30_n12 | - | 0.2 |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

#### 6.x.2.6	Analysis of MSD requirements

Based on the co-existence studies for DC_30_n12, no MSD requirements are needed for this combination.

************************************* End of TP*****************************************