3GPP TSG-RAN WG4 Meeting #116bis	R4-2514579

Prague, Czech, 13th – 17th October, 2025

Title: 	WF on HPUE for FRMCS

Agenda Item:	5.1

Source: 	Nokia, Union Inter. Chemins de Fer

Document for:	Approval

# Agreements:

# Issue 1: Maximum output power for CA

# Add a new table UE Power Class for uplink inter-band CA (two bands) for FRMCS in bands n100 and n101. No need to introduce single band PC2 and PC1.5 for bands n100 and n101.

Table 6.2A.1.3-3: UE Power Class for uplink inter-band CA (two bands) for FRMCS in bands n100 and n101

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n100-n101 | 31 | +2/-3 |
| NOTE: Uplink CA configuration CA_n100-n101 is allowed only for FRMCS cab-radio’s, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground, with UE having power class 1 capability per band. |  |  |

# Issue 2: Maximum output power for NR_DC

# Add a new table UE Power Class for uplink inter-band NR_DC for FRMCS in bands n100 and n101. No need to introduce single band PC2 and PC1.5 for bands n100 and n101.

Table 6.2B.1.3-2: UE Power Class for uplink inter-band NR_DC for FRMCS in bands n100 and n101

| Uplink NR_DC Configuration | Class 1 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n100-n101 | 31 | +2/-3 |
| NOTE: Uplink NR_DC configuration DC_n100-n101 is allowed only for FRMCS cab-radio’s, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground, with UE having power class 1 capability per band. |  |  |

# Issue 3: MPR

# No new MPR.

# Issue 4: A-MPR

# No new A-MPR.

# Issue 2: ACLR

# No new ACLR.

# Issue 5: MSD

# No MSD.