s3GPP TSG-RAN WG5 Meeting #108	R5-254236

Bengaluru, India, Aug 25th – Aug 29th, 2025

Source:	Nokia

Title:	Testability Aspects for AI/ML Based Beam Management

Agenda item:	4.2.3

Document for:	Endorsement

Introduction

In RAN5#105, the following way forward was agreed [1] for AIML PHY.

RAN5#105 Agreement - Proposal 1: RAN5 continues to look for opportunity to start contributing to topics that could speed up the work of Rel-19 AIML core WI. - Proposal 2: Besides the areas below, RAN5 continues to identify more areas where RAN5 expertise may help to progress core group work. - AIML conformance test framework. - MU/TT analysis based on RAN4 test parameters and test setup. - UE lab conformance aspects of post-deployment testing.

In RAN#107, the revised SID for the Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 was agreed [2]. In this revised SID, the following scope was agreed for RAN4 and RAN5. Hence, the WID [3] was revised accordingly.

- Testability and interoperability [RAN4]: - Further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In this contribution, we share our views on the AIML conformance test framework on the following topics:

- New UE Special Conformance Test function for AIML use cases

Discussion

New UE Conformance Test Function for AI/ML Use Cases

An example of appropriate usage of test modes in Possible AI ML Call Flow for Conformance with UE side Beam Management Case-1 is presented in the contribution  [4]. Also, a generic framework for the AIML use cases is covered by [5].

To support the conformance testing of 5G systems (5GS), UE special conformance test functions are required. They form a part of the core requirements and thus have a direct impact on the design of the UE. The UE special conformance test functions defined in TS 38.509 vary depending on the conformance testing functionality they are designed to support.

RAN1 has concluded the Rel-19 work including AIML PHY WI and sent a consolidated list of parameters for RAN2 to consider in the LS[6]. RAN5 to keep this LS in consideration and discuss if UE special test functions are needed for each use case and mode separately. Thus, having UE test function for each AIML use case and mode (data collection at UE side/NW assisted, training, inference) gives RAN5 the flexibility to design test cases to ensure each AIML mode functionality is tested independently, as highlighted in red in Figure 1.

Considering the beam management use case scenario, we have the following modes to be tested as shown in Figure 1.

Figure 1 OTA call flow diagram

In RAN2 and other WGs, the discussion on UE-side data collection for the beam management use case is still ongoing. However, RAN5 shall discuss the AIML inference mode of the AI/ML beam management use case.

The existing UE test functions aren’t sufficient to handle the Rel-19 AI/ML use cases and modes.

RAN1 has completed Rel-19 work and has sent the consolidated list of parameters to RAN2 to consider including the NR AIML PHY.

In RAN2/SA2/CT1 groups, the discussion on UE-side data collection for the AI/ML BM use case is still ongoing.

RAN5 to further discuss a new UE special conformance test function to cater to AIML-enabled functionalities.

RAN5 to evaluate and agree to add the new UE test functions for each of the AIML use cases and modes.

Conclusion

In this contribution, we share our views on UE conformance testing framework on below topics:

- New UE Conformance Test Function for AI/ML use cases

The following observations and proposals were made:

- Observation 1: The existing UE test functions aren’t sufficient to handle the Rel-19 AI/ML use cases and modes.

Observation 2: RAN1 has consolidated the list of parameters sent in LS [6] for the NR AIML PHY.

- Observation 3: In RAN2/SA2/CT1 groups, the discussion on UE-side data collection for the AI/ML BM use case is still ongoing.

Proposal 1: RAN5 to further discuss a new UE special conformance test function to cater to AIML-enabled functionalities.

Proposal 2: RAN5 to evaluate and agree to add new UE test functions for each of the AIML use cases and modes.

- References

- R5-247681, AIML_RAN5_WayFoward, Orlando, USA, November 18-22, 2024.

- RP-250308, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R5-247151, AIML performance use cases, Qualcomm, RAN5#105, Orlando, USA, November 18-22, 2024.

- R5-246542, AI ML Conformance Call flow, Qualcomm, RAN5#105, Orlando, USA, November 18-22, 2024.

- R1-2503243, Consolidated Rel-19 higher layers parameters list Post RAN1#121, RAN1#121, Malta, MT, May 19-23,2025.