3GPP TSG-RAN5 Meeting #108	R5-254720

# Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1635 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update REGISTER with feature-param for text |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add feature-param requirement for text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In A.1.1 REGISTER, the Contact header has been updated with feature-param requirement for text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for RTT according to EU directive 2019/882 remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

## A.1.1	REGISTER

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | REGISTER |  |  |
| Request-URI |  | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or SIP URI formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
|  | A14,A15 | SIP URI formed from home domain name as preconfigured in the UE |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Route |  | not present |  | RFC 3261 [15] |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1,A3, A14,A15 | IP address or FQDN, port (optional) and not checked |  |  |
|  | A2 | IP address or FQDN and, when using UDP, protected server port of the UE |  |  |
| response-port | A1,A3 | rport (when using UDP) |  | RFC 3581 [96] |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec | A1 | any IMPU within the set of IMPUs on ISIM (when using ISIM; NOTE 3) or public user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
|  | A2,A15 | same public user identity as in initial REGISTER |  |  |
|  | A3 AND NOT A7 | public user identity derived from IMSI |  |  |
|  | A7 | emergency public user identity (NOTE 4) |  |  |
|  | A14 | IMPU preconfigured in the UE |  |  |
|  | A17 | same public user identity as in initial REGISTER |  |  |
| tag |  | must be present, value not checked |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec | A1 | any IMPU within the set of IMPUs on ISIM (when using ISIM; NOTE 3) or public user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
|  | A2,A15 | same public user identity as in initial REGISTER |  |  |
|  | A3 AND NOT A7 | public user identity derived from IMSI |  |  |
|  | A7 | emergency public user identity (NOTE 4) |  |  |
|  | A14 | IMPU preconfigured in the UE |  |  |
|  | A17 | same public user identity as in initial REGISTER |  |  |
| tag |  | not present |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | A1,A3, A14,A15 | SIP URI with IP address or FQDN and indicating either an unprotected port selected by the UE or no port at all |  |  |
|  | A2 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A7 | The SIP URI shall contain the sos URI parameter |  |  |
| feature-param | A4 AND A34 | +g.3gpp.icsi-ref="(comma-separated list of tag-values)” with comma-separated list of tag-values containing at least tag-value urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (see NOTE 2,5) |  |  |
|  | A6 AND NOT A7 AND A34 | +g.3gpp.smsip |  |  |
|  | A10 AND A34 | video |  |  |
|  | A28 AND (A29 OR A30 OR A32) AND A34 | audio |  |  |
|  | A37 AND A34 | text |  |  |
|  | A11 AND NOT A7 AND A34 | +g.3gpp.cs2ps-srvcc | Rel-11 |  |
|  | A12 AND NOT A7 AND A34 | +g.3gpp.cs2ps-srvcc-alerting | Rel-11 |  |
|  | A13 AND NOT A16 AND A34 | +g.3gpp.accesstype=”cellular2” | Rel-11 | RFC 3840 [63] |
|  | A13 AND A16 AND A34 | +g.3gpp.accesstype=”wlan1” | Rel-11 | RFC 3840 [63] |
|  | A35 AND A34 | +g.3gpp.ps-data-off=”inactive” | Rel-15 |  |
|  | A36 | +sip.app-subtype="webrtc-datachannel" | Rel-18 | RFC 5688 [160] |
| c-p-instance | A5 | +sip.instance="<urn:gsma:imei: (gsma-specifier-defined-substring)>” where gsma-specifier-defined-substring shall be the IMEI code of the UE, coded as specified in RFC 7254 [122], without optional parameters | Rel-10 | RFC 5627 [61] RFC 7254 [122] |
| expires |  | 600000 (if present) |  |  |
| Expires |  | present if no expires parameter in Contact header |  | RFC 3261 [15] |
| delta-seconds |  | 600000 |  |  |
| Require | A1,A2 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | sec-agree |  |  |
| Proxy-Require | A1,A2 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | sec-agree |  |  |
| Supported |  |  |  | RFC 3261 [15] TS 24.229 [10] |
| option-tag | A5 | gruu |  |  |
|  |  | path |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value | A1,A3 A14 | must be present, value not checked |  |  |
|  | A2,A15 | must be incremented from the previous REGISTER |  |  |
| method |  | REGISTER |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | value not checked |  |  |
| Security-Client | A1,A2 |  |  | RFC 3329 [21] RFC 4835 [124] |
| mechanism-name |  | ipsec-3gpp |  |  |
| algorithm |  | hmac-sha-1-96 |  |  |
| protocol |  | esp (if present) |  |  |
| mode |  | trans (if present) |  |  |
| encrypt-algorithm |  | des-ede3-cbc or aes-cbc or null |  |  |
| spi-c |  | SPI number of the inbound SA at the protected client port |  |  |
| spi-s |  | SPI number of the inbound SA at the protected server port |  |  |
| port-c |  | protected client port |  |  |
| port-s |  | protected server port |  |  |
| Security-Client | A14,A15 | not present |  |  |
| Security-Verify | A2 |  |  | RFC 3329 [21] |
| sec-mechanism | A2 | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A1,A3 A14,A15 | not present |  |  |
| Authorization | A1 | Digest |  | RFC 2617 [16] RFC 3310 [17] |
| username | A1 | private user identity as stored in EFIMPI (when using ISIM) or private user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
| realm | A1 | home domain name as stored in EFDOMAIN (when using ISIM) or home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
| nonce | A1 | set to an empty value |  |  |
| digest-uri | A1 | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
| response | A1 | set to an empty value |  |  |
| Authorization | A14(o) | Header optional |  |  |
| username | A14 | user identity as preconfigured in the UE |  |  |
| realm | A14 | home domain name as preconfigured in the UE |  |  |
| nonce | A14 | set to an empty value |  |  |
| digest-uri | A14 | preconfigured in the UE |  |  |
| response | A14 | set to an empty value |  |  |
| Authorization | A2,A15 | Digest |  | RFC 2617 [16] RFC 3310 [17] |
| username | A2 | private user identity as stored in EFIMPI (when using ISIM) or private user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
|  | A15 | user identity as preconfigured in the UE |  |  |
| realm | A2,A15 | same value as received in the realm directive in the WWW Authenticate header sent by SS |  |  |
| nonce | A2,A15 | same value as in WWW-Authenticate header sent by SS |  |  |
| opaque | A2,A15 | same value as sent by the server in “401 Unauthorized for REGISTER” |  |  |
| digest-uri | A2 | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
|  | A15 | SIP URI formed from home domain name as preconfigured in the UE |  |  |
| qop-value | A2,A15 | auth |  |  |
| cnonce-value | A2,A15 | value assigned by UE affecting the response calculation |  |  |
| nonce-count | A2,A15 | counter to indicate how many times UE has sent the same value of nonce within successive REGISTER requests, initial value shall be 1 |  |  |
|  | A17 | value incremented by one for each re-registration request |  | RFC 2617 [16] |
| response | A2 | response calculated by UE |  |  |
|  | A15 | response calculated by UE using password px_DigestPasswordForSIP |  |  |
| algorithm | A2 | AKAv1-MD5 |  |  |
|  | A15 | MD5 |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A2,A15, A14(o), A16(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A2 AND A31 | access network information for E-UTRAN and, if applicable, the cell ID |  |  |
|  | A14,A15 | access network information for Fixed Broadband with access-type field matching “*DLS*” and a "dsl-location" parameter (value not checked) |  |  |
|  | A16 | access network information, containing any of “IEEE-802.11”, “IEEE-802.11a”, “IEEE-802.11b”, “IEEE-802.11g” or “IEEE-802-11n”, and i-wlan-node-id parameter containing a MAC address according to TS 24.229 [10], 7.2A.4.2. Value of MAC address not to be checked |  |  |
|  | A2 AND A32 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A2 AND A33 | access network information for UTRAN and, if applicable, the cell ID |  |  |
| Content-Length |  | header shall be present if UE uses TCP to send this message and if there is a message-body |  | RFC 3261 [15] |
| value |  | length of request body, if such is present |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Initial unprotected REGISTER (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | Subsequent REGISTER sent over security associations (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A3 | REGISTER using GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |
| A4 | UE supports IMS Multimedia Telephony (MTSI) (A.3A/50 3GPP TS 34.229-2 [5]) |
| A5 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]). Mandatory from Rel-10 onwards. |
| A6 | UE supports SM-over-IP receiver (A.3A/62 3GPP TS 34.229-2 [5]) |
| A7 | Initial unprotected or subsequent REGISTER for emergency registration |
| A8 | Void |
| A10 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A11 | UE supports CS to PS SRVCC (A.12/40 3GPP TS 34.229-2 [5]) |
| A12 | UE supports CS to PS SRVCC in alerting state (A.12/41 3GPP TS 34.229-2 [5]) |
| A13 | UE indicates g.3gpp.accesstype media feature tag in REGISTER (A.12/46 3GPP TS 34.229-2 [5]) |
| A14 | Initial REGISTER SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A15 | Subsequent REGISTER SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A16 | IMS registration over WLAN |
| A17 | UE initiated IMS re-registration or de-registration (A.12/51 3GPP TS 34.229-2 [5]) |
| A18-A27 | Void |
| A28 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A29 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |
| A30 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |
| A31 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A32 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A33 | UE uses UTRAN access (A.18/2 3GPP TS 34.229-2 [5]) |
| A34 | Feature tags in Contact header to be checked. By default this condition is true. |
| A35 | UE supports PS data off (A.12/62 3GPP TS 34.229-2 [5]) |
| A36 A37 | UE supports IMS data channel (A.15/yy 3GPP TS 34.229-2 [5]) UE supports Real-time text (A.15/15 3GPP TS 34.229-2 [5]) |

NOTE 1:	All choices for applicable conditions are described for each header.

NOTE 2:	The “=” may include optional linear white spaces according to the EQUAL definition in chapter 25.1, RFC 3261 [15].

NOTE 3:	Public user identity shall be the same for ‘From’ and ‘To’.

NOTE 4:	According to TS 24.229 clause 5.1.1.1A and 5.1.6.2 [10] when the UE is using ISIM the emergency public user identity is the first public user identity in the list stored in the ISIM; when there is no ISIM it is the default public user id if the UE successfully performed IMS registration with the IM CN subsystem before, and the temporary user id (derived from IMSI) in all other cases.

NOTE 5:	URN is the outcome of the  URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.

<End of modified section>