3GPP TSG SA Plenary #109								SP-251146

Beijing, CN, 16-19 September 2025

Agenda Item:		19.4

Source:			Qualcomm Inc. (Rapporteur)

Title:				Summary for WI AvCall-MED

WI code(s):			1070056

leading WG:		SA WG4

Release:			Rel-19

### 1	Introduction

The work item has successfully completed the specification of the Avatar call functionality as an extension to IMS-based AR Real-Time Communication in TS26.264 [1].

### 2	Description

AvCall-MED adds the stage-3 pieces needed to enable avatar calls. It adopts ISO/IEC 23090-39 (ARF) for base avatars; specifies support for both 3D and 2D avatars; and defines data channel-based animation messages with support for facial, joint, and landmark animation.

The changes introduce SDP and data channel-based avatar negotiation to add avatars to an IMS call. The signalling allows for activation of the animation and avatar rendering on the sender side, in the network, or on the receiver side.

An Mbar_Management service for the Base Avatar Repository (BAR) is also defined to enable the creation and management of the user’s base avatar, as well as the selection and sharing of subset of the avatar assets to be used in an Avatar call.

### 3	References

[1]	3GPP TS26.264, IMS-based AR Real-Time Communication