TSG-SA5 Meeting #162	S5-253943

# Goteborg, Sweden, 25 – 29 August 2025									Revision of S5-253467

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0583 | rev | 1 | Current version: | 18.11.0 |  |
| For HELP on using this form: compr ehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.622 Update area scope definition for NPN support (stage 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMDT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 9.3.1.169 of TS 38.413 specifies the MDT configuration parameters for NR. There are two relevant IEs in this datatype regarding area scope: - Area Scope of MDT (CHOICE), which includes 8 choices: cell based, TA based, TAI based, PLMN wide, PNI-NPN based MDT, SNPN Cell Based MDT, SNPN TAI Based MDT, and SNPN Based MDT. - PNI-NPN Area Scope of MDT, which includes list of CAG cells. This IE can be combined with the choices above, when these represent non-CAG cells (cells only providing public access). In the context of area scope for MDT, when comparing attributes between MDT Configuration – NR (TS 38.413, under RAN3 remit) and AreaScope (TS 28.622, under SA5 remit), one can notice that the AreaScope presents different problems: - It does not allow supporting RAN3 scenarios involving only cells providing non-public access, i.e. when the choice is one of the following: “PNI-“NPN based MDT, “SNPN Cell Based MDT”, “SNPN TAI Based MDT”, and “SNPN Based MDT”. The reason is that AreaScope definition always forces the consumer to specify a value for one of the CHOICE attributes: nrCellIdList (which corresponds to CHOICE = cell based in 38.413), tacList (which corresponds to CHOICE = TA based in 38.413) or taiList (which corresponds to CHOICE = TAI based in 38.413). - It does not allow supporting RAN3 scenarios involving - It allows combining “tacList” with SNPN feature, which is a RAN3 scenario that is not supported. - It allows combining EUTRA with NPN features, which is not permitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 4.3.38 of TS 28.622 (AreaScope <<datatype>>), it is proposed to reformat the datatype, so that the scenarios and combinations allowed align with RAN3 scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between RAN3 and SA5 for area scope, with SA5 lacking support of one RAN3 scenarios, while enabling non-allowed scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.38.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.623 CR 0563 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 4.3.38	AreaScope <<dataType>>

#### 4.3.38.1	Definition

This <<dataType>> defines an area scope.

The Area Scope parameter in LTE and NR contains one of the following:

-	list of Cells, identified by E-UTRAN-CGI or NG-RAN CGI. Maximum 32 CGI can be defined.

-	list of Tracking Area, identified by TAC. Maximum of 8 TAC can be defined.

-	list of Tracking Area Identity, identified by TAC with associated plmn-Identity perTAC-List containing the PLMN identity for each TAC. Maximum of 8 TAI can be defined.

The Area Scope parameter in NR can also contain:

-	list of NPN IDs in NR. It is either a list of PNI-NPNs identified by CAG ID with associated plmn-Identity (Maximum 256 PNI-NPNs can be defined) or a list of SNPN by Network ID with associated plmn-Identity (Maximum 16 SNPNs can be defined).

#### 4.3.38.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 eutraCellIdList | CM | T | T | F | T |
| CHOICE_2.1 nrCellIdList | CM | T | T | F | T |
| CHOICE_2.2 nPNIdentityList | O | T | T | F | T |
| CHOICE_3.1 tacList | M | T | T | F | T |
| CHOICE_3.2 cAGIdList | O | T | T | F | T |
| CHOICE_4.1 taiList | M | T | T | F | T |
| CHOICE_4.2 nPNIdentityList | O | T | T | F | T |
| CHOICE_5.1 nPNIdentityList | O | T | T | F | T |

#### 4.3.38.3	Attribute constraints

| Name | Definition |
| --- | --- |
| eutraCellIdList | This attribute shall be supported, when the system allows supporting by E-UTRAN cells. |
| nrCellIdList | This attribute shall be supported, when the system allows supporting by NR cells. |

#### 4.3.38.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

End of Change